Cryo-Haemolysis Assay

This assay is performed to assess the effectiveness of a test compound at preventing red blood cell haemolysis during a “fast-freeze, fast-thaw” cycle. It can be completed within < 2 hours.

| Equipment | Consumables | Reagents |
| --- | --- | --- |
| 15 mL centrifuge | 1 x 15 mL centrifuge tube | Sheep Blood in Alsever's (ordered from TCS Biosciences, £11.90 for 100 mL, 3 days for delivery) |
| Vortex mixer | 3 x 10 mL Stripettes | PBS (available in cell lab, use non-sterile stocks) |
| Micro-centrifuge | 6n+2 1.5 mL microcentrifuge tubes | Lysis buffer (in cell lab fridge) (0.32 M sucrose, 5 mM magnesium chloride, 10 mM Tris-HCl and 1% Triton X-100 in water) |
| Liquid Nitrogen dewar tank with cane system | 3n cryo-vials | AHD (alkaline haematin D-575, in cell lab fridge) (2.5 g Triton X-100, 0.4 g Sodium hydroxide in 100 mL of water) |
| Waterbath | 1 x 96-well microplate |  |
| Cryo-vial canes (drawer under microscope) | 1 mL barrier non-filter pipette tips |  |
| Microplate reader | 200 µL yellow bevelled pipette tips |  |
| Pipettes (100-100 µL and 2-20 µL)* |  |  |

Where n is your number of test samples

*Bring over from the analytical lab

Safety Considerations

Liquid Nitrogen

● Know the first aid procedures for frostbite before using, handling, or storing a cryogenic liquid.

● Wear both safety goggles and when using, handling, or storing cryogenic liquids.

● Consider wearing insulating gloves when handling cold apparatus. Remove watches and rings.

● Avoid skin contact with cryogenic liquids. Even very brief contact can cause frostbite and/or torn flesh.

● Evacuate any area in which there is an uncontrolled release of cryogenic liquid or vapour.

Sheep Blood in Alsever's

This mixture is not classified as dangerous.

Lysis Buffer and AHD

Low hazard, take usual laboratory precautions.

Experimental Procedure

- Make up your samples in PBS at twice the intended concentration

- Centrifuge 10 mL blood at 2000 rpm for 5 minutes

- Remove supernatant

- Re-suspend in PBS to make up to 10 mL

- Label up enough cryo-vials to do each of your samples in triplicate

- Add 250 µL of blood to 250 µL of your sample in a cryo-vial

- Vortex briefly to mix

- Make up your controls (in small Eppendorfs)

1 x Blood:PBS 1:1 and 1 x Blood:Lysis Buffer 1:1, these are not to be frozen

- Incubate all samples at room temperature for 30 minutes

- Put cryo-vials into metal holders and transfer into the liquid nitrogen storage for 30 minutes

- Turn on the waterbath and set to 45 °C

- Remove cryo-vials and put into 45 °C waterbath for 10 minutes

- Label up 2 Eppendorfs for each cryo-vial and 2 extras for the controls

- Transfer from cryo-vials into Eppendorfs

- Centrifuge at 2000 rpm for 5 minutes. Remember to include the controls (weren’t frozen)

- Turn on the plate reader to warm up

- Fill remaining Eppendorfs with 750 µL AHD (kept in fridge)

- Add 10 µL of supernatant into each Eppendorf of AHD.

- Vortex briefly to mix

- Transfer 100 µL of solution from the Eppendorfs into the 96 well plate

Analysis

- Read the plate at 580 nm (or use the pre-set experimental parameters)

- Set the lysis control sample to 100% lysis and the PBS control to 0% lysis and then compare all treatments relative to these.

Waste Disposal

All blood contaminated waste into the autoclave bag

Non-contaminated waste into the black laboratory solid waste bin