Abstract: To improve more in-depth understanding of the fundamental mechanism of rain-induced loess slope’s landslides in Xi’an area, some artificial rainfall simulation tests and an in-situ comprehensive monitoring were carried out on a typical part of loess slope in Hu County , Xi’an where the delimited rain area was 7m×10m along the sloping surface. The monitoring results showed that, the maximum influence depth between pore-water pressure and slope mass slide displacement were 2.2m and 2.0m if total rainfall up to 280mm per day. The shear strength of landslide body rapidly reduced and the foot of slope was softened and collapsed. Therefore, under the influence of rainfall, the slope was forming a progressive sliding rupture from surface to interior.