Appendices

The Virtual Lung Disease Management Research Group

2011

# Project 22 –

# Virtual Lung Model for Disease Management

Chin Soon Tan – 0724443

Tobiasz Jackman – 0724847

Ben Lewis – 0721227

Daniel Sheehan – 0706896

Tom Batterbury – 063218

Supervised by Dr. Y. M. Chung

###### University of Warwick

###### School of Engineering

###### 2010/11

Contents

Appendix 1.0 - Main Report	13

Appendix 1.1	13

Appendix 2.0 - Technical Report	17

Appendix 2.1	17

Appendix 2.2	26

Appendix 2.3	28

Appendix 3.0 - Business Report	29

Appendix 3.1	29

Appendix 3.2	30

Appendix 3.3	31

# Appendix 1.0 - Main Report

## Appendix 1.1

SAFETY PLAN

1. Project Details

Project Title:  Geometric modelling and fluid flow simulation of the human lungs for disease management

Student Name: Thomas J Batterbury

Email: T.J.Batterbury@warwick.ac.uk

Location of work:  Warwick University computer laboratories, Students’ home (using home computer or laptop).

Timing of works (approximate):     Start date: 07/10/10		End date: 01/06/11

2. Supervisor Details

Name:  Dr Yongmann M.Chung, Associate Professor in Mechanical Engineering

3. Scope and Activity Details

Modelling the human lungs (models given by our project supervisor) using computer software, performing CFD (computational fluid dynamics) on these models. Altering these models and again performing CFD on each model. Using computer algorithms to create 1D models of the entire bronchial system of the lungs.

All activities will be performed on a computer workstation or at home on a personal workstation.

4. Safe System of Work

- Task Assessment – All tasks will involve the use of a computer workstation only.

- Identification of hazards and risk assessment – Associated risks with all tasks.

- Description of Safe Methods – Hazard elimination using safe working methods during tasks.

- Implementing the system – Implementation of Safe methods and awareness of safety over project duration.

- Monitoring the system – Observing project members in tasks and correcting any errors or incorrect practice.

5. Risk Assessments

- Possible electric shock from computer hardware.

- Possible human injury from computer hardware failure.

- Possible human injury from work bench or office chair failure.

- Possible human injury from incorrect position and posture, or from over working.

6. Equipment Required

- Computer hardware

- Computer software

- Work bench

- Office chair

7. Chemical Management

N/A

8. COSHH Assessments

N/A

9. Emergency Procedures

- First aid may not be necessary as it is assumed all work will be carried out at the university.

- In case of electric shock, look after patient while ambulance is called.

- In case of serious injury, look after patient while ambulance is called.

10. First Aid and Injury Management

First aid assistance may be obtained by following the process outlined in: http://www2.warwick.ac.uk/fac/sci/eng/local/hands/firstaidinformation/

The latest list of First Aiders in Engineering is detailed on the following web link, however outside of the ‘normal’ working hours assistance will need to be sought via Security on 22222: http://www2.warwick.ac.uk/fac/sci/eng/local/hands/firstaidinformation/first_aid_postersept2010.docx

11. Incident reporting and investigation

Any Incident which incurs first aid support will be investigated to determine root causes and appropriate corrective actions.  An Accident Form will need to be completed in all cases,

http://www2.warwick.ac.uk/services/safety/reporting/

12. Training and Induction Requirements

- Basic computer usage

- Software usage

- Basic first aid (possibly)

13.  Safety Monitoring

- Observing practice in tasks and correcting any errors/incorrect practice

- Consistent reminders of safety procedures and safe working methods

14. Sign Off

This Safety Plan and attachments (and/or those proposed) incorporate all the safety requirements for this project.

Student Name:  Thomas J Batterbury

Signature:

Date:    27/10/10

Supervisor Name:  Dr Yongmann M.Chung

Signature:

Date:     /      /

# Appendix 2.0 - Technical Report

## Appendix 2.1

#include <iostream>

#include <fstream>

#include <cmath>

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#define DEGTORAD(x) ((x) * 0.0174532925)

#define PI 3.14159

using namespace std;

using std::cout;

using std::cin;

using std::endl;

using std::ofstream;

/////////////////////////    INITIALISATION    \\\\\\\\\\\\\\\\\\\\\\\\

int a = 0;

long i;

long j;

int l = 0;

// The geometry is defined by a series of cartesian coordinates in 3D space as well as a radius and an identification number.

// Data about each line is stored to a .CSV file. Comma Separated Variable files consist of lines of variables known as fields, sperated by commas. Individual lines are separated by carraige returns characters.

// Specify the target data file, if no file exists one will be created, if a file exists the data will be appended to the end of the file.

ofstream myfile ("example.csv", ios::out | ios::app | ios::binary);

// The properties of the fractal pattern are initialised in this section. These properties are the reduction factor and the number of generations.

// Initialise Fractal Properties

float reduction_factor;

int divergence_angle;

int generations;

// Each line is defined by the start and end coordinates in space. A perpendicular line which defines the axis rotation of each line with respect to its parent and two boolean variables defining whether the line has child defined child lines.

// Initialise Line Properties

// if loop to determine whether the parent branch has already been used to define child1?

struct line{

double length;

double radius;

int colour;

int generation;

long identifier;

double start_coord[3];

double end_coord[3];

double vector[4];

double reference_geometry[4];

bool Child1;

bool Child2;

double angle1;

double angle2;

}DataList[2000002];// Alter the size of this field depending on the number of generations of the lung model

// Three functions are defined to create child branches. Functions "fractal_branch1", "fractal_branch2" and "fractal_branch 3" define a child branch appended to the parent branch, the geometry of which is a transformation of the parent.

//Function Prototypes

line fractal_branch1 (line DataList1);

line fractal_branch2 (line DataList2);

line fractal_branch3 (line DataList3);

/////////////////////////  MAIN  \\\\\\\\\\\\\\\\\\\\\\\\

int main()

{

// Generate Initial Line

DataList[0].length = 10.0;

DataList[0].radius = 10;

DataList[0].colour = 255;

DataList[0].generation = 0;

DataList[0].identifier = 0;

DataList[0].start_coord[0] = 0;

DataList[0].start_coord[1] = 0;

DataList[0].start_coord[2] = 0;

DataList[0].end_coord[0] = 0;

DataList[0].end_coord[1] = 5.0;

DataList[0].end_coord[2] = 0;

DataList[0].vector[0] = 0;

DataList[0].vector[1] = 5.0;

DataList[0].vector[2] = 0;

DataList[0].vector[3] = 1;

DataList[0].reference_geometry[0] = 0;

DataList[0].reference_geometry[1] = 0;

DataList[0].reference_geometry[2] = 1;

DataList[0].reference_geometry[2] = 1;

DataList[0].Child1 = false;

DataList[0].Child2 = false;

DataList[0].angle1 = 25;

DataList[0].angle2 = 70;

// Initialise reference variables

i = 0;

j = 1;

// The bifurcation angle is generated using a random number generator

// producing a gaussian distribution within a specified range

// Seed the random number generator

srand ( time(NULL) );

cout<< "Enter number of generations"<<endl<<endl;

cin>> generations;

k = (4*(2^generations));

/* The following control structure calls each of the "fractal_branch" functions and stores the returned values into dynamics memory.

Each line is stored into the array "Datalist" which is indexed in a form which can be interpretted by the post processing and visualisation software.

*/

// Control Structure

/*

"if" loop to determine whether the parent branch has already been used to define child1?

If no then call Child1 function, specify that parent has been used for Child1 and increment exchange2 by one

If yes then pass on to "if" loop to determine whether the parent branch has already been used to define child1?

If no then call Child2 function, specify that parent has been used for Child2 and increment exchange2 by one

If yes then increment exchange1 by one (so that it is assigned to Datalist[1])

Continue to cycle the "do" loop until the maximum number of branches (defined by variable k) has been reached.

*/

do {

if ((DataList[i].Child1 == false) && (DataList[i].Child2 == false))

{

DataList[i].angle1 = ((rand() % 100)+30)/3;

DataList[i].angle2 = (rand() % 10)+30;

DataList[j] = fractal_branch1 (DataList[i]);

DataList[i].Child1 = true;

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

}

else if ((DataList[i].Child1 == true) && (DataList[i].Child2 == false))

{

DataList[i].angle1 = -((rand() % 100)+30)/3;

DataList[i].angle2 = (rand() % 50)+30;

DataList[j] = fractal_branch2 (DataList[i]);

DataList[i].Child2 = true;

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

DataList[j] = fractal_branch3 (DataList[j-1]);

j++;

}

else if ((DataList[i].Child1 == true) && (DataList[i].Child2 == true))

{

i+=4;

}

}

while (i <= k);

// Write data to .csv file using the ofstream operator initialised as 'myfile'.

// Each data field is separated by a comma and each line is separated by a carriage return.

do{

myfile <<DataList[a].start_coord[0]<<","<<-DataList[a].start_coord[1]<<","<<DataList[a].start_coord[2]

<<","<<DataList[a].end_coord[0]<<", "<<-DataList[a].end_coord[1]<<","

<<DataList[a].end_coord[2]<<endl;

a++;

}

while ( a < k);

myfile.close();

// End main

return 0;

}

/////////////////////////    FUNCTION DEFINITIONS    \\\\\\\\\\\\\\\\\\\\\\\\

line fractal_branch1 (line DataList1)

{

// Define newline operator

typedef struct line newline;

// call newline operator to assign memory for the child branch

newline exchange1;

newline exchange2;

exchange1 = DataList1;

double vector1[4][1] =  {exchange1.vector[0],

exchange1.vector[1],

exchange1.vector[2],

exchange1.vector[3]};

double vector2[4][1];

double vector3[4][1];

double vector4[4][1] =  {exchange1.reference_geometry[0],

exchange1.reference_geometry[1],

exchange1.reference_geometry[2],

exchange1.reference_geometry[3]};

double theta1 = exchange1.angle1;

double theta2 = exchange1.angle2;

double c, s, t, x, y, z;

// Set the start coordinates of the child branch to the end coordinates of the parent

// Initialise Start Coordinates

exchange2.start_coord[0] = exchange1.end_coord[0];

exchange2.start_coord[1] = exchange1.end_coord[1];

exchange2.start_coord[2] = exchange1.end_coord[2];

// Perform a vector transformations using roatation matrices to copy and translate the parent branch to creat each child branch

// Step 1: Rotate vector1 (line vector) around reference geometry to generate vector2

c = cos(DEGTORAD(theta1));

s = sin(DEGTORAD(theta1));

t = (1-cos(DEGTORAD(theta1)));

x = exchange1.reference_geometry[0]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

y = exchange1.reference_geometry[1]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

z = exchange1.reference_geometry[2]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

double rotation_matrix1[4][4] = {((t*x*x)+c), ((t*x*y)+(s*z)), ((t*x*z)-(s*y)), 0,

((t*x*y)-(s*z)), ((t*y*y)+c), ((t*y*z)+(s*x)), 0,

((t*x*z)+(s*y)), ((t*y*z)-(s*x)), ((t*z*z)+c), 0,

0, 0, 0, 1};

vector2[0][0] = (rotation_matrix1[0][0]*vector1[0][0])+(rotation_matrix1[0][1]*vector1[1][0])+(rotation_matrix1[0][2]*vector1[2][0])+(rotation_matrix1[0][3]*vector1[3][0]);

vector2[1][0] = (rotation_matrix1[1][0]*vector1[0][0])+(rotation_matrix1[1][1]*vector1[1][0])+(rotation_matrix1[1][2]*vector1[2][0])+(rotation_matrix1[1][3]*vector1[3][0]);

vector2[2][0] = (rotation_matrix1[2][0]*vector1[0][0])+(rotation_matrix1[2][1]*vector1[1][0])+(rotation_matrix1[2][2]*vector1[2][0])+(rotation_matrix1[2][3]*vector1[3][0]);

vector2[3][0] = (rotation_matrix1[3][0]*vector1[0][0])+(rotation_matrix1[3][1]*vector1[1][0])+(rotation_matrix1[3][2]*vector1[2][0])+(rotation_matrix1[3][3]*vector1[3][0]);

// Step 2: Rotate vector2 around line to generate vector3

c = cos(DEGTORAD(theta2));

s = sin(DEGTORAD(theta2));

t = (1-cos(DEGTORAD(theta2)));

x = exchange1.vector[0]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

y = exchange1.vector[1]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

z = exchange1.vector[2]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

double rotation_matrix2[4][4] = {((t*x*x)+c), ((t*x*y)+(s*z)), ((t*x*z)-(s*y)), 0,

((t*x*y)-(s*z)), ((t*y*y)+c), ((t*y*z)+(s*x)), 0,

((t*x*z)+(s*y)), ((t*y*z)-(s*x)), ((t*z*z)+c), 0,

0, 0, 0, 1};

vector3[0][0] = (rotation_matrix2[0][0]*vector2[0][0])+(rotation_matrix2[0][1]*vector2[1][0])+(rotation_matrix2[0][2]*vector2[2][0])+(rotation_matrix2[0][3]*vector2[3][0]);

vector3[1][0] = (rotation_matrix2[1][0]*vector2[0][0])+(rotation_matrix2[1][1]*vector2[1][0])+(rotation_matrix2[1][2]*vector2[2][0])+(rotation_matrix2[1][3]*vector2[3][0]);

vector3[2][0] = (rotation_matrix2[2][0]*vector2[0][0])+(rotation_matrix2[2][1]*vector2[1][0])+(rotation_matrix2[2][2]*vector2[2][0])+(rotation_matrix2[2][3]*vector2[3][0]);

vector3[3][0] = (rotation_matrix2[3][0]*vector2[0][0])+(rotation_matrix2[3][1]*vector2[1][0])+(rotation_matrix2[3][2]*vector2[2][0])+(rotation_matrix2[3][3]*vector2[3][0]);

// Step 3: Rotate reference geometry around line

exchange2.reference_geometry[0] = (rotation_matrix2[0][0]*vector4[0][0])+(rotation_matrix2[0][1]*vector4[1][0])+(rotation_matrix2[0][2]*vector4[2][0])+(rotation_matrix2[0][3]*vector4[3][0]);

exchange2.reference_geometry[1] = (rotation_matrix2[1][0]*vector4[0][0])+(rotation_matrix2[1][1]*vector4[1][0])+(rotation_matrix2[1][2]*vector4[2][0])+(rotation_matrix2[1][3]*vector4[3][0]);

exchange2.reference_geometry[2] = (rotation_matrix2[2][0]*vector4[0][0])+(rotation_matrix2[2][1]*vector4[1][0])+(rotation_matrix2[2][2]*vector4[2][0])+(rotation_matrix2[2][3]*vector4[3][0]);

exchange2.reference_geometry[3] = (rotation_matrix2[3][0]*vector4[0][0])+(rotation_matrix2[3][1]*vector4[1][0])+(rotation_matrix2[3][2]*vector4[2][0])+(rotation_matrix2[3][3]*vector4[3][0]);

// Step 4: Write resultant vector to exchange2

exchange2.vector[0] = 0.7*vector3[0][0];

exchange2.vector[1] = 0.7*vector3[1][0];

exchange2.vector[2] = 0.7*vector3[2][0];

exchange2.vector[3] = vector3[3][0];

// Step 5: Apply Vector Transformation to Start Coordinates

exchange2.end_coord[0] = (exchange1.end_coord[0] + 0.7*vector3[0][0]);

exchange2.end_coord[1] = (exchange1.end_coord[1] + 0.7*vector3[1][0]);

exchange2.end_coord[2] = (exchange1.end_coord[2] + 0.7*vector3[2][0]);

// Step 6: Calculate Radius

exchange2.radius = 0.7*exchange1.radius;

return (exchange2);

}

line fractal_branch2 (line DataList2)

{

typedef struct line newline;

newline exchange1;

newline exchange2;

exchange1 = DataList2;

double vector1[4][1] =  {exchange1.vector[0],

exchange1.vector[1],

exchange1.vector[2],

exchange1.vector[3]};

double vector2[4][1];

double vector3[4][1];

double vector4[4][1] =  {exchange1.reference_geometry[0],

exchange1.reference_geometry[1],

exchange1.reference_geometry[2],

exchange1.reference_geometry[3]};

double theta1 = exchange1.angle1;

double theta2 = exchange1.angle2;

double c, s, t, x, y, z;

// Initialise Start Coordinates

exchange2.start_coord[0] = exchange1.end_coord[0];

exchange2.start_coord[1] = exchange1.end_coord[1];

exchange2.start_coord[2] = exchange1.end_coord[2];

// Step 1: Rotate vector1 (line vector) around reference geometry to generate vector2

c = cos(DEGTORAD(theta1));

s = sin(DEGTORAD(theta1));

t = (1-cos(DEGTORAD(theta1)));

x = exchange1.reference_geometry[0]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

y = exchange1.reference_geometry[1]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

z = exchange1.reference_geometry[2]/(sqrt((exchange1.reference_geometry[0]*exchange1.reference_geometry[0])+(exchange1.reference_geometry[1]*exchange1.reference_geometry[1])+(exchange1.reference_geometry[2]*exchange1.reference_geometry[2])));

double rotation_matrix1[4][4] = {((t*x*x)+c), ((t*x*y)+(s*z)), ((t*x*z)-(s*y)), 0,

((t*x*y)-(s*z)), ((t*y*y)+c), ((t*y*z)+(s*x)), 0,

((t*x*z)+(s*y)), ((t*y*z)-(s*x)), ((t*z*z)+c), 0,

0, 0, 0, 1};

vector2[0][0] = (rotation_matrix1[0][0]*vector1[0][0])+(rotation_matrix1[0][1]*vector1[1][0])+(rotation_matrix1[0][2]*vector1[2][0])+(rotation_matrix1[0][3]*vector1[3][0]);

vector2[1][0] = (rotation_matrix1[1][0]*vector1[0][0])+(rotation_matrix1[1][1]*vector1[1][0])+(rotation_matrix1[1][2]*vector1[2][0])+(rotation_matrix1[1][3]*vector1[3][0]);

vector2[2][0] = (rotation_matrix1[2][0]*vector1[0][0])+(rotation_matrix1[2][1]*vector1[1][0])+(rotation_matrix1[2][2]*vector1[2][0])+(rotation_matrix1[2][3]*vector1[3][0]);

vector2[3][0] = (rotation_matrix1[3][0]*vector1[0][0])+(rotation_matrix1[3][1]*vector1[1][0])+(rotation_matrix1[3][2]*vector1[2][0])+(rotation_matrix1[3][3]*vector1[3][0]);

// Step 2: Rotate vector2 around line to generate vector3

c = cos(DEGTORAD(theta2));

s = sin(DEGTORAD(theta2));

t = (1-cos(DEGTORAD(theta2)));

x = exchange1.vector[0]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

y = exchange1.vector[1]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

z = exchange1.vector[2]/(sqrt((exchange1.vector[0]*exchange1.vector[0])+(exchange1.vector[1]*exchange1.vector[1])+(exchange1.vector[2]*exchange1.vector[2])));

double rotation_matrix2[4][4] = {((t*x*x)+c), ((t*x*y)+(s*z)), ((t*x*z)-(s*y)), 0,

((t*x*y)-(s*z)), ((t*y*y)+c), ((t*y*z)+(s*x)), 0,

((t*x*z)+(s*y)), ((t*y*z)-(s*x)), ((t*z*z)+c), 0,

0, 0, 0, 1};

vector3[0][0] = (rotation_matrix2[0][0]*vector2[0][0])+(rotation_matrix2[0][1]*vector2[1][0])+(rotation_matrix2[0][2]*vector2[2][0])+(rotation_matrix2[0][3]*vector2[3][0]);

vector3[1][0] = (rotation_matrix2[1][0]*vector2[0][0])+(rotation_matrix2[1][1]*vector2[1][0])+(rotation_matrix2[1][2]*vector2[2][0])+(rotation_matrix2[1][3]*vector2[3][0]);

vector3[2][0] = (rotation_matrix2[2][0]*vector2[0][0])+(rotation_matrix2[2][1]*vector2[1][0])+(rotation_matrix2[2][2]*vector2[2][0])+(rotation_matrix2[2][3]*vector2[3][0]);

vector3[3][0] = (rotation_matrix2[3][0]*vector2[0][0])+(rotation_matrix2[3][1]*vector2[1][0])+(rotation_matrix2[3][2]*vector2[2][0])+(rotation_matrix2[3][3]*vector2[3][0]);

// Step 3: Rotate reference geometry around line

exchange2.reference_geometry[0] = (rotation_matrix2[0][0]*vector4[0][0])+(rotation_matrix2[0][1]*vector4[1][0])+(rotation_matrix2[0][2]*vector4[2][0])+(rotation_matrix2[0][3]*vector4[3][0]);

exchange2.reference_geometry[1] = (rotation_matrix2[1][0]*vector4[0][0])+(rotation_matrix2[1][1]*vector4[1][0])+(rotation_matrix2[1][2]*vector4[2][0])+(rotation_matrix2[1][3]*vector4[3][0]);

exchange2.reference_geometry[2] = (rotation_matrix2[2][0]*vector4[0][0])+(rotation_matrix2[2][1]*vector4[1][0])+(rotation_matrix2[2][2]*vector4[2][0])+(rotation_matrix2[2][3]*vector4[3][0]);

exchange2.reference_geometry[3] = (rotation_matrix2[3][0]*vector4[0][0])+(rotation_matrix2[3][1]*vector4[1][0])+(rotation_matrix2[3][2]*vector4[2][0])+(rotation_matrix2[3][3]*vector4[3][0]);

// Step 4: Write resultant vector to exchange2

exchange2.vector[0] = 0.92*vector3[0][0];

exchange2.vector[1] = 0.92*vector3[1][0];

exchange2.vector[2] = 0.92*vector3[2][0];

exchange2.vector[3] = vector3[3][0];

// Step 5: Apply Vector Transformation to Start Coordinates

exchange2.end_coord[0] = (exchange1.end_coord[0] + 0.92*vector3[0][0]);

exchange2.end_coord[1] = (exchange1.end_coord[1] + 0.92*vector3[1][0]);

exchange2.end_coord[2] = (exchange1.end_coord[2] + 0.92*vector3[2][0]);

// Step 6: Calculate Radius

exchange2.radius = 0.92*exchange1.radius;

return (exchange2);

}

line fractal_branch3 (line DataList3)

{

// Define Newline

typedef struct line newline;

newline exchange1;

newline exchange2;

exchange1 = DataList3;

// Initialise Start Coordinates

exchange2.start_coord[0] = exchange1.end_coord[0];

exchange2.start_coord[1] = exchange1.end_coord[1];

exchange2.start_coord[2] = exchange1.end_coord[2];

// Calculate Transformation Vector

// x component

exchange2.vector[0] = exchange1.vector[0];

// y component

exchange2.vector[1] = exchange1.vector[1];

// z component

exchange2.vector[2] = exchange1.vector[2];

// Apply Vector Transformation to Start Coordinates

exchange2.end_coord[0] = (exchange1.end_coord[0]+ exchange2.vector[0]);

exchange2.end_coord[1] = (exchange1.end_coord[1]+ exchange2.vector[1]);

exchange2.end_coord[2] = (exchange1.end_coord[2]+ exchange2.vector[2]);

return (exchange2);

}

## Appendix 2.2

Excel Spreadsheet code

## Appendix 2.3

Residual Plots for the final simulations in the main report.

Residual Plot for the final Unhealthy lung simulation

Residual Plot for the final Healthy lung simulation

# Appendix 3.0 - Business Report

## Appendix 3.1

Table A3.1 Belbin Roles and Associated Qualities [1]

| Role | Qualities of the Roles |
| --- | --- |
| Plant | Highly Creative, good problem solving skills |
| Monitor Evaluator | Provides a logical eye, makes impartial judgements |
| Co-ordinators | Focus on team’s objectives and delegates work |
| Resource Investigator | Provides inside knowledge on opposition |
| Implementers | To plan a workable strategy |
| Completer Finishers | Quality control to polish off finished work |
| Teamworkers | Indentify work required and completes on behalf of the team |
| Shapers | Provide Drive to keep the team moving |
| Specialist | An individual with in-depth knowledge of the subject |

## Appendix 3.2

A table of values used to create Graph 1 in section 4.1.1 Age and Gender.

Table A3.2 Number of Deaths in 1999 and 2004 from Lung Disease

|  | Figures from 1999 |  |  |  |  |  | Figures from 2004 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Under 35 | 35-44 | 45-54 | 55-64 | 65-74 | 75 & Over | Under 35 | 35-44 | 45-54 | 55-64 | 65-74 | 75 & Over |
| Men | 674 | 574 | 2329 | 7109 | 18512 | 45216 | 467 | 499 | 1825 | 6302 | 13600 | 35658 |
| Women | 494 | 492 | 1672 | 4458 | 12893 | 58745 | 411 | 398 | 1394 | 4126 | 9455 | 43321 |

## Appendix 3.3

# Virtual Lung Model MEng Group Project

Date: 8th October 2010

Time: 14:00

Location:  Outside F211

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Tom Batterbury and Dan Sheehan

# Discussed:

- We all agreed to look at all of the aspects of the project and then delegate tasks at our next meeting

- Hopefully the web page will get updated and then we can all select out free times for future meetings using that process

- We need to arrange another meeting time to meet Dr Chung, and it would therefore be preferable to meet him at the same time every week. Ben will contact him and find what time suits him best and contact everyone else

# Next Meeting:

Meeting with Dr Chung:  Tuesday 12th October 2010, 14:00

# Virtual Lung Model MEng Group Project

Date: 12th October 2010

Time: 14:00

Location: F215

Present: Ben Lewis, Tan Soon Chin, Toby Jackman, Tom Batterbury and Dan Sheehan

# Discussed

- Toby volunteered to be project leader and will also by the keeper of the project printing credit.

- Tom volunteered to be Health and Safety officer for the project, will complete safety forms to be reviewed in the next meeting and submitted in week 3.

- Toby has made a start on the website; each person will be allocated a page to update regularly, all minutes will be uploaded to the site.

- Star-CCM+ is not available in the computer labs, staff could not give a definite date when it would become available.

- SolidWorks can open STL file easily but hard to manipulate bronchiole model in SolidWorks – everyone will have a look at making the model usable over the next week.

- Secretary will change on a rota, changing every 2 weeks, secretary will be responsible for  taking minutes in meetings and uploading to website and organising the agenda for the next meeting.

- Propose the next meeting with Dr. Chung to be on Thursday (14/10/10) at 3pm.

- High computer power required to generate geometric model 1D model to be computed in MatLAB resulting in pressures and velocities in terminal bronchioles without graphic display.

- How many generations will be needed to be modelled? How many generations do doctors need modelled for diagnosis and drug management? Modelling 28 generations may be costly.

- Clear aims and objectives for the project need to be submitted by the end of week 3 (22/10/10). Everyone to think of aims and objectives so they can be finalised and submitted in the next meeting.

- Meetings to be every Tuesday at 2pm, meet outside F211.

- At meeting with Dr. Chung need to ask about more details for the CT data, is there a fractal algorithm he can recommend?

# Next Meeting:

Tuesday, 19th October at 2pm outside F211.

# Virtual Lung Model MEng Group Project

Date:  14th October 2010

Time:  15:00

Location:  F308

Present:  Dr. Chung, Tan Chin, Toby Jackman, Tom Batterbury, Ben Lewis and Dan Sheehan

# Discussed

- Haven’t finalised allocation of roles, difficult to do when STAR-CCM+ was unavailable

- Need to start the Lit. review

- Toby is project leader and in charge of the printing credit

- Ben is group secretary and is responsible for organising the meetings and making sure people can attend

- Tom is safety officer and will draft the health and safety forms

- Hope to finalise Aims and Objectives on Tuesday (19/10/10) to submit by the end of the week

- Tan Chin (Soon)  can use the equations in the given journal to do the 1D modelling in MatLAB

- Can use a group function in the University Outlook email rather than attempting to set up a message board on the website

- Journals relating to project can be found on Dr. Chung’s page; Yongmann Chung → References → AirPROM

- Dr. Chung to give us access to reference page to add any relevant or interesting journals or papers we find

- Web of Science is a good source of journals and papers

- When pasting links on website it is useful to not only give the .pdf link but to give a link to the title page of the paper where the citations used can be seen

- We can get remote access to the engineering dept. Linux machine from F211, so we can use openFOAM

- Images on the website showing pulmonary flow were calculated using Fluent

- This is a new project for the Eng. Dept. no existing student work is available

- Need to have flow simulation completed in term 1 so analysis and interpretation of the data can be done in term 2

- Objective : look at flow characteristics in subject specific geometry

- Can’t do visualisations in MALAB? It is possible 1D model can be shown as multiple 3D lines with different thicknesses and colours.

- Processing, a C++ program, can take data from MATLAB or openFOAM and make a graphic representation

- Problems using STL file in Solidworks – just surface data need to convert to solid body, try using add in called Scan to 3D

- Voxel Position data – is the co-ordinates of the terminal points in the bronchial tree

- Need to cut rounded ends in CT model

- Is it possible to reduce resolution in .stl data so SolidWorks can produce a solid body out of it easier?

- How can we make fractal geometry fit within lung boundary conditions?

    - Distort branches to fit boundary conditions

    - Calculations shouldn’t change much for fractal tree confined to lung boundary to a fractal tree that is not confined

    - Produce an algorithm to make fractal geometry

- NEED TO SORT OUT THE GEOMETRY BEFORE THE OTHER STAGES OF THE PROJECT CAN COMMENCE

- Approximate inlet conditions from pipe flow – a grey area! Simplify inlet geometry and simplify flow

- Use specific geometry for healthy and unhealthy patient or manipulate geometry to simulate a condition and study effect of geometry change on flow

- Could use object oriented programming – it is slower but speed is not an issue – for navier-stokes equations

- Dr. Chung will find the details of the patient specific data i.e. healthy/unhealthy, what condition they have

- Background – patient with breathing trouble can be scanned and assessed using a virtual model to see if airbourne drug is working correctly, could be used as a diagnostic tool?

- Roles Allocated

  - TOM/DAN – to look at 3D modelling

  - TOBY/TAN – to look into 1D modelling

  - TOM/BEN – Business model

# Next Meeting:

Group Meeting on Tuesday 19/10/10 at 14:00

With Dr. Chung on Thursday 21/10/10 at 15:00

# Virtual Lung Model MEng Group Project

Date:  21st October 2010

Time:  15:15

Location:  A206a

Present:  Dr. Chung, Tan Chin, Toby Jackman and Dan Sheehan

# Discussed

- Chin Soon Tan has simplified the Navier-Stokes equation for the 1D modelling

- The 1D modelling should model each branch separately as a bent pipe rather than modelling the bifurcation as that is a 2d problem

- Gravity could be neglected from calculations

- Need to map u and p along branches

- Dr. Chung gave Toby and Chin the simplified Navier-Stokes Eq. and other equations needed for the 1D modelling

- Toby has created a C++ program for creating branched geometry of the lungs

- 3D graphic visualisation essential for project

- Need to fill lung space – how?

  - Set lung wall as boundary from data from CT scan

  - Create a program that will clear a fractal branch if it reaches the lung boundary before the final generation of branching and create a new branch with different starting geometry

- STAR-CCM+ now available (old version though) but tutorials should still be relevant

- Need to finalise geometry so simulations can begin

- Meet tomorrow (Friday) between 12pm and 3pm to run STAR-CCM+ tutorials together

- MEng Project room by F111 could book it for meetings

- Try to finish STAR-CCM+ tutorials by next Thursday

- Not necessary for everyone to read each paper, once a paper has been read by someone it should be marked so our reading doesn’t overlap that much.

- Read one to two papers a week

- Reading can be split into papers relevant for 3D and papers relevant for 1D

- MSc student doing similar research was present at meeting, wants our contact details if he has any questions, he is focussing more on the medical than the fluids side so may need some help on that front

- Start with modelling pulmonary airways as if they have rigid wall, then move on to modelling an expanding airway

- Need to model entire breathing cycle

- Use generic data for inlet conditions, breathing cycle time period and volume flow rate, etc.

# Next Meeting:

Group Meeting on Tuesday 26/10/10 at 14:00

With Dr. Chung on Thursday 28/10/10 at 15:00

# Virtual Lung Model MEng Group Project

Date: 26th October 2010

Time: 11:00

Location: F210

Present: Ben Lewis, Tan Chin and Dan Sheehan

# Discussed

Aims and Objectives

- Expansion of the current version

  - Needs more detail

  - Methodologies on how to achieve the objectives

- Need to discuss the ultimate goal, though unlikely to achieve a model of the particle distribution of drugs etc. but need to outline its use

3D Modelling

- Dan has tried using SolidWorks to manipulate the CT scan, to allow conversion of the file from stl to enable editing, however he has experiences difficulties with the add-ins available etc.

- Dan has therefore used a trial version of “Magics” a software package that allows one to edit stl files. However it has taken approx. 1 hour to edit 3 lung endings

- Dan will continue to edit one lung model to see if the file can then be analysed in Star CMM+ as we require, though some concerns were raised with regards to the accuracy of this method.

- It was suggested that other members of the team continue looking for add-ins that will allow for the editorial work to be completed in SolidWorks, but these must be found by 2nd November, otherwise Magics software will be employed.

Poster

- Decided to have one central member complying all of the text and images onto the poster template to ensure that the formatting remains constant

Deadlines for Thursday 4th November Week 5 (unless otherwise stated)

- Aims and Objectives (Submission 28/10/10)

- Safety form (Submission 28/10/10)

- Find software to edit the 3D models (2nd November)

- Christmas vacation availability

- Poster submission Tuesday Week 15 work for this needs to be split depending on peoples availability during the Christmas vacation

- Working model by Week 8 (should allow 2 weeks for poster work)

- Reading of Papers and Journals, everyone should read 2 of these every week to allow for a Lit. Review to be started

- Lit. Review should be started and well on its way in week 10/15 depending on the how much of the poster has been completed

# Next Meeting:

# With Dr. Chung:  Thursday 28/10/10 at 15:00 in F308

Group Meeting: Tuesday 02/10/10

# Virtual Lung Model MEng Group Project

Date: 28th October 2010

Time: 15:00

Location: F215

Present: Ben Lewis, Tan Soon Chin, Toby Jackman, Tom Batterbury and Dan Sheehan

# Discussed

- Dan showed progress that he had made with Magics software that allowed the use of Star-CCM+ to create flow profiles and pressure distribution on 2 generations as a trial

  - Has completed editing one model

  - Needs to define a co-ordinates system

- Tom has completed:

        - Review of Safety Plan – needs signing by Dr Chung

        - Online safety form has already been sent

        - Display Screen Equipment Risk Assessment – Completed and will be handed in with the Review of Safety Plan

- Aims and Objectives

  - Tom has written a copy

  - Needs to be compiled with the initial one, this was completed and sent to Dr Chung for review

- Toby collected print credits – for this we have been given a new login for the School of Engineering computers, username: project22. We can use this to print from and for file sharing, save locations for reports, the poster and Star-CCM+ results

# Next Meeting:

With Dr Chung is on 29th October at 10-11am in A401

# Virtual Lung Model MEng Group Project

Date: 29th October 2010

Time: 10:00

Location: A401

Present: Ben Lewis, Tom Batterbury and Dan Sheehan

# Discussed

- Tom showed Dr Chung the Safety Plan and got these signed and handed in

- Dan showed progress of the lung model

  - Dan edited in the model in Magic’s, however Dr Chung suggested it would be best if we were able to make the ends perpendicular, but that it wouldn’t make a vast difference

  - Dan discussed the value/properties used in his simulation in Star-CCM+

  - Need to change the outlet pressure to outlet velocity

  - Need to think carefully into the boundaries conditions, adding straight pipes onto the ends of the cut pipes to ensure that the boundaries do not effect the results, that can be seen in Dan’s results

    - Surface – default condition

    - Slip Velocity – don’t need but it is important for car wind tunnels as the surface have to be 0ms-1 this makes simulations expensive

  - Therefore need to read papers for boundary conditions; flow and velocity

    - Advised that we should read 2 papers a week each

  - Used current resolution for the surface mesh and created a volumetric mesh, but need to improve the resolution

    - Need to clear mesh and re-mesh when importing into Star-CCM+

  - Difference between tetrahedral and polynomial meshing

    - Polynomial possibly better, but the better a meshing is the  more expensive to run due as it is time consuming

    - Need a balance of resolutions so look to see where the results converge

  - Model should be steady flow to begin with

- Dr Chung suggested that we should try and find an add-in to SolidWorks if possible and that we are allowed to purchase a student version

  - Ben is going to look into SolidWorks add-ins

- Tom will look into the particle distribution models

  - Therefore needs to consider particle size and density

  - Starting from the same point as Dan’s model

  - Think about the initial location of the particles

  - Would to good to plot a distribution of which sized/density of particles reach each generation

- Can put the particles in after the simulations as the flow won’t change, nor will the velocity as it is one way interaction. In reality lots of particles will change the results as it would be a two-way interaction – these two options should be available on Star-CCM+

- Deadlines (by the end of next week):

  - Particle distribution

  - 1D – Need to speak to Toby

  - Look into the number of cells used

    - Double to number of cells and repeat the simulation

    - Repeat above

    - Repeat above

    - Repeat above until the % change can be ignored (2% change in results) as the solution should kind of converge

    - Can we assume the same number of cells for each geometry?

      - 2 Options:

        - Models section by section to find the best

        - Model just on the final geometry

      - Remeber after a few generations will be laminar flow, also remember that to prove 2 million cells is best you need to do a 4 million simulation, this is expensive

  - Possibly try the second sets of geometry

# Next Meeting:

Tuesday 2nd November

# Virtual Lung Model MEng Group Project

Date: 2nd November 2010

Time: 11:00

Location: F210

Present: Ben Lewis, Tan Soon Chin, Toby Jackman, Tom Batterbury and Dan Sheehan

# Discussed

- Ben looked SolidWorks add-ins and found “ScanTo3D”

  - Dan had previously looked into this add-in using a trial version and has experienced various difficulties and has been unsuccessful using it

  - Ben has emailed SolidWorks to see informing them of the situation, hopefully to figure out whether the add-in should work and if so how much a student license would cost

- Magic’s

  - Since the last meeting Dan has found a tool that allows him to slice the end of the airways

  - He has also extruded 50mm length pipes from those sliced ends to prevent any of the boundary conditions being interfered

  - If the SolidWorks add-in doesn’t work then we need to look into purchasing a license so that we can all use the software otherwise Dan will have a heavy workload

- Star-CCM+

  - Dan will input the newly edited model into Star-CCM+ and run new simulations

  - It was suggested in the last meeting with Dr Chung that a particle deposition could be added once the flow simulation had been completed. Dan looked into this but says that we need to define the particle size, density etc. Was suggested that we leave this part of the project for now and concentrate on completing a working model first

  - Though Tom will look into the particle deposition in  Star-CCM+ while completing the tutorials

  - To apply the correct boundary conditions it may be necessary to know the details of the CT scan patient, e.g. age, gender, medical condition

- 1D Modelling

  - Toby showed the group his progress in C++ coding, the model is currently symmetrical but he is hoping to have a way to switch to a asymmetrical version if necessary

  - Toby spoke to IT services in engineering to allow us to see the C++ in a visual format, we will hopefully be able to show this to Dr Chung.

# Next Meeting:

# With Dr. Chung:  Thursday 04/11/10 at 15:00

Group Meeting: Tuesday 09/11/10

# Virtual Lung Model MEng Group Project

Date: 4th November 2010

Time: 15:00

Location: A401

Present: Ben Lewis, Tan Soon Chin, Toby Jackman, Tom Batterbury and Dan Sheehan

# Discussed

- Dan’s Modelling

  - Dan has extended the original geometry by 50mm, to ensure that simulations are more accurate

  - Could find a similar tool to the lofting tool which is used in SolidWorks

  - Was it straight forward to select the outlets

    - A Paper/Journal suggested a pressure difference to -225Pa inside the lungs

    - Outlets were selected manually, but there should be a function that can be used to select all openings/outlets

      - What if some were forgotten?

        - It would revert to default conditions so that the end of that generation would become blocked by a wall

  - The simulation shown to Dr Chung used 60000 cells, however only 85 iterations were made and it took approximately 1200 seconds. We need to remember to continue iterations until they converge, with only a 2% difference. For this monitor the convergence graph, viewed to show x momentum

  - Dr Chung will suggest a Paper to be reviewed that should suggest some appropriate boundary conditions, these need to be edited and the simulation re-run

  - Dan has only completed one CT scan version, the second type should be attempted

- Tom

  - Has completed all Star CCM+ tutorials

  - Needs to do a simulation, and look at the particle deposition tutorial within Star-CCM+

  - One way flow interactions will work when the particle size is small

  - Then try a two way interactions, this is where the flow field can be affected by the particles

  - A good starting point would be to look at one of the Papers published on the Meng group homepage

- Note for Toby that particle deposition cannot be achieve in one-dimensional modelling

- Is it possible to get detail on the patient who CT scan it is – eventually yes but not now and obviously only within the data protection act legislation – possible info. that could be useful to use; age, gender, illness

- Soon has attempted inputting some of the equation into MatLAB

  - Issue that there are 3 unknowns – this shouldn’t be the cause there should only be 2 unknowns

    - Should take u3 as boundary condition

    - Therefore we will need two equations to solve using simultaneous equations

    - There are 3 equations; Radius, Velocity and Pressure

    - These would create a complex simultaneous equation, however the Radius equation is separate from the other two and can therefore be used to solve the issue

  - MatLAB can be used to solve matrixes

    - How many elements can it handle?

    - We will be creating matrixes that a huge and therefore require millions of calculations, it was suggested that we should compromise and aim for calculations not to exceed 10million

- Toby is still working on the one-dimensional geometry code

  - He is currently expanding the 1D geometry into the 3D planes/space and is therefore working with vector equations to all this to work in the correct way, to give a lung shaped geometry

  - Toby has a visualisation of the coding however couldn’t be shown on the computer in A401. He will bring it in a different format to the next meeting

  - The current stable version of the code is 300 lines in length, but the current working version is approximately 400 lines

  - In Papers Toby has looked at there is useful geometry modelling techniques that he has employed

  - Can we use random numbers to generate a normal distribution – yes, requires another 10 or so additional lines of code, but these should be straight forward

- Work for the next week

  - Tom – will look at the particle deposition

  - Dan – needs to get Papers to apply the correct boundary conditions

  - Ben – needs to contact Magic’s about a license so more people can have a version. This will reduce Dans workload.

  - Once the Magics license is purchased

    - We need to find an automated way to select all the outlet points, otherwise it is highly likely that we could miss one

    - Need to find the number of iterations, which is dependent on the convergence curve

  - Soon – is looking at the CFD, simultaneous equations to allow work to be carried out in MatLAB

  - Toby – Random angle generation

  - All – Papers and Journals, we need to read the abstract and the introduction, if we think it is highly linked to our project we need to contact Dr Chung to see what his view is on the Paper/Journal

- Other tasks

  - Lit. Review – start thinking about this in week 8

  - Poster week 8 onwards

# Next Meeting:

Group meeting – Tuesday 9th November

# Virtual Lung Model MEng Group Project

Date: 11th November 2010

Time: 15:00

Location: A401

Present: Ben Lewis, Chin Soon Tan, Toby Jackman and Dan Sheehan

Apologies: Tom Batterbury

# Discussed

- Chin – mathematic equations are going well, needs to discuss a few things with Dr Chung after the meeting. Everything should be completed by Monday

- Toby – Has finished the coding

  - It currently is 420 lines long and creates a 3D fractural tree but is not lung shaped yet

  - Will upload a visual version on the webpage for everyone to view

- Dan – completed simulations with new boundary conditions

  - Comparing the 2 models with the edited boundary conditions show quite a difference hence why it is important to get the boundary conditions as accurate as possible

  - Would be good to de-select temperature from the graphs

  - Dan has uploaded the graphs/visualisations of the simulations on the webpage; http://www2.warwick.ac.uk/fac/sci/eng/staff/ymc/members/meng/images/

  - The most up to date model uses 900,000 points, we need to include more at least 1,000,000 – though Dans’ laptop couldn’t handle this, it might do once the temperature has been de-selected as it is less to compute

  - There are approximately 50 outlets of the lung, unknown number of generations at the moment

  - Dan has found how to select the outlets automatically; however some don’t get selected in this process due to the geometry. It is a useful tool and once used one can then select the ones that are missing

  - Need to compare simulate the geometry that already has some simulation results/visualisations and compare the results to see if they are similar

  - Improving the quality of the simulation should be focused on more than the quantity of simulations that we perform

- IEEE paper ‘Lin, CL; Tawhai, MH; McLennan, G; Hoffman, EA 2009 Multiscale Simulation of Gas Flow in Subject-Specific Models of the Human Lung IEEE Engineering in Medicine and Biology Magazine 28 (3), 25-33. (Read by All)’ – shows pressures in Figure 4 on page 30 at peak inspiration.

  - Assume pressure at the top of our lung model is atmospheric,  this isn’t correct but we will know the error from comparing to the IEEE paper

  - Alveoli pressure is like a balloon, with a higher pressure inside than out. We are able to find the pressure outside the alveoli and can then calculate through modelling the pressure inside at any generation, and we therefore know the pressure difference

- Discussed about how a patient would hold breath for 10 seconds before being told to breathe normally, it is normal when the cycle is repeated, these are the cycles we want to model

- We may find that there is a correlation between friction and the velocity; this can be transferred from the 1D model to the 3D one. Along with other interactions between the two different models

- Ben looked into Materialise software

  - Emailed Materialise, they informed us that Greg Gibbons uses one of their pieces of software possibly Magics for rapid prototyping in the WMG

  - Dan is currently using Magics, but we are tempted to look at Mimics, email response said that we can trial a version with a online tutorial

  - AirPROM – a research project that is being conducted in the future by Dr Chung does incorporate Materialise as a sponsor, therefore it may be a good avenue to follow as it will help for future research

- By next time

  - Dan

    - Try and sort the end points as some aren’t shown on the simulation results

    - Do the new simulations with the geometry that already has results, so a comparison can be made

  - Anyone/Everyone

    - Start looking at the Lit Review

# Next Meeting:

Group meeting – Tuesday 16th November

# Virtual Lung Model MEng Group Project

Date: 18th November 2010

Time: 15:00

Location: F308

Present: Ben Lewis, Chin Soon Tan, Toby Jackman and Dan Sheehan

Apologies: Tom Batterbury

# Discussed

- Dan informed us that he noticed that he was using the expiration geometry for the simulations, however this shouldn’t make too much difference at this level but will need to be changed at a later date

- Can possibly work out the change in dimension from the volume differences within the simulations

- Ben has obtained a 30 day trial of the Mimics software however it pieces together individual CT images to create a project. Without a project being started you cannot import any stl files to then manipulate them

  - After the meeting Dan and Ben will compare the Magics and Mimics software to see which is best suited for the project

- Dr Chung is currently in talks with Materialise to see if we can obtain a license for the Magics software however progress is slow

- Toby showed the group the coding that he has been working on

  - However at the moment they don’t all end on the same number of generations, this is due to file size, though the coding does think what orientation it should be in

  - He has started to input where the lung boundaries are (see image below), obtaining data from an online CT image source, hopefully this will allow the coding to know when to stop branching

  - The coding uses random angle generation using Gaussian elimination and Toby has input information from a paper so that the next generation is only 85% the length of its original one

  - Toby can define colour and line thickness of the 1D model and this will be done at a later date

  - The number of end points is 210 = 1024

    - This is due to the program limitations

    - Ideally we want to cover all generations but due to the program limitations cover 9 generations and then follow a couple/few to their end points from each side of the lung, like in Fig 2c) in IEEE paper (image below)

- Please keep the CT information within our group due to data protection

- Next:

  - Lit. Review, questions to think about:

    - Why is this project necessary?

    - How can it be improved?

    - What problems can this method overcome?

    - Why is it not suitable for all applications?

  - Dan needs to get passed the error issues with the new geometry model

    - It has currently taken 12 hours to prepare the next geometry; the 1st one that was completed didn’t take as long as the CT image had a lower resolution with only 50 endings. The bigger files have around 150 that need editing

    - The computers within the School of Engineering cannot handle the CFD for the lung, may have to cut the number of generations, but this will give less accurate results. Therefore:

      - Speak to the technicians to see whether any of the computers have a better spec.

      - Enquire whether we are allowed upgrade a computer with extra RAM using the group budget

      - The digital lab has more powerful computers maybe worth enquiring about using one of those

      - Someone suggested clustering the computers power, however it is really hard to generate and save a visualisation

      - If not, try using the CFD program in Linux as there will be no memory limitations (proven by a PHD student already using the software to run over a 100 million points)

  - 1D simulation

    - We need to determine the number of generations we can manage otherwise follow only a few like in the IEEE papers, as mentioned earlier

    - Remember to monitor the iterations

# Next Meeting:

Group meeting – Tuesday 16th November

# Virtual Lung Model MEng Group Project

Date: 23rd November 2010

Time: 14:00

Location: F211/MEng project room

Present: Ben Lewis, Chin Soon Tan (arrived after a meeting with Dr Chung), Toby Jackman and Dan Sheehan, Tom Batterbury

# Discussed

- Lit Review

  - Dr Chung suggested some questions we should answer (mentioned in previous minutes)

  - These questions should be answered by each person for each are they are studying in this project, saying there relevance and usefulness to the project

  - Could do a table of comparisons of the papers/journals

  - Try and get these done so they can be collated next week

- A0 Poster

  - From the project guideline document it needs to include:

    - Aims and Objectives (Tom and Ben)

    - The Strategy to achieve this (Tom and Ben)

    - The work performed to this stage

      - Dan – Validation comparing the two separate geometry files to hopefully show that our CFD simulations match that of the one that has been provided

      - Soon – Relevant equations that will be used and there use

      - Toby – 1D model explanation, how he approached it with images and coding to spruce things up

    - Work proposed for subsequent months (Team effort on content)

    - Project management and budgeting issues (Tom and Ben)

  - Poster Design – Tom and Ben with group feedback

  - Draft by week 10 so each individual knows what space they are required to fill

  - Examples of posters can be seen along the 2nd floor corridor and in the engineering workshop on the ground floor (similar to the layout below)

Title

Aims and Objectives

Business

Future

Current progress of the 1D and 3D Modelling and Equations

Team

- Magic’s Installation

  - We need a computer to put the license on

  - Could be a group members laptop but then it is not accessible 24/7

  - Toby investigated and he has guaranteed us a space in the MEng project room

  - Hopefully we are acquiring a group computer from Lynne Burrow

- Tasks for next week

  - Lit. Review

  - Poster

# Next Meeting:

Meeting with Dr Chung – Thursday 25th November

# Virtual Lung Model MEng Group Project

Date: 25th November 2010

Time: 15:00

Location: A401

Present: Ben Lewis, Chin Soon Tan, Dan Sheehan, Toby Jackman (arrived late as setting up group computer systems)

Apologies: Tom Batterbury

# Discussed

- A0 Poster

  - Needs to be submitted Week 1 of Term 2

  - We have split it up into our independent sections

  - We will hopefully have a draft copy by Week 10

- Lit. Review

  - It is very important  that we sort it out and we will need to use a large number of papers/journals

  - It is getting a bit late to write it all up now therefore we need to find which papers and journals and extract the information out of them so that we can use the information in our simulations

  - To summarise; there are two main elements, the introduction to the papers/journals which can wait until a later date and the extraction as we need these conditions now otherwise the simulations will not be realistic enough

  - We all need to read up on papers and journals

    - Soon needs to concentrate on the alveoli pressure differences

- Computer Status

  - We have been supplied with 2 computers

    - 1 for Linux – has a fault DVD drive is looking into

    - 1 for Windows – we are still waiting for it to arrive

  - Dr Chung has a PC in his office that we are allowed to use if needs be

- Equations

  - Soon has hopefully completed them

    - These need to be given to Toby so that he can input them into the coding

- Dan’s task

  - Another team member won’t really help at the moment but will be useful later on for preparing other geometries this will require cutting sections of the lung and defining them appropriately (inlet/outlet).  Some of these issue will be resolved when the groups computer is set-up in the MEng project room, so others will be able to take responsibilities from Dan as the preparation is time consuming

  - Needs to resolve mesh issues

  - Has spoken to the technicians and they advised them us to use the local HDD, however this still generates the same issues

  - Will try removing some generations to identify the issue, this may allow us to eliminate variables so we can pin down the actual problem

  - Maybe a mesh issue rather than a RAM issue

  - Searching for error showed error was with the stl file itself rather than the method used to during the simulation so we might have to find an intermediate step

  - We need to run a breathing out simulation even if the geometry isn’t correct. We can later do a complete cycle by changing the pressure over time (using the same geometry)

  - Though the geometry should change when the pressure changes, however we are going to have to accept some differences to an ideal real situation and make some compromises

    - Due to the limits in Star-CCM+ can’t change the geometry during a simulations, (well it can be done but extremely complicated and is beyond our versions/OS)

    - Cross referencing geometry files would be complex if we wished to simulate at different time steps throughout the breathing cycle this would also require lots of CT images and we only have two to use at the moment.

    - However these steady state results would still be a big leap from a real time movement, and most likely not get us any closer to how the lungs would respond

- 1D – the outer boundaries of the lung have been completed, which were started and mentioned in an earlier meeting, now Toby needs to input the equations that Soon gave him

- Next 2/3 Weeks

  - Dan

    - Fix the matrix issue/error messages then compare the completed results with those provided

  - Toby

    - Implement the equations into the code

    - Find info to put into the coding from paper/journals

    - Fix bugs in the coding and test

    - Look into FORTRAN code, as Dr Chung uses this code and would be able to assist with the coding if we had any issues

    - Need to find the radius of the ends of the 3D model so that we can join the 1D and 3D models together

# Next Meeting:

Group Meeting to be confirmed at a later date

# Virtual Lung Model MEng Group Project

Date: 2nd December 2010

Time: 15:00

Location: F308

Present: Ben Lewis, Chin Soon Tan, Dan Sheehan, Toby Jackman, Tom Batterbury and Dr Chung

# Discussed

- Computers now all set-up

  - With Star-CCM+ and Magic’s installed on the Windows PC

- 1D Modelling

  - Soon has started going through the equations with Toby

  - Toby has been looking at FORTRAN tutorials

- 3D Simulations

  - Simulated breathing cycle pressure over time

    - Time step 0.1 seconds and 4 seconds long with 20 iterations per time step

    - The graphs didn’t converge

    - Took 21 hours to run

  - Managed to apply a 1.5million element mesh on the geometry

  - Still haven’t succeeded with the other geometry files

  - Now we have Magic’s we should be able to make progress quicker

  - Dan will run through the workings with the others

- Will solve section by  section be accurate enough

  - Alternatively take information from the 3D geometry rather than the results. This would be patient specific with patient specific boundary conditions

  - Try to extract centre lines from the geometry, Toby tried this but it took 6 hours and didn’t work. Co-ordinates didn’t match up so may have to complete manually

  - Or maybe try using Magic’s

  - May be worth following 10 outlets first

- Poster

  - We will be given a time slot of approx. 15 minutes and there will be two examiners who will review the poster and ask any questions

  - We should really have a draft for next week, template has been completed

    - All involved in the poster and doing our separate parts

    - Remember the examiners are not experts in this area so be careful on medical terminology

- Lit. Review

  - Are doing parts for next week and summarising

- By the end of Term

  - 1D – All of Soon equations will be input into the coding so simulations can be run over  Christmas

  - Soon – try to find a better model

  - Write up poster sections

  - Need to run a test simulation and different meshes to see if they are fine enough (flow-rate shouldn’t change this means the simulation is accurate enough

  - Tom will complete a particle deposition model

- Other

  - plural pressure should be the same throughout the lung as it is one complete cavity

  - Static pipes – are quite a good model as cartilage rigged so will reduce the flexibility

  - If the pressure was higher on the outside to the in then the lung would collapse – can  look at this effect by changing the H value enabling us to simulate more medical conditions

  - Tom hasn’t received any emails from the Health and Safety forms that we filled in. Other groups have been told their H&S isn’t detailed enough, therefore ours should be fine

# Next Meetings:

Monday 6th December – MEng project room

Wednesday 8th December – MEng project room

Thursday 9th December – place to be confirmed

# Virtual Lung Model MEng Group Project

Date: 9th December 2010

Time: 15:00

Location: F308

Present: Ben Lewis, Chin Soon Tan, Dan Sheehan, Toby Jackman, Tom Batterbury and Dr Chung

# Discussed

- Progress

  - 3D Simulation

    - Tried again with new geometry in Star-CCM+ but only the original is workings however we now have 3 different mesh sizes

    - Results from the mesh sizes will be plotted to see convergence of result and therefore the most ideal mesh size will be employed for the rest of the simulations

    - Other geometry progress has been made in Magic’s, and Dan has explored the different formats that projects can be saved in, as one file type can be opened in Star-CCM+, however an error has occurred with the first attempt

    - Tom – has completed the tutorials, (however has had 4 modules this term, with a reduced timetable next term he will be able to make more progress). Dan will help with a “non-manifold” issue within Magic’s

  - Next

    - Prepare all the geometry ready for simulations

    - Dan is here all of the following week

    - Tom will be back from vacation (“study period”) on January 2nd

- Note that the School of Engineering will be closed between the 18th-22nd December

- 1D Modelling

  - Not much progress, running out of RAM

  - Will attempt to save parts onto the HDD during a simulation and therefore free up RAM for other use

  - Remote desktop, Dr Chung will send a link (Centre for Scientific Computing) to that Toby can download the NX Client software

  - Soon – found a good paper modelling rabbit lungs and is now trying to find information on the model they used as no explanation with the paper itself

- Poster

  - Has been making progress over the last week, with tasks delegated to the correct team members

  - (Dr Chung left at this point as we discussed poster format)

  - We will have an individual poster saved on the project 22 computer and will work on these first and will finally compile them together once each section has been completed

# Next Meetings:

It is now the end of the term and therefore the next official meeting won’t be until the New Year. However a number of the group are around during the holidays and therefore they will be meeting to perform simulations etc.

# Virtual Lung Model MEng Group Project

Date: 19th January 2011

Time: 14:15

Location: MEng Project Room

Present: Ben Lewis, Chin Soon Tan, Dan Sheehan, Toby Jackman and Tom Batterbury

# Agenda

- Weekly meeting time and place

- Draft Report

- Progress on 3D

  - Magics License

  - Star-CCM+ progress

# Discussed

- Dan has made a spreadsheet if when we are available

  - Looks like we are most likely to meet: Mon - 15:00 and Friday - 15:00

  - Need to email Dr Chung to arrange a weekly meeting time

- Draft Report

  - Toby has done an outline, which will be saved either in My Documents or on the Desktop of the group PC

  - Divided into 7 or so Chapters;

    - Chapter 1 - Outline, Intros, etc.

    - Chapter 2 - Lit. Review - needs splitting into 1D, 3D modelling

    - Chapter 3 - 1D Modelling section

    - Chapter 4 - 3D Modelling

    - Chapter 5 - Model Coupling

    - Chapter 6 - Subject specific modelling

      - Why? How?

    - Chapter 7 - Results, Discussing of Results, Evaluation

  - This is all OK, but need to look in the Module Procedure Document

    - Which is as follows:

      - The Final Report should consist of:

        - Exec Summary

        - Main Report

        - Technical Report

          - Coding, Data, Deviations

        - Business, Financial & Management Report

  - Draft due: Week24

  - Final report due: Week 31

  - We will write the main report and remove technical parts for the Technical Report

  - Sections will be as follows:

    - Toby – 1D

    - Dan – 3D

    - Ben – Business – need to write a brief outline as Toby has

    - Lit. Review – Toby doesn’t mind writing as long as people can divide up the journals and papers into 1D, 3D, Particle Deposition, etc. sections

- General Progress made in the 3D simulations

  - Dan has managed to mesh the model with 3 times the number of mesh elements than previous attempts by using the newer version of Star-CCM+ in F211

    - Highest mesh has 4.5 millions elements

    - However need to perform a simulation as we don’t know if this will run

  - As soon as we know the optimum mesh element to apply to the model then it is “all systems go”

  - Magic’s license is due to run out in 12 days however we should have all the .stl files modified within the next two week – Tom and Dan to finish these by a week Monday

  - We should try and install the newer version of Star-CCM+ on the group PC to see if this solves the number of meshing issues

  - Also Dan may install a version of Star-CCM+ on his new laptop

- Goals for next meeting

  - Toby – 1D draft report

  - Ben – Business report

  - Dan – 3D draft report

  - Tom – Finish the editing of the .stl file

  - Soon – Will writing the Lit. Review for the 1D Modelling of the report

  - Bring a HARD COPY of these to the next meeting

# Next Meetings:

Meeting hasn’t been decided on yet, but it is likely that most of us are free on Mondays and Fridays at 15:00.

# Virtual Lung Model MEng Group Project

Date: 24th January 2011

Time: 15:00

Location: MEng Project Room

Present: Ben Lewis, Chin Soon Tan, Dan Sheehan, Toby Jackman and Tom Batterbury

# Agenda

- Progress on last week’s actions points/goals

- Oral Presentation

- Meetings

# Discussed

- Oral Presentation

  - We have been emailed about oral presentations

  - Ours is: 9:00-10:00 Week 32, Thursday 12th May

  - This should give us a good amount of time to practise; we therefore want to have the content of the presentation written before the beginning of the third term so that we can start practising as a group.

  - Assessors are the same as for the poster, things that we can improve on:

    - Understandable – simplify

    - Proof read, we don’t want “Mess Elements”!!

    - Attention to detail

    - Re-write our Aims

    - Make sure it is visible

    - We wish to present to a group of other students that don’t have our background knowledge so that we can use their feedback to improve our presentation

- Meetings with Dr Chung

  - We need to have a meeting with him as soon as possible

  - Our centralised timetable spreadsheet were incorrect, preventing us to find a suitable time for us all to meet, this has been sorted

  - We are all free on Wednesday afternoons – this is the suggested time to meet Dr Chung on a weekly basis, if not one person may have to miss the meetings

- General Progress

  - Soon

    - Progress on set task: Hasn’t made any progress on this section because:

    - Stuck with finding a suitable model to base our 1D simulations on, however he has made progress with some MatLab programming

  - Tom

    - Progress on set task: Hasn’t completed but has started, this is vital as there is only 8 days left on the Magic’s license

  - Dan

    - Progress on set task: Has modified the work written for the poster and has written a template on what else is required in his section of the report

      - Needs to search through some books for details information and references

      - Needs to expand why he choose Star-CCM+

      - Expand in more detail about operations performed in Magics

    - From the two different types of stl. file available Dan has now cleaned up all the files that are a lower resolution (Lego like)

    - Though he has managed to mesh a file with 4.5 million elements Dan has run the model in Star-CCM+

  - Toby

    - Progress on set task: Has filled out the template that he produced for the last meeting, however this is the “bare bones” and will expand on this work and improve the quality

    - Aside from this Toby has also started to code implement the solutions to equations into the 1D model

  - Ben

    - Progress on set task: Has produced a template for the Business report similar to that of the one Toby did for the main report

      - What detail of costing is required:

        - Student salary

        - Academic fees

        - Electricity

        - Software Licenses

      - Maybe don’t include the global market

        - Suggested looking at the US health care costs

        - May look into the British private health care costs instead

      - Project management

        - Gantt Chart

        - Timesheets of the typical week

    - Note: we are developing a technique not a product. Companies will take this technique and develop it into the end product/equipment used

    - Toby suggests that the two most important points from the module procedure document are; Your team management structure and procedures and Strategic planning

    - IRP section maybe important for the coding

    - Environmentally – positive as could show how bad pollutants are effecting people’s health, changing the way people think about carbon emissions

- Lit. Review

  - Everyone should try and sum up the journals/papers they have read, so when it comes to writing the Lit. review it won’t require as much reading

- Action Points for Friday

  - Dan and Toby – Will be working together to try and create a trial coupling of the two models

  - Toby and Soon – Will be implementing the solutions from equations into the coding

  - Tom – Needs to finish the editing of the stl. files

  - Soon – needs to upload the papers/journals he has read

  - Ben – Should continue working on the Business report,  and split the business section with Tom

# Next Meetings:

Meeting with Dr Chung: Wednesday 26th Jan. 14:00

Group Meeting: Friday 28th Jan 15:00

# Virtual Lung Model MEng Group Project

Date: 26th January 2011

Time: 14:00

Location: A401 (Engineering building 4th floor)

Present: Chin Soon Tan, Dan Sheehan, Toby Jackman and Tom Batterbury.

Apologies: Ben Lewis (only busy this week at this time)

# Agenda

- Discuss term 2 progress with Dr Chung

- Discuss poster and poster presentation with Dr Chung

- Arrange meeting times for term 2

# Discussed

- 1D Modelling

- Problem with alveoli formulas addressed, problem with acquiring values of certain variables including surface tension. Models must be more refined in the future to avoid this problem.

- The difficulties of finding the mechanical properties of the model were also addressed, due to the specific values varying too much. Elastic force can be ignored in some cases, and friction must be added to the Bernoulli equations as they assume no friction. Finding the coefficient for steady flow was essential.

- Number of points used for analysis was discussed, 10-20 was the absolute minimum stated by Dr Chung, and the recommended number was 100.

- Toby would like to look into 3D modelling technique for the ends of the alveoli and robustness analysis, but this is only if there is enough time available.

- 3D modelling

  - Star-CCM issues were discussed, a new licence has been acquired this term, we have the old licence which should work, but if we can use the new licence the software should work. Dan has spoken to the F211 technicians about the problem. An alternative would be to use a different computer but we would prefer to avoid this. The problem should be resolved in the next few days.

  - All stl files cut and ready, except for one of Toms which is nearly finished, Dan discussed taking measurements of the models (diameter of tubes etc.) and has indexed bronchiole endings on his 3D models for easy reference. Tom will do this by the next meeting.

  - We do have pipe geometry values, which can be acquired from Dr Adrain Thomas’s model.

  - Problems with memory of our personal computer were discussed, as we do not have enough to run meshes with a very high number of elements. We cannot purchase more memory, 4 Gb is the maximum memory allocation for an version of the XP OS.

- Draft Report

- Time issues for this term were discussed, any ambitious modelling and analytical techniques were discouraged by Dr Chung.

- The report introduction must explain all parts of the report in detail, and be easy for the reader to understand (as should all parts of the report but especially the introduction).

- General points

- Decided to meet at this time each week for term 2 with Dr Chung

- Dr Chung was pleased with our progress marks as nearly everyone marked themselves lower than their piers did.

- Dr Chung was not greatly pleased with the poster, as it read more like a mini report than a poster. Time issues and management were discussed as Dr Chung thought the poster was rushed.

- Goals for the next meeting were left to be decided at the next meeting.

- Poster must be presented to potential students on Wednesday 2nd February. Dan and Toby would like to present but it has not been fully discussed who will be present.

# Next Meetings:

Meeting with Dr Chung: Wednesday 2nd Feb. 14:00

Group Meeting: Friday 4th Feb 15:00

# Virtual Lung Model MEng Group Project

Date: 28th January 2011

Time: 15:00

Location: MEng Project Room

Present: Ben Lewis, Dan Sheehan, Tom Batterbury and Chin Soon Tan (arrived late: 15:20)

# Agenda

- Wednesdays poster presentation

- Update on set tasks

- Tack allocation

# Discussed

- Though Dan expressed interest in the Wednesday presentation he is no longer free, therefore Tom has volunteered to replace him, Toby also showed interest in the last meeting but we are waiting for confirmation. The situation will be sorted out via e-mail.

- Progress on Task:

  - Dan – was meant to be working together with Toby to couple the two models

    - Dan has split the lung into quadrants and labelled the bronchioles in those areas

    - A table has been created to show the diameters of these tubes and which generation they are in

    - May look into an indexing systems in journal, but will use the current referencing system to ensure that the information required is gathered before the Magic’s license runs out in 4 days time

    - Star-CCM+ is now working on Dan’s laptop, but still not completely operational due to a license key being incorrect

  - Tom – task in progress was to finish the editing of the .stl files and completed an outline for the business plan

    - There is only 1 file that needs finishing

    - Hasn’t started the business outline

  - Soon – was going to upload paper/journals online

    - Hasn’t completed this yet, but has written summaries of a number of journals and expanded on information that he originally wrote up for the poster

  - Ben – To continue with the business plan

    - Has split the section with Tom and found information on strategic planning and the Belbins roles.

- AOB

  - A simple model for the alveoli has been agreed upon as a starting point, this model come from some of the initial models provided by Dr Chung

  - 5 weeks until the Draft Report is due

  - Advanced apologies from Dan for the Wednesday meeting

  - Advanced apologies from Soon for Fridays meeting

- Goals

  - Tom

    - To finish cutting the .stl files

    - Index the remaining .stl files and measure their diameters etc.

    - Outline the section of the business report

  - Ben

    - Develop the business plan section and expand on sections that have already been started

  - Dan

    - Finish all the file ready for the coupling stage

    - Run some Star-CCM+ simulations

  - Soon

    - Tidy up his written progress

    - Upload the journals online

    - Look into increasing model accuracy

# Next Meetings:

Monday 31st Jan 2011 - 15:00 in the MEng Project room

Wednesday 2nd Feb – 14:00 with Dr Chung location to be confirmed

Friday 4th Feb – 15:00 in the MEng Project room

# Virtual Lung Model MEng Group Project

Date: 31st January 2011

Time: 15:00

Location:  MEng Project Room

Present: Ben Lewis, Toby Jackman, Tom Batterbury, Dan Sheehan

Absent: Tan Soon Chin (no apologies sent)

# Agenda:

- Progress

- Wednesdays Presentation

- AOB

# Discussed

- Toby

  - Has written code on paper and is currently writing it up

  - This calculates the relative pressures throughout the system

  - 1D draft report is making steady progress

- Tom

  - Has written a brief outline of his part of the business report, it is saved on the group computer for people to review. Tom is also going to speak to B. Crofts this week about IPR and Licensing section

  - Tom has also completed all the cuts required within Magic’s however has only labelled one model, the others will hopefully be completed tonight

- Dan

  - Has completed measuring 4 models, so there are 3 left that he needs to finish

  - Done some simulations on the low resolution files, however didn’t fully work, Dr Chung suggested that we need to smooth the lung before we can run simulations

- Ben

  - Has made some progress on the business section and has started research useful business tools and has created some graphs

  - We aren’t creating a product, it is a research project therefore we need to look at if we required funding how we would pitch for funding

  - Therefore need to change business plan questions/tools to cover our needs

- Wednesday Presentation

  - Soon and Tom will be representing out group

- AOB

  - Magic’s license is running out, therefore we have installed Meshlab to allow us to view and measure the stl files if Tom and Dan don’t complete their tasks on time

  - Dr Chung would like an Report Introduction done asap, he said that this is a very important part of the project

  - Dan sends is apologies in advanced for the meetings on Wednesday and Friday

- Goals

  - Dan

    - Start an introduction and finish around this weekend and send to Toby to review

    - Finish .stl file editing

  - Toby

    - Finish coding by Wednesday to show progress to Dr Chung

    - Think about coupling methods – Dan mentioned he may know a method within Star-CCM+

  - Tom

    - Finish editing .stl files, labelling etc.

    - Wednesdays Presentation

    - Meet with Ben on Thursday to discuss business report

    - Contact Crofts with questions about the business section

  - Ben

    - Email Tom questions for Crofts

    - Concentrate on the Business section

    - Email Soon

  - Soon

    - Alveoli model paper still isn’t online, upload this and review the model

# Next Meeting:

Meeting with Dr Chung:  Wednesday 2nd February 2011, 14:00

Group Meeting: Friday 4th February 15:00

# Virtual Lung Model MEng Group Project

Date: 2nd February 2011

Time: 14:00

Location:  A401

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Dr Chung

Apologies: Tom Batterbury, Dan Sheehan

# Discussed:

- Coding is going well

- Dan and Tom have completed all but 2 of the .stl file editing

- This time last week we had an issue with Star-CCM+ license, but Dan now has a working copy on his laptop. He has also tried the F211 computer suite but we are unaware of his progress

- Eq. 39 in a sheet suggested to us by Dr Chung won’t converge. We agreed to use Gaussian elimination, do we need to know more equations? No if there are 32 elements there will be 32 unknowns, if there are 1,000,000 elements there will be 1,000,000 unknowns.

- Dr Chung – Coding should be relatively simple getting the results will be hard

- At the moment we are working with 32 elements which should take under a second to compute, but we should be aware that 1,000,000 may take up to an hour

- Dr Chung – you will always make a mistake in coding so don’t try not to.  Try and find a good method to find the mistakes made, e.g. Talking to Soon who also knows the algorithms you’re using

- Soon will send the MatLAB file that he has developed to Toby

- Question regarding number unknowns – potentially 4 but normally 3

  - Toby is using the coding to review 32 elements following one line through the lung with no split branches

    - In this model as the area decrease the velocity increases, though this won’t happen in reality. But this will be a good way for checking the coding

  - At the moment each section has four elements similar to below:

  - It may be worth being able to change the number of these elements for different generations, as it will allow you to focus on certain areas, speeding up the simulation times

# Next Meeting:

Meeting with Dr Chung:  Wednesday 9th February 2011, 14:00

Group Meeting: Monday 7th February 15:00

# Virtual Lung Model MEng Group Project

Date: 7th February 2011

Time: 15:00

Location:  MEng Project Room

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Tom Batterbury and Dan Sheehan

# Discussed:

- Our general progress

  - Tom – has emailed Dr Crofts to acquire details of the business section of the report, however we was instructed to speak to Dr Chung about it

  - Dan

    - Has completed a simulation and the low resolution lung files, now work and converge

    - Therefore these files have been smoothed, but this loses some of the features, but this is negligible

    - The “high resolution” files have already had a similar process performed on them, which Dan has worked out to do but not as good

    - Simulations performed to know what the optimal number of mesh elements may need to be restarted as the version of Star-CCM+ has been changed, so will speak to Dr Chung

    - Dan has written and Introduction which is approximately 700 words long

  - Soon

    - Has finished the excel spreadsheet for the symmetrical models and will now do one for the 1D asymmetrical models

    - There is no paper to upload for the alveoli model, it was an email source provided by Dr Chung

  - Toby – has written the code out, and is currently testing it to ensure it is working correctly

  - Goals

    - Tom and Ben – Continue writing the business section

    - Soon – Continuing with the excel spreadsheets

    - Dan – Will try and finish simulations so we know what mesh base to use

    - Toby – Will stabilise the code and will read through Dan’s introduction which will be shown to Dr Chung on Wednesday

# Next Meeting:

Meeting with Dr Chung:  Wednesday 9th February, 14:00

Group Meeting: Friday 11th February 15:00

# Virtual Lung Model MEng Group Project

Date: 9th February 2011

Time: 14:00

Location:  A401

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Tom Batterbury, Dan Sheehan and Dr Chung

# Discussed:

- Our general progress

  - Dan

    - Has been running simulations however they have been slow, but no errors warnings

    - Thought it was the new geometry but tried an older version and didn’t work correctly

    - Might need to re-mesh the geometry as we changed the version of Star-CCM+ that we were using

    - Dan showed some results from simulations

      - Streamlines with colour for velocity were better on the poster as a representation than the ones we viewed today, perhaps we should look into this

  - All geometry editing has been completed, but not all low resolution version have been smoothed

    - There are two unprocessed models left to smooth but the Magic’s license has run out

  - Toby

    - 1D code for fixed geometry is complete , but Toby wants to test the code to see how accurate it is

  - Soon

    - Finished spreadsheet, we need to analyze the data, e.g. plot friction vs generation number

    - Could also input the angle of each generation

      - This is useful as we want to start with pressure difference and determine the velocity, to do this we need the angles

      - Would taking the average angle be accurate enough? As the 1D model does use separate angles for each branch but this will be complicated to input within excel.

        - We could sum 20 to 30 angles for each generation and then average

        - Then can produce graphs without angle changing and then re-plot with a changing angle and compare the results. We could also do this for the 1D model as well

        - Assuming flowrate Q (no number) is a symmetrical model then after a split, Q will half as you know the radius of the branch and therefore you can calculate velocity – but we know this won’t be the case

- Dan has written and introduction – he will send it to various members of the group to read through

- Business Section of the Report

  - When Tom contacted Dr Crofts he said that we could take one of two different aspects:

    - Traditional business plan – for if you have an end product

    - Or sponsorship/financial report if the project will continue in the future (which ours is)

      - Say what we are going to or could achieve

      - How much savings it could create

      - Why someone would sponsor such a venture

  - We should focus on the economic impact of the research rather than the money made. As from the view of the country in general as these skills don’t exist yet. Once we have this technology would we be able to decrease the cost to the nation? Can you put a value on a life? And how many you would/could save?

  - What is the current technology available?

    - At the moment we can’t see people’s problems, so pharmaceutical companies have target areas which the develop drugs for. But what if you have a lung abnormality? The drug may never reach the target zone.

  - Licensing – we the research lead to a development of a final product then yes licensing will need to be thought through

  - Costing – cost everything, people power, Magic’s licenses, etc.

    - Maybe look into future costs; training of skilled labour to operate machinery

    - How long the simulations take – would you be able to complete the results in 24 hours and how many could you do in the time frame? 2 patients?

    - Perhaps knowing this information may cure people that had a reoccurring issue – the dressing example made by Dr Chung – that though a new dressing type was more expensive a patients infection was cleared up quicker and therefore saved money in the long run.

  - Remember that if this diagnostic tool was develop it would be the only thing like it on the market and therefore the only competition would be today methods of analysis – the only drawback of this method is cost

    - Forecast if the package was developed

- Goals

  - Week 8 – DRAFT REPORT – make sure it is perfect and complete apart from results sections that we can update later on!

  - Ben – Contact Greg Gibbons about Magic’s License

# Next Meeting:

Group Meeting: Friday 11th February 15:00

# Virtual Lung Model MEng Group Project

Date: 11th February 2011

Time: 15:00

Location:  MEng Project room

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Tom Batterbury and Dan Sheehan

# Discussed:

The meeting was cancelled today but all the group members were contacted to see what progress they will make over the weekend in preparation for Mondays meeting.

- Tom – Planning on meeting up with Ben to look at the business section on Sunday

- Dan – Will explore the number of mesh elements and sizes and simulate to see what the optimal size etc. will be

- Soon – Will try and solve the problems presented to him by Dr Chung, now using Bernoulli instead of Poisseulle

- Toby – Continuing with coding

- Ben – Will contact Greg Gibbons and continue with the business section of the report

# Next Meeting:

Group Meeting: Monday 14th February 15:00

# Virtual Lung Model MEng Group Project

Date: 14th February 2011

Time: 15:00

Location:  MEng Project room

Present: Ben Lewis, Toby Jackman, Tan Soon Chin and Dan Sheehan

Absent: Tom Batterbury (no apologies sent)

# Discussed:

- Our general progress

  - Soon

    - Met with Dr Chung on Friday. Solving using the Bernoulli equation was rejected by Dr Chung and the angle of bifurcation of the lung is nearly impossible so we will not be inputting this into excel

    - Still looking for the new formulae/method to apply to the spreadsheets

  - Dan

    - Has done 3 further simulation to determine the best number of mesh elements these are as follows:

      - 1.9 million elements

      - 2.7 million elements

      - 3.8 million elements

    - These all produced similar figures

    - Dan is going to re-runt eh previous simulations to see if the mass flowrate had converged as initially he was stopping the simulations after a certain number of iterations had been performed

    - He will then validate out simulations with the results from the following paper “J.W. De Backer et al. 2007 Flow analyses in the lower airways: Patient-specific model and boundary conditions Medical Engineering and Physics Journal 30 872-879 Ref “
This is due to Dr Chung having the results from these simulations, they have used the same patient geometry that we are using

    - Dan has also created a new webpage “Work in Progress” so we can upload our work to show progress

  - Toby

    - Still working on the coding, had made a good amount of progress but issue with deleting the incorrect files meant that he has been set-back

  - Ben

    - Showed the group their Beblin roles

    - Has also found a new paper by the British Thoracic Society from 2006, so we can now compare to the numbers that were presented in their 2001 report to see if deaths from lung disease has increased or decreased between these dates

    - No word limit for the Business section yet, but no more than 10,000

- Goals

  - Ben

    - Needs to find some numbers to input into the Introduction

    - Upload the British Thoracic Society Papers

    - Upload Business Section to “Work in Progress” section

    - Contact Greg Gibbons

  - Dan

    - Will upload the Introduction to the “Work in Progress“ page

  - Everyone

    - Bring hard copies of your work to the meeting on Wednesday to show group progress on your sections, (results, coding etc.) and the draft parts of the report

- AOB

    - Dr Chung wants to see, Introduction and Methodology parts of the report

# Next Meeting:

Group Meeting with Dr Chung: Wednesday 16th February 14:00

# Virtual Lung Model MEng Group Project

Date: 16th February 2011

Time: 14:00

Location:  A410

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Dan Sheehan and Dr Chung

Absent: Tom Batterbury (no apologies sent)

# Discussed:

Progress:

- Toby

  - Showed Dr Chung transcript of the code – It is all working without any errors

  - Radius is changing in space not time

  - Got some flow rate figures, but these won’t be accurate, why? Because the radius are the same at the moment

- Soon

  - Took values “mass flow rate” and atmospheric pressure (no length of the trachea)

  - Has develop a flow rate vs generation graph

  - From Poiseuille flow rate equation and take into account the wall friction, but is independent of velocity and dependant on length

How was this done? Poiseuille equation was used to calculate the resistance:

  - Graph shows pressure within the airway decreases

  - We can’t find Um of each generation without having a large spread sheet

  - Asymmetrical version – we can get overall resistance for each generation and will have to work out the resistance for each pipe within the generation

- Dan - 3D Simulations

  - Has been working on the sensitivity of the mesh– called: “mesh independence test”

  - Plotted a graph to shows the effects where:

    - Blue Line– Mass flow-rate(last point on the graph is incorrect as the simulation hasn’t finished running

    - Red Line – Time per iteration

    - Would be a good idea to take the logs of these and plot another graph

    - All the values are within 1% of each other

    - This is mass flow rate in the trachea

      - Chosen as it was the easiest parameter to model

      - Could pull any value out as all the information is available, therefore can do a similar graph of any generation we have data for

      - We are hoping the final simulation will level off

  - Why is the flow-rate increasing with a higher resolution mesh?

    - Could be the resolution, see image below, the image on the right has a better accuracy.

Trachea

Mesh Resolution

    - But if this was the case the mass flow rate would decrease as the smaller the hole with the same pressure the less fluid that would be able to travel through it over a certain time

  - However the trachea is much larger than the smaller generation. Are we using the same base size of mesh for the whole model?

    - Yes at the moment we are, Dan has tried to change the mesh size for different parts of the model but there have been errors

  - We therefore need to know the accuracy of the mesh for the smaller generation, so try another point within model and see the % difference on the most ideal mesh size etc.

  - We didn’t always have a pressure difference in the end of the generation due to the mesh size, but since we have changed the mesh size we have had results all the way to the end generations

    - A 3 million element mesh took 61,000 seconds to complete (17 hours)

  - We need to do a simulation with a few thousand iterations (over a weekend) per time step to see whether it is worth the time. Five or six iterations per time step is probably just as good, but we need to monitor this.

  - Then do a cycle with different time steps sizes and compare results

    - When the 1D and 3D models are coupled we will need to use the same time steps. Toby is currently using 100 time-steps per cycle

- Other 3D Geometry

  - Our license for Magic’s had run out, but Ben has contact Greg Gibbons and obtained a new license key

  - We can now set-up a new simulation as we know the optimal mesh base size

  - Currently using a breathing-out geometry so the airways will be smaller – will we use the same base size for the mesh for a breathing-in model?

  - We can then apply pressure from the other models (by Toby and Soon)

    - Soon – work out the alveoli pressure

    - Toby – uses the pressure given to him from Soon and calculates his on pressures for generation 6

    - Dan – uses the pressure Toby generates

    - However the airways in the CT scans don’t always finish at the same generation, we could edit this

    - Could we average the data, otherwise each outlet/inlet will have a different pressure – we can do this if there isn’t a large variance in the results

  - Perform a simulation on the smooth and rough models and compare CFD results

- Tasks

  - Dr Chung wants Soon to look into the equations he has used to know the background behind them

# Next Meeting:

Group Meeting:  Friday 18th February 15:00, MEng Project room

# Virtual Lung Model MEng Group Project

Date: 18th February 2011

Time: 15:00

Location:  MEng Project room

Present: Ben Lewis, Toby Jackman, Tan Soon Chin and Dan Sheehan

Absent: Tom Batterbury (has a lecture at this time)

# Discussed:

Progress:

- Soon

  - Still working on the spread sheets, as he is having to change cells values individually

    - He will look again over the weekend to see if there is an automatic way to do this

    - If not Toby suggested that he may be able to produce a code that could do the task for Soon

- Dan

  - Magic’s is now working with the new license key

  - The last point in the base size/number of mesh elements graph that shows the time taken for a simulation has levelled off, with only a 0.5% difference to the previous value (this was discussed on Wednesday)

    - We can now choose an optimal mesh size for the simulations, based on cost efficiency and  accuracy

  - After this Dan will apply this mesh size to his models and then validate his simulations against some performed in a paper/journal

- Ben

  - Has looked at the 2nd year business report for ideas/template

    - Will remember to show/emphasise on the cost saving to the nation and the environmental impacts

- Toby

  - Continuing with coding

# Next Meeting:

Group Meeting:  Monday 21st February 15:00, MEng Project room

# Virtual Lung Model MEng Group Project

Date: 21st February 2011

Time: 15:00

Location:  MEng Project room

Present: Ben Lewis, Toby Jackman, Tan Soon Chin and Dan Sheehan

Absent: Tom Batterbury

# Discussed:

Progress:

- Soon

  - Still hasn’t succeeded with the spread sheet he will have one more attempt and then he will using coding instead

- Dan

  - Set-up the simulation for the validation with the journal/paper, will run this soon

- Ben

  - Has made progress with the business report

    - Budget management section should be how we spend our funding

    - Need to have a separate section for the “Effects of our research” national savings etc.

- Toby

  - Checking code and equations, will re-write the code to simplify and shorten, which will also check for any errors

- Goals

  - Less than 2 Weeks until Draft Report is due

  - Monday bring all your progress so far and be prepared to piece them all together

  - Who is writing what?

    - Toby has uploaded and emailed us a copy of the report contents, we have all been assigned a colour and this is the section we are to complete for Monday. Colours are as follows:

      - Tom	Dan	Ben	Toby	Soon

# Next Meeting:

Group Meeting with Dr Chung: Wednesday 23rd February 14:00, A401 (or A312 if A401 is unavailable)

# Virtual Lung Model MEng Group Project

Date: 23rd February 2011

Time: 14:00

Location:  A401

Present: Ben Lewis, Toby Jackman, Tan Soon Chin, Tom Batterbury, Dan Sheehan and Dr Chung

# Discussed:

Progress:

- Dan

  - The simulation for approximately 3 million elements took around 10 hours, though this was performed on Dan’s laptop, which has better RAM and is using a new version

    - Dan is likely to use around 3 million elements due to this number of elements being used by another research paper/journal

    - The base size used for this simulation was 0.4mm

  - The other geometries

    - Dan has meshed a new breathing-in model with 10 million elements – this is due to the size of the model

  - What boundary conditions –we are just changing pressure – this pressure difference is used to achieve the velocities wanted in the trachea

  - Dan has completed two simulations so far

    - One from Riken which we can now smooth this as we have got a new Magic’s license. We can then compare with the un-smoothed file

  - Dan will perform a complete breathing cycle simulation this weekend

  - We now have no issues with the geometries

    - Jan De Backer has provided us with pressure and velocity  flows for healthy and unhealthy patients performed in Fluent

    - We should aim to achieve this but in Star-CCM+ and compare with his results

      - Note: we shouldn’t change the boundary conditions because in reality people breathe to achieve a certain velocity within the lungs, therefore someone with asthma will breather harder

- Soon - 1D Asymmetric Lung

  - Soon hasn’t worked out how to automatically work out the flow-rate for each independent pipe in a generation in Excel

    - Will look at coding this in MatLAB, C++, etc.

  - He could follow one branching for a simple solution for the time being

    - But how do we know how the flow is divided when it reaches a branch?

1

2

3

Q1=Q2+Q3 – this is a ratio which we don’t know but we know that length and radius of these pipes are controlling factors

We can look at the resistances of the pipes and this should produce the ratio for us, using:

8μLπr4

- Toby

  - Still fixing bugs in the coding

  - Flow-rate drops in his model, which is good, however down the far right hand-side branches a much higher flow rate is visible

- Business Report

  - Will be completed for a hypothetical business

  - We have split the sections required

    - Tom has completed licensing and environmental impact

  - We are meeting after the group meeting to compare our work and discuss progress so that it can be completed over the weekend

- Main Report

  - Lit. Review

    - Write up what we know about and have been working on and then merge into one document

    - What is the Lit review about?

      - Past Steps

      - What methods have been used in the past, which work and which don’t

      - Which methods we have chosen from these and chosen to follow/continue

    - Most importantly

      - The medical need for this research this section must be clear

# Next Meeting:

Group Meeting: Friday 25th February 15:00, MEng Project Room

# Virtual Lung Model MEng Group Project

Date: 25th February 2011

Time: 15:00

Location:  MEng Project Room

Present: Ben Lewis, Tan Soon Chin, Tom Batterbury and Dan Sheehan

Absent: Toby Jackman

# Discussed:

Progress:

- Business Section

  - Tom and Ben have set an outline out

  - We will split it into town main sections

    - Our background

    - Say that we are writing a report to say the benefits of investment using business tools

  - Goals for Monday

    - Tom

      - Environmental

      - Email Ben the edited version so he can compile the two sections

      - Improve Economic Impact section

    - Ben

      - Compile

      - Finish Market Analysis

- Dan

  - Added another 500 words to his report section

  - Nearly at the stage to run the validation simulations

    - Will make it ready to run over the weekend

- Soon

  - Toby has written a code to read tube length etc. and output the resistances

  - Will now concentrate on his sections for Monday

- Goals

  - Look at Monday meeting minutes for your tasks

  - To the next meeting bring as much work as possible

  - Tom – finish labelling last .stl file (may look at Riken files and label)

  - Safety Report – create a short section for the appendices of the report including the forms that were submitted

  - Import all your work to the website so we can all access it at the next meeting

# Next Meeting:

Group Meeting: Monday 28th February 15:00, MEng Project Room

# Virtual Lung Model MEng Group Project

Date: 28th February 2011

Time: 15:00

Location:  MEng Project Room

Present: Ben Lewis, Tan Soon Chin, Tom Batterbury, Toby Jackman and Dan Sheehan

# Discussed:

Progress on project report allocations, all work was uploaded on to the webpage “work in progress” section so we could all access it.

- Tom

  - Has completed his section for the Introduction

    - Content was different to Dan’s introduction, this section is to concentrate on the aims and objectives

      - This should be the a carry on from Dan’s introduction so we need to make it flow

    - Content Issues?: “measuring flow parameters” – sounds like it is an experiment, this should be changed to simulating, simulate etc.

- Ben

  - Hasn’t started the Lit. Review Section

    - Soon has written a summary of various reports, this is online as well, but not visible so look in the file store, it’s called ‘lung_parenchymal_review_draft_28_02’

  - The business report has been completely edited, with section from Tom incorporated

    - Some sections require a little improvement

- Toby

  - Has started looking into the background, which was started today. Therefore needs to edit his information and compile – this section may also mention about the lung expansion and elastic dynamics etc.

  - Section ‘Why we used a coupled model?’

    - This is covered in the poster section, but needs to be expanded, parts of this is written in the introduction and vice versa so we need to be careful what we include and where

  - Section: ‘Why we chose the method we are using?’

    - Also on the poster and will be moved to the correct place and put into more detail

- Soon

  - Requirement of 1D simulations

    - Suggests that the CT scans resolution can scan up to the 6th generations, however it is more of a range, should say it varies from 5 to 7 generation depending on the patient

    - Maybe too detailed for the main report, might need to move some of the maths to the technical report

  - Toby has previously written the geometry section for the poster, this will be expanded

- Dan

  - 3D Modelling Methods

    - Is online – we all need to read and make recommendations

    - Give feedback on Wednesday

- Business report

  - Look through and give feedback on Wednesday

After the meeting Tom and Ben stayed behind to discuss the Business Report

- We need to update the figures in calculations and they are from old sources

- Do a cost table of this section with worst and average case scenarios

- Move ‘current treatment’ section

- Compile a Glossary

# Next Meeting:

Group Meeting: Wednesday 02nd March 14:00, A401

Friday meeting – cancelled

# Virtual Lung Model MEng Group Project

Date: 2nd March 2011

Time: 14:00

Location:  A312

Present: Ben Lewis, Tan Soon Chin, Dan Sheehan and Dr Chung

Apologies: Tom Batterbury and Toby Jackman

# Discussed:

- Business section is 90% complete

  - Ben will add other ideas/developments this weekend

- Anatomy section – is very important and it should be reassigned if Toby hasn’t completed the code – Dan will take this section over and forward it to Toby to improve if he wishes

- Simulation

  - De Becker file has produced an error, Dan will look into this tonight as other assignments have been handed in

  - He has two simulation results from De Becker for us to validate/compare our results

    - Boundary conditions may be slightly different between the models, so look for similar trends rather than exact results

  - De Becker modified his geometries changing the pipe diameters (narrower and wider)

    - Though larger diameter helped in the upper airways, it caused difficulties in other areas

  - Easter Targets

    - Complete validation

    - Perform an unsteady flow simulation

    - During Easter perform a full breathing simulation – waiting for number from Toby

    - Could start by estimation a pressure difference

- Soon’s 1D Modelling

  - Is nearly complete

  - Can we include friction loss?

    - Soon will try again

    - Forget the above, the friction loss is already accounted for but we need to account for minor losses

  - Minor loss – is when the flow splits it distorts, need an equation for this

    - Could generate a figure from a simple geometry by simulation across 1 bifurcation

    - Though it may have a small effect it should be accounted for as the condition could accumulate changing conditions

    - Start with k = 0.2 until we find a more accurate number this is for a 45° curved pipe, though it’s not the same it will be fairly similar

      - This value is from experimentation

      - From Chapter 6.9 – 3rd year fluid book

- Need to mention if why we used either LES or unsteady RANS (standard k-epsilon) model

  - LES, (large eddy simulation) ignore small turbulence – the size of turbulence is ignore is relative to geometry

- The Reynolds transmission number will be different to normal, as the standard number quoted is for an ideal symmetrical smooth pipe

  - The lung is not symmetrical but the Reynolds number will be in the thousands

- Produce a timeline (in Business section)

    - Do a predicted progress and an actual, this will show that we had issues with acquiring a computer etc.

    - Show how we overcame this issue – there is NO sympathy vote!

# Next Meeting:

Remember the meeting on the 04/03/2011 has been cancelled

Group Meeting: Monday 7th March 15:00

# Virtual Lung Model MEng Group Project

Date: 7th March 2011

Time: 15:00

Location:  MEng Project Room

Present: Ben Lewis, Tan Soon Chin, Dan Sheehan, Toby Jackman

Apologies: Tom Batterbury (had a lab at this time)

# Discussed:

- Report Writing

  - Business Report

    - Separate referencing system etc. to the rest of the report

  - Main Report

    - Will have to compile all the sections that individuals are writing up

    - For the time being we should keep each “Chapter”/section with their own references. Once the draft report is marked and we can compile closer to the deadline if necessary

- Soon – 1D

  - Will use the coding from Toby to complete his section, however should note that it only encompasses resistances not flow through the pipes, this can be added if necessary

  - Is using the 0.2 k value as Dr Chung suggested for minor losses but feels that this value is too high so will continue to look for more information

  - Other than those points this section only needs a few touch ups, therefore we can allocate more work for Soon

- Dan

  - Tried to run an un-steady simulation this weekend but it didn’t complete

  - Running simulation at home with De Beckers models so that we can compare Star CCM+ to Fluent, however this simulation is taking 0.5 hours for one iteration

  - May consider splitting the model into smaller sections

- Toby

  - Continuing coding

- Report progress review

  - We have read through the assigned tasks and after seeing which have been completed we have reallocated tasks

    - This can be found on the project webpage, to see change look at the colour coding

- Internal Report Deadlines

  - Physically compile on Monday

  - Read through and improve by Wednesday to show progress to Dr Chung

  - Thursday - make any changes

  - Hand-in to the EUO/ESO

- Task Allocation

  - Ben – Read through Soon’s work and make comments

  - Soon – Read through the Business plan and make comments

  - Toby/Dan perform a similar operation

  - Tom – Budget Management

# Next Meeting:

Group Meeting with Dr Chung: Wednesday 9th March 14:00

Group Meeting: Friday 11th March 15:00 – MEng project room

# Virtual Lung Model MEng Group Project

Date: 9th March 2011

Time: 14:00

Location:  A312

Present: Ben Lewis, Tan Soon Chin, Dan Sheehan, Tom Batterbury, Toby Jackman and Dr Chung

# Discussed:

- 3D Modelling

  - Using tet mesh (Dr Chung mentioned it as the "poor man’s" mesh), could we use poly or hex

    - These could take longer to perform simulations but is a possibility

  - All geometry is complete

    - Has smoothed Riken

    - Worth explaining how this was generated, maybe more in the Technical Report

  - Using pressure as boundary condition, it is in the order of 3ms-1

    - There are some areas which are approximately 20ms-1, this could be due to mesh elements, if so this is signalling that we need to change the mesh, but in reality we are pushing the boundaries of cost effectiveness so could be ignored

  - There is one week left till the draft report is due, what other simulations are you intending on completing before this deadline?

    - Has run a simulation on the Riken geometry

    - Has smoothed the file and will run another

    - The result form this will be interesting as it may even suggest that smoothing of the geometry isn’t required

    - (All 3 files are of a similar size)

  - What is the pressure drop over the geometry? - Approximately 1000Pa

    - Toby suggested that he saw a pressure drop of around 300Pa in text books from the very top generations to the bottom

    - Dan used -1000Pa as a forced inhalation from De Backers simulations

- Toby - 1D Coding

  - Haven’t made much progress as report writing

  - Still not converging but is confident that there will be results to show in the final report

- Tom

  - Particle Simulations

    - Has been going over the Star-CCM+ tutorials

  - Business Report

    - Glossary of Terms has been completed but will email Ben them later along with the Budget Management

- Soon

  - Will use Bernoulli if it is found that k value makes a noticeable difference

  - Asymmetrical – will hopefully be completed by the final report

  - Symmetrical – will be completed for the final report

- Dr Chung mentioned:

  - What would we prefer?

    - A complete report with no results?

    - Results with no report?

  - We should remember that even if there is a report with no results then we won’t get any marks allocated for analysis

  - Look at re-allocation of task to try and combat this issue

    - 1D coding is really separate from any other part of the report – therefore Toby should concentrate on this section but should prioritise on getting some results and only briefly write an analysis

  - Don’t leave too much to complete over the Easter vacation

# Next Meeting:

Group Meeting: Friday 11th March 12:00

# Virtual Lung Model MEng Group Project

Date: 11th March 2011

Time: 12:00

Location:  MEng Project Room

Present: Ben Lewis, Tan Soon Chin, Dan Sheehan, Tom Batterbury and Toby Jackman

# Discussed:

- Draft report is not handed into the EUO, hand in directly to Dr Chung

  - Need to ask him when we will receive feedback so we know when to schedule improvements

  - Content of the report is more important than actual work so make sure that the report gives a detailed outline on what we shall include, bullet points etc.

- Feedback from allocations

  - Business Section

    - Make sure sources are all official, i.e. from papers and journals rather than websites

    - Input more diagrams and graphs

    - Elaborate on other diagnostic tools in detail

      - Maybe part of SWOT

      - Are they cheaper to run?

      - Quicker to perform?

  - Ben and Soon have read through each other’s documents and made changes and suggestions – they will both look through them this weekend

  - Toby

    - Met with Dr Chung to discuss his coding earlier today, he will make changes on some of the deviations

  - Dan

    - Finished simulations with De Backers files so will therefore have some results to add to the draft report

- Tasks

  - Tom

    - Look at new sources for the Business section

    - Look at existing techniques and compare

    - Particle simulations

      - Let Ben know if the workload is too great and he will try and continue one of these points

  - Toby

    - Coding

    - Write up some fractural geometry part

  - Dan

    - Will continue to write his section

  - Soon

    - Compare k values with Dan

    - Will try to review  Dan’s sections

    - Finish diagrams for his 1D section

  - Ben

    - Continue to compile the Business Report when Tom sends him the sections

    - Will review the feedback given by Soon

    - Continue any work Tom can’t complete

# Next Meeting:

Group Meeting: Monday 14th March 15:00

# Virtual Lung Model MEng Group Project

Date: 14th March 2011

Time: 15:00

Location:  MEng Project Room

Present:  Toby Jackman, Tan Soon Chin, Tom Batterbury and Dan Sheehan

Absences: Ben Lewis (sent apologies)

# Discussed:

Progress:

- Dan

    - Has written up 3D modelling section of report (4800 words total)

    - Performing simulations, will try to write up as much as possible before the draft report hand in.

- Soon

      - Has written up his parts of the draft report, and has uploaded the literature review section online on the project webpage (may be some grammar mistakes but he will update this as soon as possible).

      - Currently compiling his 1D spreadsheet, is comparing his values with Dan’s work

- Toby

- Is currently working on 1D code, has been spending a lot of time on it so has not finished his parts of the report write up.

- He is currently working on his part of the report and will finish it very soon.

- Tom

- Has finished altering business section, has also changed sources used for his half, as some of them were not reliable enough. The project timeline has also been completed.

- Has written most of the competition comparison section, will finish and upload it later on today or tomorrow before the draft report hand in

- Has tried to produce particle simulation diagrams but has had a lot of difficulties with convergence and solver settings so no diagrams have been created. He will contact Dr Chung to see if it is worth spending more time trying to produce results, or to just move on and help Dan with his work.

# Goals for next meeting:

- Toby  - Will try to complete code by Wednesdays meeting

- Soon – Will carry on with producing his spreadsheet as his report sections have been completed

- Dan – Will read through report once compiled, to make sure it flows smoothly

- Tom – Will finish his competition comparison section, and will also carry on with particle simulations to try to produce results to show Dr Chung

# Next Meeting:

Wednesday 16th March in A401 at 14:00

# Virtual Lung Model MEng Group Project

Date: 16th March 2011

Time: 14:00

Location:  A401

Present: Ben Lewis, Dan Sheehan and Dr Chung

# Apologies: Tom Batterbury, Toby Jackman and Tan Soon Chin

# Discussed:

- Tom’s particle simulations

  - Is it worth continuing now? As it is pretty late in the term

  - What other tasks could Tom take control of to help lighten the loads else where

- Business Section

  - Nearly complete

  - Could do with comparing data from the 1999 and 2004 British Thoracic Society

  - Otherwise will be complete once final polishing of report has been done

- Magic’s Software section

  - There is a simple explanation – maybe Tom could expand

- 1D Simulations

  - Soon and Dan met to look at the simple bifurcation, minor loss

  - There results were an order on 1,000,000 out, Dan has since validated his version from Star-CCM+ with manual calculations, so the issue is thought to be with the spreadsheet

    - Should be using slightly modified Bernoulli equation

      - Ignore ‘h’ (elevation)

      - Velocity is determined by pressure difference

      - Ρ – constant

      - Firstly ignore frictions, so the ‘perfect pipe’ find velocity then look at inputting the friction element as well

    - The following was discussed in the meeting:

U1

∆P=u22-u12-friction

They are dependent on one another therefore:

∆P=∝u12-u12

=(∝-1)u12

U2

Perform this section first to see what velocity

Using:

∆P=100, then veloctiy ≈10ms-1

Following centre line therefore is 2 times other velocities therefore 4u1

2πR1L1μ4u1R

Friction

We only need to know real values we can ignore imaginary numbers.

A resistance function dependent on u1.

From the 3D model we can:

+γu12

So=∝-1u12+βu1+γu12

- Toby

  - Waiting to hear progress, hopefully we will see more progress over Easter

- Results

  - Dan’s results will do, but it will be harder to explain/validate/prove, therefore we will require lots of ‘hand waving’ to explain and put across our views

- Draft report

  - Initial hand-in on Friday but Dr Chung has requested that we hand it in on Tuesday 22nd March

  - Once marked – we will have a meeting with Dr Chung to discuss our feedback, this shouldn’t be too late in the vacation to ensure that we have enough time to prepare for the presentation

- AOB

  - Make sure the introduction is clear – we don’t want assessors asking ‘what is the point of the project?’

  - During the Presentation everyone must speak DO NOT leave it to one or two people, otherwise this does not show that workload has been spread well

  - IMPORTANT – Try to print the report in the last week of the holiday because we don’t have enough time in the first week. The first week back is as follows:

    - Wednesday – start term

    - Thursday – 2nd day back

    - Friday – National Holiday (Royal wedding)

    - Weekend

    - Monday – Bank Holiday

  - Ben will chase Karen Bradbury about the £40 of group credit awarded for poster presentations

  - Dr Chung will be in his office for the majority of the vacation if we have any issues

# Next Meeting:

Group Meeting: Friday 18th March 15:00

# Virtual Lung Model MEng Group Project

Date: 18th March 2011

Time: 15:00

Location:  MEng Project room

Present: Ben Lewis, Dan Sheehan, Tom Batterbury, Toby Jackman and Tan Soon Chin

# Discussed:

- Poster marks have been uploaded to the module webpage

- Oral presentation dates and timings are now available;

  - Week 32, Thursday 12th May, A401, 9-10am

- We are handing in our draft on Tuesday as request by Dr Chung

- Everyone but Soon will be available the majority of the time over the Easter vacation

- Soon

  - Soon’s computer has crashed and he has lost some work, but he has since re-written those sections

  - Soon has re-written the spreadsheet formulae

    - Dan will show Soon what equations/method Dr Chung explained at Wednesdays meeting (16/03/11)

- Toby

  - Re-doing deviations after a meeting with Dr Chung

  - Get results for symmetric fractal geometry by Tuesday

    - Will go into more detail and asymmetric branching with more mesh elements for the final report

- Dan

  - Sent on of our geometry files to Dr Chung, so that he could test them on the new version of Star-CCM+ out recently, version 6

  - Dr Chung has suggested a few other simulations to perform, therefore we will have more comparisons of results

  - We are currently running laminar models

    - This gives a snap-shot of the flow over time, so if there is a turbulence we won’t notice it

    - We therefore need to run an un-steady simulation to see where turbulence is occurring

  - Currently using a tet mesh, as building a hex mesh was too time consuming

    - De Becker also doesn’t agree with the hex mesh

    - Dan will attempt to run a simulation with a hex mesh this weekend and compare the results to see if there is a big contrast or not

- Tom

  - Comparison or the currently available techniques to examine the lung is now complete

    - Will finalise with a table to show the advantages and disadvantages

  - Hasn’t input the glossary yet but will do soon

  - Has tried particle simulations on all of the models, these still aren’t working out – so the group has agreed to drop this aspect

- News just in:

  - Dan has looked at the spreadsheet for Soon, and has fixed one of the issues

- TO DO List (Business Section)

  - Compare/show difference between the 1999 and 2004 British Thoracic paper’s number of deaths (the 2 graphs already included)

  - Input glossary

  - Comparisons of methods

  - Table of contents of the figures and graphs

  - Add more figures if there is enough time

- AOB

  - When editing the draft report it would be ideal to only have one copy so that one version is always up to date, otherwise we could muddle up the sections

    - Therefore, when you are transferring your changes on the web site version, write engaged above the link, so no one else downloads it.

  - We need references for the Navier Stokes method that Soon got from Dr Chung

  - Peer Assessment due TODAY by midnight

# Next Meeting:

Group Meeting: Tuesday 22nd March 09:00

# Virtual Lung Model MEng Group Project

Date: 22nd March onwards

Time: N/A

Location:  MEng Project room

Present: N/A

# Discussed:

Further meetings were conducted to combine work and encourage ourselves to meet deadlines, however it was deemed that this information did not require documenting.

# Next Meeting:

N/A