Analysing experimental data on the long wave dynamics and statistics of shoreline motion

Background

Generally, long surface waves are defined as having wave lengths L much longer than the water depth. These waves exist on scales ranging from tidal waves (L~10,000 km) and tsunamis (L~100 km) in the open ocean, to storm surges (L~10 km) on the continental shelf, down to high speed ship waves (L~100 m) in shallow confined waters with depths of a few dozen meters. Most of the dangerous waves attain unusually large amplitudes as they approach the shoreline (during shoaling and run-up). Freak waves reaching as high as 10 m can wash people from seawalls and breakwaters.

Understanding shoaling and run-up of long waves on a beach is of major importance for understanding the resilience of coastal communities to changing climate and extreme events. One of the major bottlenecks in current understanding is the uncertainty of incident sea wave properties owing to difficulties making detailed observations of the properties of long waves far from the shore. Previous analytical studies show that an asymmetric waves with a steep front (e.g. nonlinearly deformed waves) penetrate a much longer distance inland and with greater velocity than  symmetric ones (Didenkulova et al. 2006b; 2007b).  The influence of nonlinearity on the run-up characteristics for random wave fields climbing the beach has also never been studied in laboratory conditions, although it has recently been suggested theoretically (Didenkulova et al. 2011) that nonlinear effects change the statistical characteristics of run-up height, leading to the increase in the probability of flooding.

Project

This project is about analysing a unique experimental measurement dataset from experiments at the GWK large-scale wave flume in Hannover. Experiments have been carried out on wave run-up at smooth and rough beach. Approximately 100 hours of time series data have been recorded at the 250 metre long and 5 metre wide Large Wave Flume.  Waves of 20s period were recorded by an array of 16 wave gauges, with both regular and random wave fields tested. The experimental dataset includes time-series of wave gauges, wave run-up, near bottom velocity, and force measurements at roughness elements as well as laser-sheet visualisation of water surface profiles at the dyke. Fig.1 shows a schematic of the experimental setup.

Fig.1 – Schematics of experimental setup

This project will focus on developing a non-linear wave equation-based algorithm to describe statistical features of the dataset specific to long strongly nonlinear and weakly dispersive waves. This could be described under the following specific objectives:

- Understanding the wave run-up of nonlinearly deformed Riemann waves;

- Understanding the wave run-up of nonlinearly deformed solitary waves;

- Understanding the wave run-up of random wave fields with Narrow Band and JONSWAP spectra.

The outcome of this project will advance understanding of the physics of run-up, in particular run-up of random wave fields and the run-up in presence of a notable slope roughness. The obtained data would also be an invaluable tool for the code validation for near-shore CFD codes as well as for mathematical theories of non-linear water wave propagation.

Supervisory team

Dr Soroush Abolfathi – School of Engineering

Dr Petr Denissenko – School of Engineering

Dr Ed Brambley – Mathematics Institute