| Title of Assessment | Superconductivity and Magnetism Labs Risk Assessment |  |  | Date of assessment |  | 14/03/19 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Date for review |  | Continuous |
| Department | Physics |  |  |  |  |  |
| Descriptions of Activities | Risk assessment for laboratory work at the University of Warwick site conducted by Matthew Coak pertaining to the research project entitled ‘ExtremeQuantum’ (PI: Dr Paul Goddard). |  |  |  |  |  |
| Name of those working to this assessment | Matthew Coak |  | Any others who may be affected by this assessment |  | Other team members who use the lab spaces and equipment |  |
| Assessment carried out by | Matthew Coak |  |  |  |  |  |

| ##### Additional information |  |
| --- | --- |

| Foreseeable Significant Hazard | Existing control measures | Inherent Risk | Additional control measures | By whom & when | Controlled Risk Level |
| --- | --- | --- | --- | --- | --- |
| Laser exposure - ruby spectrometer | Laser equipment only to be used by trained personnel. Full enclosure fitted to prevent accidental exposure. | Severe | Additional dedicated laser safety training and risk assessment to follow. | User, and (reduced risk) personnel in lab during operation | Low |
| Handling chemical samples – inhalation or ingestion | MSDS to be consulted in each case and appropriate PPE worn and individual measures taken such as se of fume hoods in some cases. Standard lab procedures to be observed - no eating or drinking, hands to be washed after leaving etc. | Minor |  | User, and (reduced risk) personnel in lab during operation | V Low |
| Use of chemical solvents – irritation to skin or eyes | Appropriate PPE and steps to be taken as with safety data sheets, as above. | Low | Any spills must be correctly cleaned, and major spills require department to be notified. All solvents to be correctly disposed of. | User, and (reduced risk) personnel in lab during operation | V Low |
| Laboratory sharps - blades, needles and used glassware | Adherence to safe lab protocol to be followed. Safe storage of sharps, and tidiness of work surfaces to keep hazards clear and identifiable. All sharps to be disposed of in dedicated sharps containers. | Low |  | All personnel in lab environment | Low |
| Use of cryogenic liquids | Suitable ventilation is provided - asphyxiation hazard due to boiling croygenic liquids. Dewars containing gases or cryogenic liquids are not to be accompanied in elevators. Eye and hand protection to be worn when handling croygens. | Major |  | All personnel in lab environment | Low |
| Use of instruments involving large magnetic fields – possible interference with pacemakers or metal implants | Warning signs displayed to warn any entering lab of hazard. Lab is keycode locked to allow only trained users access. Loose metal objects or sensitive electronics to be kept outside safe radius of magnet – this is marked on the floor. | Severe | Non-essential personnel to be kept out of labs with strong magnetic fields active or potentially active. | All personnel in lab environment who may enter 5 Gauss line of magnet in question | Low |
| Use of instruments involving X-ray generation (Ionising radiation source) – possible radiation burns or cellular damage | Diffractometers are located in designated X-ray areas and are commercial controlled setups including interlocking doors and shields to prevent radiation exposure the surroundings, coupled with warning systems to indicate source is in use. Only to be used following proper training. | Severe | Ionising radiation safety course and training on individual instruments to be completed. | User of instrument, and other personnel present while radiation source is in use. | V Low |
| Moveable platform – falls | Brakes to be firmly applied and stability checked before mounting the platform. Overreach or overly strenuous tasks over platform rails to be avoided. Large loads to be lifted with additional aid. | Minor |  | User | Low |
| Spillages and trip hazards | Fluid spillages or trailing wiring etc may constitute slip or trip hazards. Labs to be kept tidy and organised, and any spillages or mess to be cleared promptly. | Minor |  | Any personnel in environment | V Low |

Work should not be carried out until the assessment is completed to a suitable & sufficient level and all required control measures are in place.

Is assessment suitable and sufficient       Yes

| Any further actions required to allow work to commence |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | Paul Goddard |  | Position | Principal Investigator |
| Date | 14th March 2019 |  |  |  |

Please print a copy, sign it and keep for your records

|  | Severity of injury |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  |  |  |
| Unlikely | Very low | Very low | Low | Low | Moderate |  |  |  |
| Possible | Very low | Low | Low | Moderate | High |  |  |  |
| Likely | Low | Low | Moderate | High | Very high |  |  |  |
| Very likely | Low | Moderate | High | Very high | Very high |  | Overall Risk Rating (highest level found) | Low |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.