| Title of Assessment | Safe Use of the Arc Furnace in P122 and laue XRD kit |  |  | Date of assessment |  | 04/01/17 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Date for review |  |  |
| Department | Physics |  |  |  |  |  |
| Descriptions of Activities | Risk assessment for using the arc furnace in P122 to melt together different elements as part of my PhD project. As of 11/10/16, I am just being trained. Actual work will start later in the year, this to be updated then. Also using the Laue XRD kit |  |  |  |  |  |
| Name of those working to this assessment | Amy Walton |  | Any others who may be affected by this assessment |  | Anyone in the lab when the equipment is in use |  |
| Assessment carried out by | Amy Walton |  |  |  |  |  |

| ##### Additional information |  |
| --- | --- |

| Foreseeable Significant Hazard | Existing control measures | Inherent Risk | Additional control measures | By whom & when | Controlled Risk Level |
| --- | --- | --- | --- | --- | --- |
| Arc Furnace-contact with when on can cause minor to severe burns | The equipment is water cooled. Do not attempt to remove sample until cooled to room temperature. | severe | Mandatory training and risk assessments provided, along with a user manual of best practice to be followed | Training before work can start, trained by Dr Martin Lees | low |
| Arc Furnace- looking directly at it can cause burning/blindness | UV shield that fits around equipment to protect user. Also a shield around lab to protect people outside the lab | major | Mandatory training and risk assessments provided, along with a user manual of best practice to be followed | Training before work can start, trained by Dr Martin Lees | v. low |
| Arc Furnace-high pressure Argon gas | Ensure all valves and pipes and gauges are working correctly | severe | Mandatory training and risk assessments provided, along with a user manual of best practice to be followed | Training before work can start, trained by Dr Martin Lees | v. low |
| Chemicals melted in furnace-handling of any reagent can lead to poisoning, irritation, burns etc depending on chemical | Refer to COSHH data for particular chemicals. Wear PPE (safety glasses, lab coat, gloves) appropriate to the chemical. No food/drink in lab. Wash hands after experiments. (To be updated when I know what I’m going to be making!) | v. low |  |  |  |
| Chemicals-Titanium Getter used in furnace | Don’t ingest, don’t handle when hot | v. low | Mandatory training and risk assessments provided, along with a user manual of best practice to be followed | Training before work can start, trained by Dr Martin Lees | v. low |
| Chemicals-acetone, ethanol, propanol used in cleaning-harmful if ingested | Wear appropriate PPE (safety glasses, gloves, lab coat). Don’t ingest. Consult COSHH. | v. low |  |  |  |
| Fire-opening furnace to early may expose samples to oxygen rich atmosphere whilst hot-risk of explosion, fire. | Ensure samples and equipment have cooled before removal. | minor | Mandatory training and risk assessments provided, along with a user manual of best practice to be followed | Training before work can start, trained by Dr Martin Lees | v.low |
| Laue XRD-high levels of radiation can cause burns, cancer, damage eyes | Undergo training by Oleg (done in December 2016). Do not override safety features. Do not use if in doubt. Report any faults before using. Ensure Xray source is off before attempting to open doors. Drive up voltage and currently slowly to avoid damaging equipment. | v. minor |  | Trained by Dr Oleg Petrenko | v. low |

Work should not be carried out until the assessment is completed to a suitable & sufficient level and all required control measures are in place.

Is assessment suitable and sufficient       Yes

| Any further actions required to allow work to commence |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By |  |  | Position |  |
| Date |  |  |  |  |

Please print a copy, sign it and keep for your records

|  | Severity of injury |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  |  |  |
| Unlikely | Very low | Very low | Low | Low | Moderate |  |  |  |
| Possible | Very low | Low | Low | Moderate | High |  |  |  |
| Likely | Low | Low | Moderate | High | Very high |  |  |  |
| Very likely | Low | Moderate | High | Very high | Very high |  | Overall Risk Rating (highest level found) | low |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.