## Risk Assessments for Space – Guidance

A space risk assessment must be completed for each area which contains hazards.  The assessment should identify hazards associated with accessing the space and using general pieces of equipment (including such items as fume cupboards, hot plates and lab specific alarms).  Larger items of equipment should be covered by their own specific risk assessment.

The risk assessment should highlight any training courses which must be completed before a person is granted access to the space.  Please consult the training matrix spreadsheet for courses.

Other outcomes of a space risk assessment should be a set of lab rules to be followed (including any PPE required to enter the space) and the generation of a lab noticeboard which reflects those rules.

A copy of the risk assessment should be maintained in a location (physical or electronic) where it is readily accessible to those who access the space, with an electronic copy also stored on Quemis under the space to which it relates.

## Space Risk Assessment Form

| Title of Risk Assessment | Superconductivity and Magnetism Labs Risk Assessment | Date of assessment |  | 23/02/22 |
| --- | --- | --- | --- | --- |
| Department | Physics | This risk assessment must be reviewed whenever a significant change or an accident occurs |  |  |
| Training Courses Required as Identified by Risk Assessment | Chemical Handling, Cryogenics, High pressure gasses, |  |  |  |
|  | Glove box training, Ionising radiation, Arc melting training |  |  |  |
| Assessment carried out by | Tom Chamberlain |  |  |  |
| Additional information |  |  |  |  |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| General lab assessment Falling / slipping | Self, other staff, students and cleaners | Do not run. Be fully aware of slippery surfaces/obstacles. Raise awareness of wet surfaces where required | L |  | None | L |
| Chemicals | Self, others in the vicinity | Observe all warning labels. Use PPE. Avoid contact with skin. Carry out risk assessments for all chemicals used. | M | Observe all warning labels. Use PPE. Avoid contact with skin. Seek Assistance if unsure. Ensure correct storage. | Follow control measures at all times | L |
| Strong acids and bases | Self and other lab users | Read all relevant safety data sheets, university and Departmental Safety regulations and guidance, and pass any required safety tests. Minimise exposure, use appropriate PPE. | M | Use PPE. Avoid contact with skin. Seek Assistance if unsure. Ensure correct storage. Ensure lab users are aware when acids and bases are being used. | Follow control measures and ensure lab users are aware when acids and bases are in use. | L |
| Use of solvents for cleaning | Self, nearby lab users | Use PPE. Try to avoid direct contact with skin. | VL |  | None | VL |
| Very cold surfaces, liquids or gases | Self, other staff and students | Use PPE. Avoid distractions. Ask if unsure. | L |  | None | L |
| Radiation (x-rays, gamma rays, etc) | Self, nearby lab users | Ensure instrument doors are fully closed before use. Do not but head in beam path! | VL |  | None | VL |
| Electrical system | Self, nearby lab users | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | VL |  | None | VL |
| Food and drink | Self | Do not eat or drink in labs. Wash hands thoroughly after handling chemicals and before eating / drinking | VL |  | None | VL |
| Using tools (hacksaws, etc) | Self, students and other lab users | Use PPE. Avoid distractions. Ask if unsure. | L |  | None | L |
| Storage desiccators under vacuum | Self, other staff/cleaners or students | Do not tamper with desiccators. Vent and pump out with care. Beware of inhaling pumped gas. Tape desiccators. Turn off pump immediately after use. | L |  | None | L |
| Exposure to chemical fumes | Self, other lab users | Use fume hood, PPE. | L |  | None | L |
| Working at height. | Self, nearby lab users | Use kick step with care. Do not use ladders or stools. | VL |  | None | VL |
| MAS1.25 Glove box 1. A leak in the glove box or nitrogen line causes room to be filled with nitrogen. | Individuals, students and staff | 1. The room in equipped with an oxygen sensor to alert all occupants if the oxygen content in the room should fall | Low |  | None | Low |
| 2. Nitrogen valve is left open while antechamber is open to room, causing the room to be filled with nitrogen. | Individuals, students and staff | 2. Nitrogen valve is left open while antechamber is open to room, causing the room to be filled with nitrogen. | Low |  | None | Low |
| 3. Glove box becomes over pressurized. | Individuals, students and staff | The pressure should be monitored regularly by the users and can be controlled by a foot pedal that operates a relief valve. | Low |  | None | Low |
| 4. Exposure to chemicals while using glove box gloves. | Individuals, students and staff | Disposable gloves and a lab coat should be worn when using the glove box | Low |  | None | Low |
| 5.Issues with vacuum pump (see hazards below). | Individuals, students and staff | 5. In addition to controls below, the pump should be turned off if the glove box is unused for an extended period (weekends, holidays, etc.). | Low |  | None | Low |
| Vacuum Pumps 1. Hot surfaces during use, risk of burn to the skin. | Self | Do not touch parts labelled as hot when in use. | VL |  |  | VL |
| 2. Exhaust fumes could be mildly irritant / toxic | Self, other lab users | Check that exhaust is appropriately vented or filtered. Do not tamper with exhaust. | L |  |  | L |
| 3. Risk of glass implosion when pumping glass chambers. | Self, nearby lab users, cleaners | Check that there are no cracks in chambers to be evacuated. Wear appropriate PPE (safety glasses, lab coats, gloves) | L |  |  | L |
| Cryogenics 1. Very cold surfaces | Self | Use PPE. Avoid distractions. Ask if unsure. Give surfaces time to cool after contact with cryogenics | L |  |  | L |
| 2. Very cold liquid / gas | Self, others nearby | Use PPE, Avoid distractions. Ask if unsure. | L |  |  | L |
| 3. Moving large vessels | Self, others nearby | Move with caution. Not fast. | L |  |  | L |
| 4. Gas in atmosphere | Self, other lab/space users | Ensure adequate ventilation. Follow procedure to ensure valves are not mistakenly left open. Do not enter lifts or enclosed spaces with dewars | M | Oxygen sensors fitted in necessary rooms to alert users to danger of reduced oxygen |  | L |
| Handling hot objects Solvents, paper, flammable samples. | Self, nearby individuals | Store chemicals and solvents properly, use solvents sparingly, avoid sources of ignition. | L | Fire extinguishing equipment is readily available. |  | L |
| Hot surfaces / samples | Self | Do not remove samples at T > 300 K. | L |  |  | L |
| Explosion of vacuum/pressure system | Self, nearby lab users, cleaners | Ensure cooling. Received tuition. | M | As before: PPE & gauge. Quartz and chamber were checked for damage. |  | L |
| Broken glassware/metalic wires | Self, other lab users, cleaners | Use PPE. Ensure gauge reads safe pressure/ | VL | None |  | VL |
| Furnaces | Self | take care, dispose of sharps and broken glass in the approved receptacles provided. | L | None |  | L |
| Exposure to chemical fumes | Self, lab users, cleaners | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. | L | None |  | L |
| Furnaces gases. Evaporation of chemicals from crucibles, evaporation of chemicals deposited in the furnace lining | Self, other lab users | Use fume hood, PPE. | VL | Use PPE. Ensure gauge reads safe pressure/ |  |  |
| Box Furnace Explosion of vacuum/pressure system | Self, nearby lab users | Use PPE. Ensure gauge reads safe pressure/ | v low | take care, dispose of sharps and broken glass in the approved receptacles provided. |  | v low |
| Broken glassware/metalic wires | Self, lab users and cleaners | take care, dispose of sharps and broken glass in the approved receptacles provided. | v low | Visual inspection of electric. Do not open any electrical system. Ensure power is off. |  | v low |
| Electrical system | Self | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | v low | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. |  | v low |
| Furnaces | Self | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. | low | Use fume hood, PPE. |  | low |
| Exposure to chemical fumes | Self, other lab users | Use fume hood, PPE. | low | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. |  | low |
| Furnaces gases. Evaporation of chemicals from crucibles, evaporation of chemicals deposited in the furnace lining | Self and other lab users | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. | v low | Ensure water cooling is on whilst equipment is in use. |  | v low |
| Fire due to overheating of equipment. | Self, potentially others in building | Ensure water cooling is on whilst equipment is in use. | v low | Use PPE. Ensure gauge reads safe pressure/ |  | v low |
| Tube furnaces |  |  |  |  |  |  |
| Explosion of vacuum/pressure system | Self, nearby lab users | Use PPE. Ensure gauge reads safe pressure/ | v low | take care, dispose of sharps and broken glass in the approved receptacles provided. |  | v low |
| Broken glassware/metalic wires | Self, lab users and cleaners | take care, dispose of sharps and broken glass in the approved receptacles provided. | v low | Visual inspection of electric. Do not open any electrical system. Ensure power is off. |  | v low |
| Electrical system | Self | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | v low | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. |  | v low |
| Furnaces | Self | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. | low | Use fume hood, PPE. |  | low |
| Exposure to chemical fumes | Self, other lab users | Use fume hood, PPE. | low | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. |  | low |
| Furnaces gases. Evaporation of chemicals from crucibles, evaporation of chemicals deposited in the furnace lining | Self and other lab users | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. | v low | Ensure water cooling is on whilst equipment is in use. |  | v low |
| Bridgman Furnaces |  |  |  |  |  |  |
| Explosion of vacuum/pressure system | Self, nearby lab users | Use PPE. Ensure gauge reads safe pressure/ | v low | take care, dispose of sharps and broken glass in the approved receptacles provided. |  | v low |
| Broken glassware/metalic wires | Self, lab users and cleaners | take care, dispose of sharps and broken glass in the approved receptacles provided. | v low | Visual inspection of electric. Do not open any electrical system. Ensure power is off. |  | v low |
| Electrical system | Self | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | v low | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. |  | v low |
| Furnaces | Self | Do not open the furnace doors when in use and wait until the furnace is cooled down before removing sample. | low | Use fume hood, PPE. |  | low |
| Exposure to chemical fumes | Self, other lab users | Use fume hood, PPE. | low | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. |  | low |
| Furnaces gases. Evaporation of chemicals from crucibles, evaporation of chemicals deposited in the furnace lining | Self and other lab users | Clearly indicate on furnace user sheet the nature of samples and the treatment to be followed. Do not modify gas ventilation systems unless indicated by risk assessment. | v low | Ensure water cooling is on whilst equipment is in use. |  | v low |
| Arc melting Hot surfaces / samples | Self, other lab users | Ensure cooling. Received training. | v low |  |  | v low |
| Brightness of arc | Self, people nearby | Use PPE and safety curtain. Avoid distractions. Wear goggles and insulating gloves | v low |  |  | v low |
| Cutting titanium or other metal samples. | Self, Cleaners | Take care. Avoid distractions. | v low |  |  | v low |
| Explosion of vacuum system | Self, nearby lab users | Use PPE. Ensure gauge reads safe pressure/ | v low |  |  | v low |
| Electrical system | Self, nearby lab users | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | v low |  |  | v low |
| UV Burn | Self, others nearby | wear long sleeves, and welding gloves. Avoid prolonged use. Use safety curtain | low |  |  | low |
| Arc shock | Self | Wear welding gloves. Do not wear metal jewellery or metal watches on wrists. Wear rubber soled shoes | v low |  |  | v low |
| Use of ethanol for cleaning | Self | Use PPE. Try to avoid direct contact with skin. | v low |  |  | v low |
| VSM, SQUID and PPMS Liquid Nitrogen for SQUID/VSM/PPMS |  | Use PPE, Avoid distractions. Ask if unsure. | v low |  |  | v low |
| Electrical system |  | Visual inspection of electric. Do not open any electrical system. Ensure power is off. | v low |  |  | v low |
| Magnetic field |  | Follow precaution signs | v low |  |  | v low |

Work should not be carried out until the assessment is completed to a suitable & sufficient level and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) |  | Low |  |
| --- | --- | --- | --- |
| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |  |
| Any further actions required to allow work to commence |  |  |  |  |  |
| Approved By | Geetha Balakrishnan |  |  | Position | Professor |
| Date | 8 March 2022 |  |  |  |  |

Please print a copy, sign it and keep for your records

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.