Risk Assessment Form

| Title of Risk Assessment | Experimental work in P422 P423A P424 |  |  | Date of assessment |  | 06/02/2018 |
| --- | --- | --- | --- | --- | --- | --- |
| Department | Physics |  |  | Date review due |  |  |
| Description of Task/Process | Surface science experiments in physics labs P422 P423A P424 |  |  |  |  |  |
| Assessment carried out by | Gavin Bell |  |  |  |  |  |

| ##### Additional information | Small amounts of solvents (acetone, isopropanol, ethanol) are kept in P422, P424 and P423A for general sample cleaning. Acids are kept under the fume hood in P426 and may also be used there for surface etching / cleaning. Other chemicals are in solid form (lump/powder/rod for effusion cells, e.g. As, Sb, Mn) and are only used inside vacuum chambers. Some metal foil and wire materials are kept in P427 and P423A (e.g. W, Ta). Gas cylinders are kept in P422 and P424 (hydrogen, argon). Smaller canisters are used where possible. Only trained and signed-off users may work on the vacuum systems in these labs: update of personal risk assessments are a mandatory part of the training. LAB RULES are given here: https://warwick.ac.uk/fac/sci/physics/staff/academic/bell/lab/safety/ |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Chemicals used in sample cleaning, etching and spin-coating (acids, solvents, PEI, PEIE, KI): could cause skin burns, irritation to skin or eyes. Fire risk with solvents | Experimenter, other lab users, staff | Complete online chemical safety training if required by each specific process; wear suitable PPE; no food and drink in labs; ensure familiarity with all safety data sheets; store chemicals only in specified locations with proper labelling; minimise amount of chemicals kept locally; use drip trays for all wash bottles. Ensure no heat sources in vicinity when solvents in use. | M | Ensure spills are cleaned, waste chemicals disposed of properly (see below) and work benches & fume cupboard in P426 left clean and tidy; larger spills must be immediately reported to technical staff; refer to LAB RULES. | All lab users prior to performing each new chemical process. | L |
| Chemicals used inside vacuum systems (MBE source materials including Ni, Sb, Mn, As, Sm): may be harmful or toxic via ingestion or inhalation | Experimenter, other lab users | As above plus: only open up vacuum systems with supervision and planning; only use specific benches in P422 and P424 for effusion cell work; ensure all effusion cells and crucibles are tracked and labelled. | M | Only trained users may operate vacuum systems; do not tamper with exhaust lines; shut down effusion cells and report immediately to technical staff if a chamber is accidentally vented to air; refer to LAB RULES. | All vacuum chamber users during regular experiments. | L |
| Preparing substrates for vacuum deposition: may cause minor injury (burn / cut) or harm by dust inhalation | Experimenter, other lab users | Only use flow cabinet in P422 for substrate preparation; store sharp tools safely; wear suitable PPE including gloves & dust mask for sample cleaving; take care using hot plate for indium soldering. | M | Spot welder in P426 may be used with Ta wire to secure samples to carrier plated: only use this once trained; keep spot welder bench tidy (no sample fragments left etc.); refer to LAB RULES. | All vacuum chamber users during regular experiments. | L |
| Chemical disposal: may cause physical harm or damage to environment | Personnel, general public (environment) | Only use dedicated disposal bins for solvents, PEI, PEIE, KCl; alert technical staff if bins are getting full (should not happen for years!). | M / L | Dispose of all sharps in P423A bin and all substrate wafer fragments in waste wafer bin in P422; refer to LAB RULES. | All lab users during regular experiments. | VL |
| Gas cylinders: may cause injury, damage to gas lines or fire hazard | Experimenter, other lab users, staff | Only operate large cylinders and small canisters once trained in “gas admittance” as detailed on web page; do not tamper with regulators. | M / L | Alert technical staff if any regulator problem is suspected; observe schedule of regulator replacement; refer to LAB RULES. | Technical staff and space owner to check regulators. | VL |
| Trip hazards: may cause injury or damage to equipment | Experimenter, other lab users, other staff | Keep lab uncluttered, including coats and bags; take care to route cables securely using “false floor” sections where appropriate. | L |  |  |  |
| Cryogens: filling and transferring liquid nitrogen dewar and use of liquid nitrogen in labs; may cause burns or asphyxiation | Experimenter, other lab users, other staff | Nitrogen dewar fills are done outside in the physics yard; be aware of unguarded drop in Physics yard area; dewars must not be accompanied in the goods lift and signage used to prevent access during transit; store dewar in P427 when not in use. | M | Department training must be completed before liquid nitrogen use (E. Woodward). | Staff and students when trained for low temperature Hall and some vacuum systems. | VL |
| High voltages: up to 20kV may cause lethal shock | Experimenter | High voltages are present on some vacuum chamber equipment; only trained users on chambers – follow established procedures; do not tamper with any racks, vacuum feedthroughs or power supplies; if any electrical problem is suspected shut down if safe to do so and consult the Electronics Workshop. | VL |  |  |  |
| X-rays: may cause harm through exposure | Experimenter | X-ray source in P424 produces Al K-alpha radiation (1.4 keV photons); X-rays are confined inside vacuum system; do not remove or tamper with leaded glass over vacuum windows. | VL |  |  |  |
| Manual handling: sometimes heavy equipment must be moved which may cause injury to staff or damage to equipment | Experimenter, staff assisting | Plan any manual handling task in advance; do not overload the group’s trolley; consider any assistance required, PPE such as safety boots and high grip gloves, whether mechanical assistance is needed (if so consult technical staff); do not add to risk by unsafe storage of heavy items (e.g. on unsuitable shelves); refer to LAB RULES. | L |  |  |  |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | L |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By |  |  | Position |  |
| Date |  |  |  |  |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.