Risk Assessment Form – RA-EMX+EMX-E_V2.1

| Title of Assessment | Bruker EMX/EMX-E EPR Spectrometers |  |  | Date of assessment |  | 04/01/2018 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Date of review |  | 25/01/2019 |
| Department | Physics |  |  | Date for review |  | 25/01/2020 |
| Descriptions of Activities | Use of the Bruker EMX/EMX-E EPR spectrometers located in Millburn house room G78 |  |  |  |  |  |
| Name of those working to this assessment | Trained EPR spectrometer users listed in the px-epr-users webgroup. |  | Any others who may be affected by this assessment |  | All other lab users |  |
| Assessment carried out by | Ben Breeze |  |  |  |  |  |

| ##### Additional information | Users are responsible for having performed a risk assessment for their samples Users are responsible for having performed a risk assessment for their samples which must be logged at: https://warwick.ac.uk/research/rtp/spectroscopy/Safety/Sample When booking all users will provide a reference to the risk assessment for the sample used. Bookings are restricted to trained users who are recorded in the px-epr-users web group. |
| --- | --- |

| Foreseeable Significant Hazard | Hazard Risk | Existing control measures | Controlled residual risk | Further Action where risk remains moderate/high | By whom & when | Controlled Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Mains electricity | Electric shock and/or burns | Equipment should not be tampered with. All equipment requires an in-date PAT test. | (Major, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |
| Three-phase electricity | Electric shock and/or burns | Three-phase power supply/cables should not be tampered with. Visual inspection should be regularly carried out. | (Major, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |
| Water leak | Electric shock and slips | Water cooling pipes/fitting should be inspected for leaks. Flooding sensors are located at all the spectrometers, which cut the water supply if a water leak is detected. Lab flooding procedure (c.f. general risk assessment) should be followed. | (Major, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |
| Microwave source | Burns to eyes/skin | Microwave power should not be on if waveguide is open. Waveguide seals should be properly fitted and replaced if damaged. Goniometers should only be removed from the cavity when spectrometer is in Standby mode. | (Serious, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |
| Magnetic field | Bruising, ill-health, failure of electronic health devices | Magnetic items should not be held within the 5 G line All users, and especially users with pace-makers/other electronic health devices, should not cross the 5 G line which is the yoke of the spectrometer. | (Major, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |
| Moving parts | Trap fingers, hair/clothing | Be aware of risk of rotating goniometer arm during use – do not lean into spectrometer. Loose clothing should be avoided and long hair should be tied back. | (Minor, Possible) Low | None | Ben Breeze 04/01/18 | Low |
| Broken glassware | Cuts | Take care, dispose of sharps and broken glass in the approved receptacles provided. | (Minor, Likely) Low | None | Ben Breeze 04/01/18 | Low |
| Obstructed walkways | Slip, fall | Do not block walkways and be aware of risk | (Minor, Possible) Low | None | Ben Breeze 04/01/18 | Low |
| Dirt or polluted work area | Poisoning, eye and skin irritation | Keep work areas clean and tidy | (Minor, Possible) Low | None | Bene Breeze 04/01/18 | Low |
| Cryogenics | Inhalation or ingestion causing loss of consciousness, asphyxiation or Burns. | Read all relevant safety data sheets, University and Departmental Safety regulations and guidance, and pass any required safety tests. | (Major, Unlikely) Low | None | Ben Breeze 04/01/18 | Low |

Work should not be carried out until the assessment is completed to a suitable & sufficient level and all required control measures are in place.

Is assessment suitable and sufficient       Yes

| Any further actions required to allow work to commence |  | None |  |  |
| --- | --- | --- | --- | --- |
| Approved By | Ben Breeze |  | Position | RTP - Manager |
| Date | 25/01/2019 |  |  |  |

Please print a copy, sign it and keep for your records

|  | Severity of injury |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  |  |  |
| Unlikely | Very low | Very low | Low | Low | Moderate |  |  |  |
| Possible | Very low | Low | Low | Moderate | High |  |  |  |
| Likely | Low | Low | Moderate | High | Very high |  |  |  |
| Very likely | Low | Moderate | High | Very high | Very high |  | Overall Risk Rating (highest level found) | Low |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.