Risk Assessment Form – RA-Nicolet-iN10

| Title of Assessment | Thermo Fisher Nicolet iN10 |  |  | Date of assessment |  | 07/12/2020 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Date for review |  |  | 07/01/2022 |  |
| Department |  | Physics |  |  |  | Date for review |  |  |  |  |
| Descriptions of Activities | Use of the Thermo Fisher Nicolet iN10 FT-IR microscope |  |  |  |  |  |
| Name of those working to this assessment | Trained Spectrometer Operators Listed in the px-optical-absorption-users webgroup |  | Any others who may be affected by this assessment |  | All other Lab users |  |
| Assessment carried out by | Ben Breeze |  |  |  |  |  |

| ##### Additional information | For details of general lab risk assessment please see appropriate. A separate Risk assessment must be carried out by the user for each sample being used these must be logged on the online form prior to experiments https://warwick.ac.uk/research/rtp/spectroscopy/Safety/Sample These individual sample assessments are available from B Breeze or M Newton on request. |
| --- | --- |

| Foreseeable Significant Hazard | Hazard Risk | Existing control measures | Controlled residual risk | Further Action where risk remains moderate/high | By whom & when | Controlled Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Main electricity. | Electric shock or burns. | Users are not to tamper with the equipment. All equipment must have an in date Pat test. | (Major, Unlikely) Low | none | Ben Breeze 07/12/20 | Low |
| IR light Source. | Irritation to eyes/skin | Users are not to remove the covers from the spectrometer. Spectrometer door should be in place during operation. Light is low power. | (Minor, Unlikely) Very Low | none | Ben Breeze 07/12/20 | Very Low |
| Cryogenics | Inhalation or ingestion causing loss of consciousness, asphyxiation or Burns. | All users must have completed the University and Departmental Safety regulations and guidance on Cryogenics, and pass any required safety tests. Appropriate PPE (googles and gloves) must be worn at all time whilst handling cryogenics. Work to be conducted in well ventilated room with Oxygen level monitoring | (Major, Unlikely) Low | none | Ben Breeze 07/12/20 | Low |
| Broken Glassware | Cuts | Take care, dispose of sharps and broken glass in the approved receptacles provided. | (Minor, Likely) Low | none | Ben Breeze 07/12/20 | Low |
| Obstructed walkways | Slip, Fall | Do not block walkways, be aware of risk | (Minor, Possible) Low | none | Ben Breeze 07/12/20 | Low |
| Dirt or polluted work area | Poisoning, eye and skin irritation | Keep all work areas clean and tidy | (Minor, Possible) Low | none | Ben Breeze 07/12/20 | Low |

Work should not be carried out until the assessment is completed to a suitable & sufficient level and all required control measures are in place.

Is assessment suitable and sufficient       Yes

| Any further actions required to allow work to commence |  | None |  |  |
| --- | --- | --- | --- | --- |
| Approved By | Mark Newton |  | Position | RTP-Manager |
| Date | 07/12/20 |  |  |  |

Please print a copy, sign it and keep for your records

|  | Severity of injury |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  |  |  |
| Unlikely | Very low | Very low | Low | Low | Moderate |  |  |  |
| Possible | Very low | Low | Low | Moderate | High |  |  |  |
| Likely | Low | Low | Moderate | High | Very high |  |  |  |
| Very likely | Low | Moderate | High | Very high | Very high |  | Overall Risk Rating (highest level found) | Low |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.