| Substance | Oxygen (Compressed gas) |  | UN No: 1072 Transport Cat: 2.2, 5.1 |  | University Hazard Classification |  | Medium |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Oxy/acetylene welding, bio research, analytical equipment requiring O2. (DOES NOT cover use of medical oxygen) |  |  |  |  |  |  |
| Handling Requirements e.g. fume hood, MSC, glove type etc. | - If used in laboratory, you must ensure there is sufficient air changes to minimise risk of oxygen enrichment. Odour threshold is subjective and is inadequate to warn of over exposure. - Gas detection must be used where the level is likely to go above 23.5%. - Ensure correct regulator is used. - Keep valves and fittings free from oil and grease. Only use oxygen approved lubricants and sealants. - Close container valve after each use and when empty, even if still connected to equipment. - Replace valve outlet caps or plugs and container caps where supplied as soon as container is disconnected from equipment. - Follow suppliers handling instructions. - In case of fire; stop the leak only if safe to do so. |  |  |  |  |  |  |
| Laboratory Info | Appropriate to department/school e.g. containment level, room number etc. |  |  |  |  |  |  |
| Additional Information | - Violently oxidises organic material. - Even materials considered to be non-flammable will combust in increased oxygen atmospheres. - May react violently with combustible materials. - May react violently with reducing agents. - Consider the potential toxicity hazard due to the presence of chlorinated or fluorinated polymers in high pressure (>30 bar) oxygen lines and equipment in case of combustion. |  |  |  |  |  |  |
| Hazard Statements H270 H280 | High concentrations may cause asphyxiation Continuous inhalation of concentrations higher than 75% may cause nausea, dizziness, respiratory difficulty, convulsion and/or death. | May cause or intensify fire. |  | Gas under pressure, may explode if heated. |  |  |  |
| Handling Precautions Always follow supplier’s instructions. | Wear general handling gloves (EN 388). | Wear safety goggles (EN 166) |  | Wear Protective footwear (EN ISO 20345). |  | Do not smoke. Do not use near naked flame or other heat source. |  |
| Storage & Transport | - Containers should not be stored in conditions likely to encourage corrosion. - Stored containers should be periodically checked for general conditions and leakage. Damaged valves should be reported immediately to the supplier. - Container valve guards or caps should be in place and cylinder firmly secured to prevent falling. - Store containers in location free from fire risk and away from sources of heat and ignition. - Keep away from combustible material. - Avoid asphalted locations for storage, transfer and use (ignition risk if spilt). - Segregate from flammable gases, flammable materials and other sources of ignition including static discharge when being stored. - Store below 50°C and in a well-ventilated space. - Avoid transport on vehicles where the load space is not separated from the driver's compartment and follow departmental SOP. |  |  |  |  |  |  |
| Spillage and Waste Disposal | - Prevent spreading of vapour through sewers and ventilation systems. - Provide adequate ventilation. - Monitor the concentration of the released product - Return empty cylinders to supplier. - Damaged cylinders should only be handled by specialists. |  |  |  |  |  |  |
| First Aid | EYES: Rapidly expanding gas may cause frost bite. Seek medical advice. |  |  |  |  |  |  |
|  | SKIN: Remove any contaminated clothing, take care if clothing has become frozen. Wash affected area thoroughly with water. Seek medical attention |  |  |  |  |  |  |
|  | INHALATION: Remove to fresh air, keep warm and rest. Seek medical attention without delay. |  |  |  |  |  |  |
|  | INGESTION: Not a known route of entry. |  |  |  |  |  |  |