| Substance | Cleaning Agents (Non-Lab) |  | UN No: Dependant on the product Transport Cat: |  | University Hazard Classification |  | Medium |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | General cleaning and surface preparation - This note does not apply to materials regarded as corrosive |  |  |  |  |  |  |
| Handling Requirements e.g. fume hood, MSC, glove type etc. | - Use in a well- ventilated area. - Protect skin by using pre-work creams. - Wash hands after use |  |  |  |  |  |  |
| Laboratory Info | Appropriate to the Department (Lab number and containment level where appropriate) |  |  |  |  |  |  |
| Additional Information | - Do not use outside the manufacturer’s recommendations - DO NOT mix with other chemicals unless instructed to do so by the manufacturer. - Prolonged or repeated contact may cause dermatitis - Any change in health status you think may be related to using these products must be reported to your supervisor/line manager immediately and an occupational health referral made. |  |  |  |  |  |  |
| Hazard Statements H315 H319 H302 EUH066 H400 | Irritating to the Skin and eyes Skin/eye reddening and soreness. Excessive eye watering | Harmful if swallowed May cause sickness and feeling unwell |  | Very toxic to aquatic life |  | Do not smoke. Do not use near naked flame or other heat source. |  |
| Handling Precautions Follow advice given in the section “Good Hygiene Practice” | If there is prolonged contact wear gloves when handling any cleaning chemical (EN 388). Remove gloves before touching door handles | Wear protective overalls when handling large quantities Heavily contaminated clothing should be changed and must not be worn again until it has been thoroughly washed. |  | If there is a risk of splashing wear eye protection (EN166). |  |  |  |
| Storage & Transport | - Store in original containers. - Some cleaning agents may react with acids – always read the manufacturers guidelines - Transport in a secure manner in line with manufacturers guidelines. |  |  |  |  |  |  |
| Spillage and Waste Disposal | - Use all PPE stated above. - Large spillages should be absorbed using absorbent material and placed in a plastic bag for disposal with other hazardous waste in line with departmental procedures. Neat solutions must not be sent to drain. - Small spillages should be diluted and flushed to drain. |  |  |  |  |  |  |
| First Aid | EYES: Rinse immediately with eye wash solution for at least fifteen minutes. Remove contact lens if safe to do so. Seek medical attention if discomfort persists. |  |  |  |  |  |  |
|  | SKIN: Remove any contaminated clothing. Wash affected area thoroughly with soap and water. Seek medical attention if discomfort persists. |  |  |  |  |  |  |
|  | INHALATION: Not usually a route of exposure. Move to a well-ventilated area. If symptoms persist seek immediate medical attention. |  |  |  |  |  |  |
|  | INGESTION: Rinse mouth thoroughly with sterile water. Seek immediate medical attention. |  |  |  |  |  |  |