| Substance | Methanol/Methyl Alcohol |  | UN No: UN1230 Transport Cat: Class 3, Cat 6.1 |  | University Hazard Classification |  | Medium |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Chemical reactions, denaturing proteins, glassware washing, permeabilisation of cells, HPLC, Coomassie staining |  |  |  |  |  |  |
| Handling Requirements e.g. fume cupboard, MSC, glove type etc. | - When carrying out the risk assessment for the use of the material, ensure consideration is given to the concentration and volume being used - Methanol should be handled in a fume cupboard (LEV) due to inhalation risk (avoid breathing in mist and vapours). - Avoid contact with skin and eyes; wear protective butyl-rubber or Viton gloves (EN374) and eyewear (EN166). |  |  |  |  |  |  |
| Laboratory Info | Appropriate to the department (state lab number and containment level where appropriate) |  |  |  |  |  |  |
| Additional Information | - Do Not exceed workplace exposure limits - Long Term Exposure Limit (8H time weighted average (TWA)) – 200ppm/266mg-m3 - Short Term Exposure Limit (15 min ref. period) – 250ppm/333mg-m3 - Flash point is 11°C i.e. the liquid gives off sufficient vapour at 11°C to ignite if a flame or spark is applied - Vapours may travel to the source of the ignition and flash back - Methanol vapours may be heavier than air. They will spread along the ground and collect and stay in poorly-ventilated, low-lying, or confined areas (e.g., sewers, basements, and tanks). - Liquid methanol is lighter than water - Carbon oxides can be produced during a fire |  |  |  |  |  |  |
| Hazard Statements H225 H301 H311 H331 H370 | Highly flammable liquid and vapour | Toxic if swallowed, inhaled or in contact with skin |  | Causes specific organ damage (Eyes) |  |  |  |
| Handling Precautions P210 P280 P301 + P310 + P330 P302 + P352 + P312 P304 + P340 + P311 | Wear appropriate chemical resistant safety gloves (EN 374) | Wear close fitting eye protection (EN 166) |  | Wear protective clothing e.g. fastened lab coat |  | Do not smoke. Do not use near naked flame or other heat source, sources of ignition. |  |
| Storage & Transport | - Keep container tightly closed in a cool, dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage - Highly/extremely flammable solids and liquids may be stored together, in a vented fire-resistant cabinet. Segregation according to secondary hazards is advised. - Ensure storage cabinet is suitably earthed. - Possible hazardous chemical reactions with: Oxidizing agents, Halogens, sodium hypochlorite, sulphuric acid, nitrogen oxides, chlorates, chromium(VI) oxide, chromosulfuric acid, halogen oxides, hydrides, salts of oxyhalogenic acids, perchlorates, perchloric acid, permanganic acid, hydrogen peroxide, zinc diethyl, nonmetallic oxides, powdered magnesium, Nitric acid. - Creates exothermic reaction with: Acids, Chloroform, Acid anhydrides, Reducing agents, Bromine, Chlorine, tetrachloromethane, acid halides, magnesium - Risk of ignition or formation of inflammable gases or vapours with: Fluorine, Oxides of phosphorus, Raney nickel - Generates dangerous gases or fumes in contact with: Alkali metals, Alkaline earth metals - Must NOT be stored with oxidisers, Magnesium, zinc alloys and various plastics |  |  |  |  |  |  |
| Spillage and Waste Disposal | Spills Ensure suitable arrangements exist to deal with all foreseeable spillage situations. Give due consideration to the concentrations and volumes involved. Spillage inside an LEV: - For a spill of liquid absorb into inert material. Dispose as hazardous waste. Do not allow product to enter drains. Spillage outside an LEV: - Evacuate all personnel from the lab. - Ensure adequate ventilation. - Discuss with competent person as whether it is suitable to allow the spill to dissipate with increased ventilation (small spill <100 ml, lab closed), or whether absorption is required. See below. - Wear respiratory protection (AXBEK (EN 14387)). Avoid breathing vapours, mist or gas. - Contain spill and do not touch or walk through the spilled agent - Remove all sources of ignition. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas. - Following manufacturers guidelines, absorb spill with inert material e.g. Spill-X-S®, Trivorex® and allow recommended time to absorb. Transfer waste to plastic bags or containers, label and dispose of as hazardous waste. Add more absorbent material if required. Once neutralised, wash down area with copious amounts of water and treat all waste as hazardous. Do not allow product to enter drains. Waste: - Dispose of hazardous waste in line with written departmental procedures. |  |  |  |  |  |  |
| First Aid | EYES: Rinse thoroughly with plenty of water for at least 15 minutes and seek medical attention. |  |  |  |  |  |  |
|  | SKIN: Wash off with soap and plenty of water. Seek immediate medical attention. |  |  |  |  |  |  |
|  | INHALATION: If breathed in, move person into fresh air. If not breathing, give artificial respiration. Seek immediate medical attention |  |  |  |  |  |  |
|  | INGESTION: After swallowing: fresh air. Make victim drink ethanol (e.g. 1 drinking glass of a 40% alcoholic beverage). Call a doctor immediately (mention methanol ingestion). Only in exceptional cases, if no medical care is available within one hour, induce vomiting (only in fully conscious persons) and make victim drink ethanol again (approx. 0.3 ml of a 40% approx. 0.3 ml of a 40% alcoholic beverage/kg body weight/hour). |  |  |  |  |  |  |