| Substance | Acetic (Ethanoic) and Formic (Methanoic) Acid |  | UN No: UN2789/ UN1779 Transport Cat: 8 (class 3) |  | University Hazard Classification |  | Medium |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Etching, Mass Spectrometry, Chromatography, de-staining gels, chemical reactions |  |  |  |  |  |  |
| Handling Requirements e.g. fume cupboard, MSC, glove type etc. | - Solutions should be handled in a fume cupboard (Local Exhaust Ventilation (LEV)) - Wear protective butyl-rubber/neoprene gloves (EN374) and tight fitting eyewear (EN166) when handling solutions. - Adhere to glove breakthrough times. - When carrying out the risk assessment for the use of the material, ensure consideration is given to the concentration and volume being used. - Follow local rules at all times. |  |  |  |  |  |  |
| Laboratory Info | Appropriate to the department (state lab number and containment level where appropriate) |  |  |  |  |  |  |
| Additional Information | - Workplace exposure limits must NOT be exceeded - Acetic Acid: STEL: 15 ppm/ 37 mg-m3 / (15 min); TWA: 10 ppm (8h) - Formic Acid: STEL: 15ppm/28.8 mg-m3 (15 min); TWA: 5ppm/9.6mg -m3 (8h) - Thermal decomposition can lead to release of irritating gases and vapours (do not heat outside of a fume cupboard). - Lachrymator (substance which increases the flow of tears). - First Aiders MUST NOT give mouth to mouth resuscitation if these acids have been ingested or inhaled; compressions only, if required. - During a fire carbon monoxide, carbon dioxide and hydrogen can be produced - Formic acid only- Flammable. Containers may explode when heated. Vapours may form explosive mixtures with air. Vapours may travel to source of ignition and flash back. |  |  |  |  |  |  |
| Hazard Statements H226 H314 H318 H302 H331 EUH071 | Causes severe skin burns and eye damage Corrosive to the respiratory tract | Toxic if inhaled |  | Harmful if swallowed |  | Flammable liquid and vapour |  |
| Handling Precautions P210 P303+P361+P353 P280 P301+P330+P331 P305+P351+P338 P310 | Wear chemical resistant gloves (EN374) | Wear close fitting eye protection (EN166 ) |  | Wear protective clothing e.g. fastened lab coat |  | Insert appropriate safety symbols |  |
| Storage & Transport | - (Highly/extremely) flammable solids and liquids may be stored together, in a vented fire-resistant cabinet. Segregation according to secondary hazards is advised. - Keep away from heat and sources of ignition. - Keep containers tightly closed in a dry, cool and well-ventilated place. - Take precautionary measures against static discharges - Incompatible with: strong oxidizing agents, strong bases, metals, powdered metals. - Formic acid only - containers should be vented periodically in order to overcome pressure build-up. Hygroscopic, heat sensitive, Decomposes to water and carbon dioxide - Transport between labs in secondary containment e.g. Winchester carrier |  |  |  |  |  |  |
| Spillage and Waste Disposal | Spills: Local arrangements should be in place to deal with all foreseeable spillage situations. Give consideration to volume and concentration of material spilt. Spillage inside an LEV: - For a spill of liquid, dilute with excess water and absorb into inert material. Dispose as hazardous waste. Do not allow product to enter drains. Spillage outside an LEV: - Evacuate all personnel from the lab. - Ensure adequate ventilation. - Wear respiratory protection (Type E (EN 14387)). Avoid breathing vapours, mist or gas. - Contain spill and do not touch or walk through the spilled agent - Remove all sources of ignition. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas. - Following manufacturers guidelines, absorb spill with inert material e.g. Spill-X-A®, Trivorex® and allow recommended time to absorb. Transfer waste to plastic bags or containers, label and dispose of as hazardous waste. Add more absorbent material if required. Once neutralised, wash down area with copious amounts of water and treat all waste as hazardous. Do not allow product to enter drains. Waste: - Dispose of hazardous waste in line with written departmental procedures. |  |  |  |  |  |  |
| First Aid | EYES: In the case of contact with eyes, rinse immediately with plenty of water and seek medical attention |  |  |  |  |  |  |
|  | SKIN: Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention |  |  |  |  |  |  |
|  | INHALATION: Do not induce vomiting. Seek immediate medical attention. |  |  |  |  |  |  |
|  | INGESTION: Do not use mouth-to-mouth method if victim ingested or inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper respiratory medical device. Move to fresh air. Immediate medical attention is required. If not breathing, give artificial respiration |  |  |  |  |  |  |