| Substance | Acrylamide, Bis-acrylamide (liquid only) |  | UN No: ADR/RID 2074 Transport Cat: 6.1 |  | University Hazard Classification |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Dispensing and preparation of gels for electrophoresis, chromatography |  |  |  |  |  |  |
| Handling Requirements e.g. fume cupboard, MSC, glove type etc. | Do not autoclave acrylamide. - Liquid solutions or precast polyacrylamide gels should be used where possible and powdered acrylamide should be avoided (see separate SIN for powder). - Wear nitrile gloves (EN374) when handling. Breakthrough times must be strictly adhered to. - A fume cupboard (LEV) must be used when handling liquid solutions. - Always follow manufacturer’s instructions. - Operators must be trained to carry out task. |  |  |  |  |  |  |
| Laboratory Info | Appropriate to the department (lab number and containment level where appropriate) |  |  |  |  |  |  |
| Additional Information | - Acrylamide Workplace exposure limit (WEL): 0.3mg.m-3. - Pregnant workers should refrain from working with this material. - Carbon oxides, nitrogen oxides (NOx) can be formed during a fire. Exposure limits must not be exceeded. |  |  |  |  |  |  |
| Hazard Statements H301 H312 H302 + H332 H315 H317 H319 H340 H350 H361f H372 | Harmful in contact with skin or if inhaled. Causes skin irritation. Causes serious eye irritation. May cause an allergic skin reaction. | May cause genetic defects. May cause cancer. Causes damage to organs through prolonged or repeated exposure e.g. peripheral nervous system. |  | Toxic if swallowed, inhaled or through the skin Reproductive toxicity -Suspected of damaging fertility. |  |  |  |
| Handling Precautions P201, P260, P264, P280, P301+P310+P330, P308+P313 | Wear safety goggles, when risk of splash into eye (EN166). | Wear nitrile gloves (EN374) |  | Wear protective clothing e.g. fastened lab coat. |  |  |  |
| Storage & Transport | - Do not store with acids, oxidising agents, iron and iron salts, copper, brass, free radical initiators - Store in cool, dry, well-ventilated chemical cabinet. - Light sensitive – keep in secondary container - Store under inert gas - Stable under recommended storage conditions |  |  |  |  |  |  |
| Spillage and Waste Disposal | Spill inside an LEV - Ensure fume cupboard is working - Use absorbent to contain liquid – DO NOT wash to drain - Transfer into plastic bags or containers, label and dispose of as hazardous waste in line with departmental arrangements. Spillage outside an LEV - Evacuate personnel to a safe area and trained users don respiratory protection with chemical absorbing filters (for example 3M - ABEK1P3). - Prevent further leakage or spillage if safe to do so. - Do not let product enter drains. Discharge into the environment must be avoided. - Following manufacturers guidelines, absorb spill with inert material. Transfer to plastic bags or containers, label and dispose of as hazardous waste. Waste: Dispose of hazardous waste in line with written departmental procedures. |  |  |  |  |  |  |
| First Aid | EYES: Rinse immediately with plenty of clean running water for at least 15 minutes. Seek medical advice. |  |  |  |  |  |  |
|  | SKIN: Wash well with soap and water. Seek medical advice. |  |  |  |  |  |  |
|  | INHALATION: Remove to fresh air. If not breathing give artificial respiration. Immediately seek medical advice. |  |  |  |  |  |  |
|  | INGESTION: Immediately call the emergency services and seek medical attention. |  |  |  |  |  |  |