| Substance | DIESEL |  | UN No: 1202 Class Code: F1 Transport Cat: 3 |  | University Hazard Classification |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Dispensing into tractors, agricultural vehicles and generators etc. |  |  |  |  |  |  |
| Handling Requirements e.g. fume hood, MSC, glove type etc. | - Diesel dispensing must only be carried out in well-ventilated areas or outdoors as it can release vapours that can readily form flammable mixtures. - Smoking is strictly prohibited. - Wash skin thoroughly after handling. |  |  |  |  |  |  |
| Laboratory Info | Use only outdoors or in well-ventilated areas |  |  |  |  |  |  |
| Additional Information | - Only trained individuals must dispense diesel fuel. - A responsible person must be nominated for ordering and organising deliveries. - A strict log of dispensed quantities of diesel MUST be maintained at all times. - Diesel store is locked at all times. - Workers handling diesel may be at risk of dermatitis and skin drying. They should regularly check their skin condition to ensure no noticeable changes have occurred - Any change in health status you think may be related to using these products must be reported to your supervisor/line manager immediately and an Occupational Health referral made. |  |  |  |  |  |  |
| Hazard Statements H226 H304 H315 H332 H351 H373 H411 | Flammable liquid and vapour Keep away from hot surfaces, heat, sparks, open flames and other ignition sources | Causes skin irritation. Harmful if inhaled. |  | May be fatal if swallowed and enters airways. Suspected of causing cancer. May cause damage to organs through prolonged or repeated exposure (bone marrow, liver, and thymus). |  | Toxic to aquatic life with long lasting effects |  |
| Handling Precautions P210 P260 P273 P280 P301 + P310 P331 | Chemical resistance gloves to EN 374 – must cover wrist | Wear long-sleeved protective overalls. Contaminated clothing should be changed and not worn again until thoroughly washed. |  | Wear goggles (EN166) where there is a risk of splashing. |  | Insert appropriate safety symbols |  |
| Storage & Transport | - Storage is in an enclosed tank within the Boiler House and dispensed via an approved metered nozzle housed on an exterior wall of the building in a secluded cupboard with a key entry system. - Red diesel (gas oil) is dispensed directly into the tractor / agricultural vehicle tanks by approved operators in Horticultural Services team. This occurs in an open air environment and therefore maximum ventilation available. - Use explosion-proof electrical, ventilating and lighting equipment. - Use non-sparking tools - Take action to prevent static discharge |  |  |  |  |  |  |
| Spillage and Waste Disposal | Minor spillages on sealed ground: - Eliminate all sources of ignition in immediate area. Use an absorbent spill kit available and follow local procedures. - Place all waste in black plastic bin liners and carefully transfer to contaminated waste bin for approved supplier to dispose of appropriately. Land Spill: - Eliminate all ignition sources (no smoking, flares, sparks or flames in immediate area). - Stop leak if you can do so without risk. - All equipment used when handling the product must be grounded. - Do not touch or walk through spilled material. - Protect entry into waterways, drains, basements or confined areas using spill control booms. - Absorb the spillage with absorbent material and non-sparking tools and transfer into waste containers. - Water spray may reduce vapour, but may not prevent ignition in enclosed spaces. Notification: In the event of a major spill or accidental release, notify the H&S team immediately who will contact the relevant authorities in accordance with all applicable regulations. |  |  |  |  |  |  |
| First Aid | EYES: Use one bottle of Diphoterine® per eye as per manufacturer’s instructions. If Diphoterine® is not available flush thoroughly with clean water for at least 10 minutes. Seek medical attention. |  |  |  |  |  |  |
|  | SKIN: If skin surface is damaged apply a clean dressing and seek medical attention. If skin surface ins not damaged, wash affected areas thoroughly with soap and water. If irritation or redness develops seek medical attention |  |  |  |  |  |  |
|  | INHALATION: If respiratory symptoms or other symptoms of exposure develop, move victim away from source of exposure and into fresh air in a position comfortable for breathing. If symptoms persist, seek immediate medical attention. If victim is not breathing, clear airway and immediately begin artificial respiration. Seek immediate medical attention.. |  |  |  |  |  |  |
|  | INGESTION: Seek immediate medical attention. Do not induce vomiting. If ingested, material may be aspirated into the lungs and cause chemical pneumonitis. Treat appropriately. |  |  |  |  |  |  |