Safe site & environment

In order to ensure that the site and immediate environment are safe the following will need to be controlled:

- compactors must be located and so positioned so as to preclude or decrease transport risks and access by unauthorised persons;

- compactors must be located away from overhead power lines;

- if used during hours of reduced light then adequate lighting should be provided;

- in areas where the general public/students have access, then hopper doors, control panels and electrical supply must be effectively isolated to prevent unauthorised use;

- the immediate environment should be kept free from obstructions , accumulation of rubbish and other items that may hinder the exchange of compactors; and

- the concrete/tarmac standing should be in good condition so as not to present a risk of trips or falls.

Safe equipment

There is a legal obligation to keep equipment safe in order to reduce the risk of harm to users and others who may be affected:

- access to dangerous parts of the compactor must be prevented;

- any guarding and/or interlock devices must be well maintained and checked for effectiveness on a regular basis;

- the control panel must be secured by way of effective electrical isolation, lockable controls and/or by dedicated key operation, so as to prevent unauthorised use;

- fundamental instructions on the use of the compactor should be displayed on the unit itself, along with access to operating manuals/procedures; and

- signage and instructions on units should be simple, bold, clear and legible.

Safe working procedure

In order to minimise any risk, safe working procedures should be implemented and should include the following:

- information from the manufacture/supplier of a specific compactor on its safe use;

- the type and construction of the compactor;

- the location of the compactor;

- environmental conditions;

- who will be using it;

- which company will maintain it; and

- pertinent circumstances pertaining to routine and non-routine activities, for example container exchange and clearing of any blockage

Container pick-up/drop-off

Ease of access is essential in order to facilitate the safe collection and return of the compactor in a safe manner, for this reason:

- vehicles and pedestrians should be segregated wherever possible;

- the immediate environment around the compactor should be kept clear, for example; parked cars, waste bins and refuse;

- a designated area (yellow-painted hatched area) will identify a clearly defined area where parking is prohibited;

- members of the public and non-essential staff should be excluded from the area during the exchange of any compactor;

- before an exchange of compactors prior notice should be given by the service provider in order that the area can be made safe;

- the utilisation of a trained banks- man or reversing assistant would be deemed ‘best-practice’, of which:

  - would need to wear high-visibility clothing,

  - would agree with the driver their positioning and system for signalling, and

  - remain in full view of the vehicle driver at all times and the driver must stop with immediate effect if they lose sight of that person.

Safe user

In order to maintain the safety of users:

- ensure ‘authorised’ users are suitably trained;

- consider the special training requirements of temporary, part-time workers and those who may not speak English as their first language;

- provide regular refresher training and review of safe systems of work during  health & safety ‘tool-box’ talks; and

- monitor and review operations at regular intervals so as to ensure safe working procedures are undertaken effectively and remain so.