External power supply and control panels

In order to safeguard and prohibit the unauthorised use of compactors the electrical supply and/or the control panel must be isolated where:

- the main electrical isolation switch is protected within a housing this must be locked at all times;

- a mounted rotary electrical isolator switch is in use then this must be locked-off with a lockable padlock at all times (when not in use);

- a compactor control panel is protected within a lockable housing this must be locked-off with a lockable padlock at all times (when not in use); and

- where a compactor control panel is not protected within a lockable housing then the electrical supply must be isolated at source.

Unauthorised users must not be able to access and/or be able to use compaction equipment

Emergency-stop buttons (fail-safe)

- emergency-stop buttons must be clearly marked for ease of identification; and

- emergency-stop buttons should be checked during the compaction cycle to ensure that they are working effectively.

Hoppers

- hopper access doors must be padlocked-off at all times (when not in use); and

- access (height from ground-level) should not present a risk of musculoskeletal injury to those using the compactor.  Where ease of access (height from ground-level) may be hindered then nominated managers must implement and manage effective control measures to eliminate/reduce any such risk.

Operator instructions and safety signage

The following instruction and safety signage must be displayed on compactors and must be legible at all times:

- Compactor Rules: Trained and Authorised Users Only

- Warning do not go to the rear of compactor when machine is in operation

- Do not enter hopper

- Only authorised personnel may operate this machine

NB: Any missing and/or damaged signage should be replaced with immediate effect and must be reported to the Waste & Recycling Manager (Estates Dept.) on ext. 50548/07876 218197 or via e-mail to j.g.watson@warwick.ac.uk  and waste&recycling@warwick.ac.uk

Training of personnel

Prior to use of compactors:

- ‘named’ and ‘nominated’ managers’ must be inducted on compactor-specific instruction (on-location), of which shall form part of the induction process delivered by the Waste & Recycling Manager or Waste & Recycling Supervisor;

- ‘authorised’ users must be inducted on compactor-specific instruction (on-location), of which shall form part of the induction process delivered by ‘named’ and ‘nominated’ managers’ for that area;

- on completion of compactor training, signatures of attendees must be secured and kept as part of the departments training records with a copy sent to the Waste & Recycling Manager to be held centrally;

- once trained ‘authorised’ users must be supervised until deemed confident and competent;

- refresher training must be offered every 2 years; and

- in the event that changes are made to a compactor, modification to a mechanical aspect or there has been an incident, then the training programme must be revisited with appropriate training delivered by the Waste & Recycling Manager or Waste & Recycling Supervisor.