Acoustic panel moves

Important notes;

To rotate the 2 panels nearest door 1 the 5 removable seats will need to be unbolted.

Ensure care is taken when moving the panels behind row VV near Door 1, possibility of slip down stairs and narrow aisle to work in.

Panels behind rows Y and V only have a narrow aisle to work in

Check the floor area of the platform above door 6 for loose equipment etc before starting work.

Think of what is behind you before starting each panel; they will force you to move backwards.

- Stand squarely in front of each panel

- Pull on the edge of the panel farthest from the hinge bracket.

- Pull this edge out towards you.

- When it is 90 degrees to the wall grasp both edges of the panel

- Continue rotating panel and gently push back to wall

- If required get help for the longer panels, particularly to push back to wall

- Keep fingers clear when the panel goes back to the wall