To reset the Green break glass point boxes

If the box has been triggered the doors will not be locked and a thin yellow stripe will be visible at the top of the glass.

- You will need the black plastic release key, in the Op’s key press

- Insert the twin prong end into the holes in the underside of the unit

- The single leg should be nearest you when inserting prongs

- It should then be possible to remove the front from the unit

- Remove key from unit

- Replace front section by;

- Push into slots about 2cm from top location

- When located push upwards, this should move glass up into place and the door magnets re-energise

- Check both doors are secure and return key to key press