| Harness |  |
| --- | --- |
| - Check for fraying and abrasions |  |
| - Ensure sources of heat (e.g. blowlamp) are kept well away from the material |  |
| - Inspect buckle areas for pilling. Buckling and unbuckling can cause wear and tear |  |
| - Check the condition of all stitching |  |
| - Check for fraying and abrasions |  |
| - Ensure there are no cuts in the webbing |  |
| - If any equipment is involved in a fall, remove from service and return to your Line Manager |  |
| - Accessories must be used correctly |  |
| Rope Grabs and other line attachment devices |  |
| - Check all metal parts for damage or corrosion |  |
| - Check direction of travel |  |
| - Ensure safety catch engages when activated |  |
| - Check that the teeth of the rope grab grips the anchorage line |  |
| Lifelines and Lanyards |  |
| Original length of rope | cm |
| - Inspect rope from end to end. Look for worn, heat damaged or cut fibres. |  |
| - Discard any line or lanyard that is significantly worn |  |
| - If the anchorage line becomes stretched by more than 10% of it’s original length, discard it and replace |  |
| - Be aware that the shock absorber will extend by more than one metre when operated |  |