Risk Assessment Form

| Title of Risk Assessment | Use of Autoclaves |  | Date of assessment |  | <insert date> |
| --- | --- | --- | --- | --- | --- |
| Department | Sciences |  | Date review due |  | <review date 3 years later. To review earlier following an incident or significant change> |
| Description of Task/Process | Autoclaves used for sterilising biological waste, preparation of sterile media and sterilising laboratory instruments. Autoclaves can be front or top opening and be fixed or semi-portable. Pressure cookers are not covered by this risk assessment. |  |  |  |  |
| Assessment carried out by | <insert name of individual or competent persons> |  |  |  |  |

| ##### Additional information | Note that this is a generic risk assessment so should be tailored to take into consideration any additional hazards present connected with the activity, the materials used, the people involved or the environment. All autoclaves (including bench-top) must be on a maintenance contract and undergo statutory inspection and testing, according to a written scheme, at least, every 14 months. It is the purchaser’s responsibility to ensure that any new autoclave bought into a department is placed onto the existing department’s maintenance contract. Local laboratory rules must be followed at all times. There must be a written departmental procedure in place for the correct use of autoclaves. Only trained and competent personnel must operate autoclaves. |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Steam Scalding/ hot burns | Operators, bystanders, service engineers | Training Interlocks/shielding prevent rapid steam release Exposed unlagged pipes carrying steam are marked Heat-resistant gloves (BS EN407) and face shield (bench-top) to be worn when removing items from autoclaves | Low |  |  | Low |
| Explosion of sealed glass containers Heat burns and impact injury from sharp fragments. | Operators, bystanders, | Training Do not screw glassware tops or bungs tightly Allow sufficient time for cooling of autoclave contents before removal. | Low |  |  | Low |
| Incorrect container used for sharps Cuts, puncture injury | Operators | Training Lab users to ensure all sharps are only placed in sharp bins | Low |  |  | Low |
| Autoclaved contents Inhalation of toxic substances | Operators, bystanders | Training Lab users must ensure contents placed into autoclave tins can be autoclaved. Make reference to any relevant SDS and/or SIN. Allow autoclave contents to cool before opening | Low |  |  | Low |
| Manual handling Musculoskeletal injury | Operators | Training All large autoclave operators attend a face to face manual handling course every 3 years. Manual handling aids e.g. trolleys used for autoclave tins to be manoeuvred in and out of the large autoclave | Low |  |  | Low |
| Slippery waste materials or free liquid. Slips, trips, falls |  | Training Good housekeeping Clean up spills immediately | Low |  |  | Low |
| Objects blocking access points Blocking safe egress in the event of an emergency |  | Routes for loading/unloading should be direct, short and kept free of clutter. Temporary storage of containers should be away from pedestrian routes Contingency areas in case of waste backlog | Low |  |  | Low |
| Lab instruments Cuts, abrasions | Operators | Training Lab users to separate lab instruments which can entangle e.g. forceps and scissors | Low |  |  | Low |
| Glassware moulding to plastic ware Sharp injury to operators | Operators, bystanders, | Training Lab users to separate bagged waste from contaminated glassware | V Low |  |  | V Low |
| Deterioration of seals, pressure relief valves and other components Failure of component, inadvertent release of steam | Operators, bystanders, service engineers | Statutory testing to ensure seals etc. are fit for purpose. Named person to ensure maintenance schedule does not lapse. Users to immediately report to named person any defects of which they become aware. Users should be made aware of inspection intervals and any results of periodic examination and maintenance. | V Low |  |  | V Low |
| Explosive displacement of a door Impact injury | Operators, bystanders, service engineers | Statutory testing Interlocks to prevent pressurisation if door is not secured properly. | V Low |  |  | V Low |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | Low |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | <Name of Peer, Supervisor or Line Manager> |  | Position | <insert role> |
| Date | <add date of approval> |  |  |  |

Please print a copy, sign it and keep for your records

Document history:

| Version | Date | Reviewer | Comments/Changes Made |
| --- | --- | --- | --- |
| V1.0 | 30/07/2018 | Ian Graham | New version |
| V2.0 | 15/08/2019 | Caroline Farren | Reviewed – no significant changes made |
| V2.1 | 03/08/2023 | Caroline Farren | Reviewed – added document history and altered dates on front page – this section should be completed by those adopting this generic. |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.