Risk Assessment Form

| Title of Risk Assessment | Use of Centrifuges |  | Date of assessment |  |  | <insert date> |
| --- | --- | --- | --- | --- | --- | --- |
| Department | Sciences |  | Date review due |  |  | <review date 3 years later. To review earlier following an incident or significant change> |
| Description of Task/Process | Use of centrifuges including loading, unloading samples, changing rotors etc. This risk assessment includes all types of centrifuges from microfuges to Ultra centrifuges. |  |  |  |  |  |
| Assessment carried out by | <name of individual or competent persons> |  |  |  |  |  |

| ##### Additional information | Note that this is a generic risk assessment so should be tailored to take into consideration any additional hazards present connected with the activity, the equipment, the materials used, the people involved or the environment. All lab users must be trained on how to use a centrifuge safely (including do’s and don’ts) and what to do in the event of a spillage, cracked tubes etc. All departments should have written emergency procedures in place on how to deal with a spill in a centrifuge for departmental appropriate samples. Lab rules must be adhered to at all times All centrifuge defects must be reported immediately via appropriate departmental channels e.g. technical support team, nominated person etc. All Ultra-centrifuges must have a log book in place which includes the rotor speed, length of spin, rotor type etc. so that the rotor can be monitored and discarded in line with the manufacturers guidelines. |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Mechanical failure Impact injury | Operators, bystanders | All centrifuges and their rotors are serviced on an annual basis High speed centrifuges rotor use is logged All worn parts are replaced as instructed at service. | Low |  |  | Low |
| Rotor imbalance Aerosolization of biological and chemical substances, centrifuge damage | Operators, bystanders | Training Pre-use check Out of balance error message/cut-off switch installed on machine | Low |  |  | Low |
| Incorrect tube use Sample exposure, spills, centrifuge damage | Operators, bystanders | Training Pre-use check for cracks in tubes prior to sample preparation Users ensure that the tube is compatible with its contents | Low |  |  | Low |
| Overfilling tubes Sample exposure, spills, centrifuge damage | Operators, bystanders | Training Tubes are ¾ full or as per manufacturer’s instructions | Low |  |  | Low |
| Sample exposure Occupational ill-health | Operators, bystanders | Training Use screw top tubes with O-rings where there is an aerosol risk O-rings checked prior to use Tube height is compatible with the selected rotor Tubes are ¾ full or as per manufacturer’s instructions Allow aerosols to settle prior to opening the centrifuge Door interlocks in place Swinging bucket rotor – make sure all buckets are hooked correctly and move freely | Low |  |  | Low |
| Manual handling Musculoskeletal injury | Operator | Training Rotors stored at bench-height Trolley used to move large rotors Rotor aids (T- bar) used where available | Low |  |  | Low |
| Low flash-point samples Fire, burns | Operators, bystanders | Training Flammable substances only centrifuged as per manufacturer’s instructions Rotor speed is not exceeded | Low |  |  | V Low |
| Electricity Electrocution | Operator | User pre-use checks All centrifuges are PA tested on an annual basis Statutory fixed wire tests every 5 years | Low |  |  | V Low |
| Defective or missing Interlock Entanglement of clothing, friction burns, impact injury | Operators, bystanders | Training All centrifuges and rotors are serviced on an annual basis Defects reported immediately to nominated person or technical team and centrifuge placed out of use | Low |  |  | V Low |
| Incorrect rotor use Impact injury, Centrifuge damage | Operators, bystanders | Training Rotor error message/cut-off switch installed on machine Pre-use check rotor is compatible with the centrifuge | Low |  |  | V Low |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | Low |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | <name of peer, supervisor or line manager> |  | Position | <insert role> |
| Date | <insert date of approval> |  |  |  |

Please print a copy, sign it and keep for your records

Document history:

| Version | Date | Reviewer | Comments/Changes Made |
| --- | --- | --- | --- |
| V1.0 | 02/02/2019 | Ian Graham | New version |
| V1.1 | 15/08/2019 | Caroline Farren | Reviewed – no significant changes made |
| V1.2 | 03/08/2023 | Caroline Farren | Reviewed – added document history and altered dates on front page – this section should be completed by those adopting this generic. |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.