Risk Assessment Form

| Title of Risk Assessment | General use for lathes |  |  | Date of assessment |  | <insert date> |
| --- | --- | --- | --- | --- | --- | --- |
| Department | Engineering |  |  | Date review due |  | <review date 3 years later. To review earlier following an incident or significant change> |
| Description of Task/Process | This risk assessment covers the general use of lathes for working suitable materials (as defined by manufactures guidance). |  |  |  |  |  |
| Assessment carried out by | <name of individual person or competent persons> |  |  |  |  |  |

| ##### Additional information | Note that this is a generic risk assessment so should be tailored to take into consideration any additional hazards present connected with the activity, the materials used, the people involved or the environment. This risk assessment must be appropriately adapted for the type of lathe used and the material being worked. This risk assessment does NOT cover: - Maintenance - Cleaning A separate risk assessment and/or SSOW must be developed prior to permitting maintenance to be carried out on machinery in the Workshop. Cleaning to follow manufacturer’s instructions. |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Inadvertent start-up e.g. during mounting removal of work piece; or removal of waste material from lathe. | Operator or others in vicinity of lathe | - Machine is equipped with power isolator which can be locked “off”. - Machine locked off when not in use. | L | Double isolations may be required to completely power down some machinery. | Workshop Manager to identify as part of PUWER assessment. | Low |
| Entanglement in rotating chuck – potential for drawing into sharp and/or moving parts (Brake may not stop machine immediately and may continue to rotate briefly after machine is disabled.) | Operator and others who may be assisting in its operation | - Emergency stop button installed on lathe. - Lathe is only operated by (sufficiently) trained staff or under the guidance and supervision of (sufficiently) trained staff - Interlocked chuck guard must be used where available and a fixed guard at back of machine. - Long hair must be tied back. - Any loose clothing / jewellery must be secured to prevent contact with moving parts. - Gloves must not be worn when operating lathe. - Pulley/drive/gearing system must be totally enclosed. - There must be sufficient space around the lathe to prevent any inadvertent contact with moving parts. - Planned preventive maintenance to ensure that safety features of the machine operate correctly - Fail-safe system in event of power loss which prevents machine restart when power is restored | M | • All persons operating machine to be trained in its safe use (training records maintained) • Supervision until person proven competent • Induction on Workshop Rules provided | Competent operator to train others in safe use and to determine supervisory level needed. | Low |
| Contact with sharp material | Operator | - Built up waste material must be removed prior to operation using a brush and pliers where necessary to avoid removal by hand. - If small volumes need to be moved from hand, gloves to EN 388 standard (cut resistance) to be worn. | M | Local swarf bins to be suitably sited in workshop space which are emptied at regular intervals. | Workshop Manager | Low |
| Ejected material /swarf/ work piece - potential for eye injuries or impact injuries for larger pieces | Operator and others who may be assisting in its operation | - Adjustable guard to be used to protect operator. - Work piece to be mounted by a suitably trained person. - Controls used to mount work piece in machine must be locked when chuck is being driven. - Cutting blades/tools must be sharp and free of defects. - Wear minimum eye protection conforming to at least BS. 2092 (general purpose). - Machine should initially be run at low speed after mounting workpiece to ensure security of mounting - Trained, competent operator (or under direct supervision of a trained, competent operator) | L |  |  | Low |
| Hot surfaces – work piece/tools may become hot and may cause minor burns if handled | Operator and others who may be assisting in its operation | - Allow work piece to cool before handling - Use cooling fluids to minimise temperature of workpiece and tooling | L |  |  | Low |
| Slips trips and falls | Operator or others in vicinity of lathe | - Cleaning and housekeeping standards must be maintained. - Area around lathe must be clear of trip hazards. Any power supply cables etc. must be suitably covered where they cannot be situated in a more suitable location. | M | Line manager to improve housekeeping in the area. Regular walk-throughs to be carried out to check housekeeping is maintained | Workshop manager / local supervisor | Low |
| Electricity – electric shock | Operator or others in vicinity of lathe | - Operator to visual check lathe for damage prior to use including the electrical cord. - Periodic electrical inspection as part of annual PAT test programme |  |  |  |  |
| Noise – operation of lathe may result in high noise levels | Operator or others in vicinity of lathe | - Appropriate* ear protection to be available where noise is in excess of 80 dB(A) and always worn when at or near 85 dB(A) when operating the equipment. *The need for appropriate level of ear protection can be determined by manufacturer’s instruction/guidance. [Noise levels from stock bars rotating in stock bar tubes can be very high, particularly on multi-spindle machines] | M | Obtain noise data from supplier / manufacturer of new equipment and consider noise levels in procurement choice. | Purchaser of machine | Low |
| Dust and mists – potential for respiratory conditions and long term health effects | Operator or others in vicinity of lathe | - COSHH assessment of materials being cut on lathe must be conducted as well as for cutting fluids and lubricants used. - Local exhaust ventilation (LEV) should be used where available. - Where LEV is unavailable, appropriate* respiratory protective equipment (RPE) should be used. - Good general ventilation - Do NOT use compressed air to clean down machine * The appropriate type of RPE should be determined by a separate COSHH assessment of the material to be cut on the lathe. | M | Adhere to controls outlined in COSHH Assessment. Consider splashguards if machining at high speeds. | Operator to determine | Med/Low |
| Chemicals (metal work fluids) – may cause irritation to skin and dermatitis |  | - Splashguards to protect from splashing of suds. - COSHH assessment of any fluids used on lathe must be conducted. - Contact with fluid must be minimised and hands must be washed thoroughly following use of metal working fluids. - Follow good working practices when mixing fluids, cleaning and topping up sumps. - Layers of scum or large amounts of tramp oil on top of the sump fluid, or if the sump fluid is dirty or smelly to be reported to Workshop Manager. - Barrier creams should be available to operators | M | Ensure sufficient provision of hand cleaning materials in vicinity. Good personal hygiene by operators. Check skin for signs of irritation / consider health surveillance | Workshop Manager. Operators Workshop Manager and Operators | Low |
| Manual Handling (changing chuck) – muscular skeletal injury | Operator and others who may be assisting in its operation | - When changing a heavy chuck a manual handling assessment may be required. - Manual handling training may be required. | L | Implement any additional controls identified as a result of a manual handling assessment (as necessary) | Operator | Low |
| Entanglement due to emery paper use | Operators | - SSOW in place for internal and external emery paper application. - No application of emery paper by hand. - Emery cloth not to be used on CNC lathes. - Training and instruction of operators in use of acceptable methods of emery paper application (i.e. use of clamping devices) as detailed here - http://www.hse.gov.uk/engineering/lathes.htm | L |  |  | Low |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | M/L |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | <name of peer, supervisor or line manager> |  | Position | <insert role> |
| Date | <insert date of approval> |  |  |  |

Please print a copy, sign it and keep for your records

Document history:

| Version | Date | Reviewer | Comments/Changes Made |
| --- | --- | --- | --- |
| V1.0 | 30/07/2018 | Ian Rose | New version |
| V2.0 | 16/08/2019 | Ian Rose | Reviewed – noise levels from stock bars and double insulation control measure added to risk assessment |
| V2.1 | 03/08/2023 | Caroline Farren | Reviewed – added document history and altered dates on front page – this section should be completed by those adopting this generic. |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.