Risk Assessment Form

| Title of Risk Assessment | Management and collection of chemical and biological waste |  |  | Date of assessment |  | <insert date> |
| --- | --- | --- | --- | --- | --- | --- |
| Department | Bioscience Laboratories and Facilities |  |  | Date review due |  | <review date 3 years later. To review earlier following an incident or significant change> |
| Description of Task/Process | Management and collection of chemical and biological waste from containment level 1 and 2 laboratories. This risk assessment does not cover containment level 3 activities. |  |  |  |  |  |
| Assessment carried out by | <name of individual or competent persons> |  |  |  |  |  |

| ##### Additional information | Note that this is a generic risk assessment so should be tailored to take into consideration any additional hazards present connected with the activity, the materials used, the people involved or the environment. Local rules must be followed at all times. A written departmental process must be in place to manage the generation of all types of waste produced; including from chemical and biological samples. All laboratory-based staff, students and visitors must be made aware of the procedures within their department and follow them; the PI is responsible for ensuring this is done. Biological and chemical waste must not be allowed to build-up in laboratories and it is the individual’s responsibility to ensure this does not happen. Licenced material must be disposed of in-line with the relevant legal requirements e.g. radioactive material, biological agents, plant material etc. There is also a SLS/WMS Waste Management & Safety course covering the processes, procedures and decision-making in purchasing and using materials for the laboratory as well as choices that will help lab users to manage, reduce waste and increase the safety of those working in the lab. This is available for staff and students to self-enrol onto and is a good starting point as part of induction training. |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Exposure to biological samples (CL1 and CL2 only) Occupational illness | Staff, students, visitors | Training Biological waste enclosed in an autoclave tin – lid must be flush and not overfilled prior to collection Biologically contaminated glassware is disinfected with a validated disinfectant prior to collection Local procedures in place and followed for other biologically-active samples disposal routes | Low |  |  | Low |
| Exposure to chemical waste Occupational illness, burns | Staff, students, visitors | Training Re-usable glassware rinsed with water, as chemical appropriate, and made safe prior to collection Empty waste bottles or out-of-date chemicals taken to appropriate chemical waste stream for collection – outer bottle is free from chemical residue and lids are securely tightened Chemicals taken from individual waste collection points to the outside storage must be transported in a deep-well trolley and packed to ensure they are kept upright Broken glassware is separated and placed in a suitable, labelled, impervious receptacle and those who collect the waste informed | Low |  |  | Low |
| Manual handling Musculoskeletal disorders, injury | Staff, students, visitors | Training – all support staff attend manual handling training every 3 years Waste bags are not over-filled Trolley aids are used to transport bagged waste, autoclave tins etc. Lab users place waste at waist-height in designated areas ready for collection Trolleys are easily manoeuvrable, suitable for outdoors use and ideally height adjustable Trolleys are not over-loaded | Low |  |  | Low |
| Sharps Cuts, puncture wounds | Staff, students, visitors | Sharps are ONLY placed in sharps bins Sharps bins are sealed/ locked into place prior to collection Broken glassware is kept separate from other waste in a suitable robust container and support staff informed | Low |  |  | Low |
| Spills Slips, trips, falls, sample exposure | Staff, students, visitors | Collection trolleys are not over-loaded. Bagged waste is not over-filled Bags are checked for splits prior to use Autoclave tins are not over-filled and lid is flush to the bottom of the tin Waste bottles/out-of-date chemicals are transported to waste collection points using safepaks, Winchester carriers or similar Suitable in-date spills kits are readily available Trained personnel clean up spills immediately | Low |  |  | Low |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | Low |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | <name of peer, supervisor or line manager> |  | Position |  |
| Date |  |  |  |  |

Please print a copy, sign it and keep for your records

Document history:

| Version | Date | Reviewer | Comments/Changes Made |
| --- | --- | --- | --- |
| V1.0 | December 2017 | Ian Graham | New version |
| V2.0 | 16/08/2019 | Ian Graham | Full review conducted to bring in line with risk assessment template. |
| V2.1 | 03/08/2023 | Caroline Farren | Reviewed – added document history and altered dates on front page – this section should be completed by those adopting this generic. |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.