Risk Assessment Form

| Title of Risk Assessment | Soldering electronic components |  |  | Date of assessment |  | <insert date> |
| --- | --- | --- | --- | --- | --- | --- |
| Department | Sciences |  |  | Date review due |  | <review date 3 years later. To review earlier following an incident or significant change> |
| Description of Task/Process | Soldering of electronic components using lead, rosin and unleaded/non-rosin based cored solder wire |  |  |  |  |  |
| Assessment carried out by | <name of individual or competent persons> |  |  |  |  |  |

| ##### Additional information | Note that this is a generic risk assessment so should be tailored to take into consideration any additional hazards present connected with the activity, the equipment, the materials used, the people involved or the environment. Unleaded solder, which requires a higher soldering iron temperature, precise temperature control and the use of special types of flux to rework solder joints should be conducted so far as is reasonably practicable. Soldering of older electronic components which could warrant the use of lead-based, rosin-cored solder should note the additional hazards applicable in this risk assessment above and beyond unleaded/non-rosin based soldering work. Any work of this nature MUST be JUSTIFIED and the detail stated in this box, and agreed with your line manager/supervisor BEFORE you carry out the soldering activity. This risk assessment should be read in conjunction with the Substance Information Note 008 when soldering (jointing and repairing metals and printed circuit boards (PCB’s) using rosin free and lead free fluxes. |
| --- | --- |

| Hazards and how they may cause harm | Who may be at Risk? | Existing Control Measures | Current Risk Level (VL,L,M,H,VH) | Where current risk is M, H or VH, what additional Control Measures are required? | Action required by whom & by when? | Final Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Inhalation of fumes from lead and rosin-free solder Exposure to fumes can cause eye, throat and lung irritation. | Users, co-workers | Refer to the solder wire data sheet as this will advise on composition of wire to be used. Use of local exhaust ventilation (LEV) essential, i.e. on-tip extraction wherever practicable. LEV must possess a suitable filter system before recirculating back to the room. Routine maintenance and inspection of the LEV to meet statutory requirements. Training to undertake soldering operations close to point of extraction wherever on-tip extraction cannot be used. | L | Where on-tip extraction is not used, the LEV must be directed as close to the soldering location as possible to capture fumes generated. | Users each time they set up their soldering station prior to undertaking the work | Low |
| Inhalation of rosin fumes causing respiratory complaints Exposure to rosin can cause eye, throat and lung irritation, nose bleeds and headaches. Repeated exposure can cause respiratory and skin sensitisation, causing and aggravating asthma. | User, co-workers | Refer to the solder wire data sheet as this will advise on composition of wire to be used. Use rosin-free and lead-free solders wherever possible. Use of local exhaust ventilation (LEV) essential, i.e. on-tip extraction wherever practicable. LEV must possess a suitable filter system before recirculating back to the room. Routine maintenance and inspection of the LEV to meet statutory requirements. Training to undertake soldering operations close to point of extraction wherever on-tip extraction cannot be used. | M | Consider whether lung function health surveillance may be required for regular users. | Line Manager at regular intervals, or as circumstances change | Moderate |
| Contact with lead (skin contact, ingestion or inhalation) causing lead poisoning Exposure could be through accidental ingestion through contact from your skin. Lead can give rise to serious chronic health effects. | Users, co-workers | Refer to the solder wire data sheet as this will advise on composition of wire to be used. Use rosin-free and lead-free solders wherever possible. Use of lead-based solder restricted to low temperature applications only or for soldering older electronic components. Wear gloves if directly handling solder. Ensure good personal hygiene whilst handling solder wire. Handwashing facilities to be used where gloves are not worn before eating, drinking, smoking. Use of local exhaust ventilation (LEV) essential, i.e. on-tip extraction wherever practicable. LEV must possess a suitable filter system before recirculating back to the room. Routine maintenance and inspection of the LEV to meet statutory requirements. Training to undertake soldering operations close to point of extraction wherever on-tip extraction cannot be used. | M | Use of lead solder to be evaluated on each occasion and used on an exception basis. Occupational health screening may need to be evaluated pre-lead use by referring member of staff to Occupational Health. | Line Manager pre-use of lead solder wire. | Modearate |
| Fire, caused by contact of hot tip on combustible materials | Building occupants | Good housekeeping to remove combustibles from soldering area. Training to place soldering iron in holder when not in use. Work on a fireproof surface. | L |  |  | Low |
| Burns, caused by contact with hot tip | User | Use of tools to keep hands away from hot tip. Care whilst soldering. | M | Those under instruction to be reminded that soldering iron tip will remain hot for some time post soldering. | Supervisor at start of soldering activity | Low |
| Cleaning solvents, inhalation of solvent vapour, lung irritation |  | Cleaning solvents kept in dispensing bottles | L |  |  | Low |
| Burns from moving hot work | User | Training awareness that workpiece becomes hot during soldering operations. Use of appropriate tools to hold workpiece. | L |  |  | Low |
| Electrocution caused by faulty iron | User | Routine Portable Appliance testing, inspection of flex and plug before each use. | L |  |  | Low |
| Muscle strain caused by extended period of soldering | User | Take regular breaks and change body position frequently | L |  |  | Low |
| Indirect ingestion of solder materials | User | Good hygiene practices, ensure hand washing after contact with solder. | L | Where a space is used for soldering which does not have local handwashing facilities, persons should be directed to nearest facilities. | By supervisor / line manager prior to soldering | Low |

Work should not be carried out until the assessment is completed and all required control measures are in place.

| Overall Final Risk Rating (Highest level in final column above) | Moderate |
| --- | --- |

| Additional Comments from Risk Assessor (e.g. funding or practical implications) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved By | <name of peer, supervisor or line manager> |  | Position | <insert role> |
| Date | <insert date of approval> |  |  |  |

Please print a copy, sign it and keep for your records

Document history:

| Version | Date | Reviewer | Comments/Changes Made |
| --- | --- | --- | --- |
| V1.0 | 28/05/2019 | Ian Rose | New version |
| V2.0 | 15/08/2019 | Ian Rose | Format change. |
| V2.1 | 03/08/2023 | Caroline Farren | Reviewed – added document history and altered dates on front page – this section should be completed by those adopting this generic. |

|  | Severity |  |  |  |  |  |  | Risk Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likelihood | Superficial | Minor | Serious | Major | Extreme |  | Very low | Acceptable risk - no action required |
| Unlikely | Very low | Very low | Low | Low | Moderate |  | Low | Tolerable risk - further control measures not required, but status must be monitored |
| Possible | Very low | Low | Low | Moderate | High |  | Moderate | Further control measures required to reduce risk as far as is reasonably practical |
| Likely | Low | Low | Moderate | High | Very high |  | High | Urgent action required to allow activity to continue |
| Very likely | Low | Moderate | High | Very high | Very high |  | Very high | Risk intolerable - activity must cease until the risk has been reduced |
| Extremely likely | Moderate | High | Very high | Very high | Very high |  |  |  |

See ‘Matrix for risk evaluation’ for further guidance.