Guide on Use of Fire Extinguishers

This information will enable staff to operate fire extinguishers that are found on campus.

Do not put your personal safety at risk.  Only use a fire extinguisher in an emergency to aid escape from a fire or to extinguish a ‘small’ fire.

- Ensure that the alarm has been raised

- Select the correct extinguisher for the type of fire in accordance with the following table:

Types of fire extinguisher used on University Premises:

| Water | CO2 | Powder | Foam |
| --- | --- | --- | --- |
| USE on: |  |  |  |
| Wood, paper, textiles and solid material fires | Flammable Liquid and Electrical fires | Flammable Liquid, Flammable Gases, Electrical and Wood, paper, textiles fires | Flammable liquid, wood, paper, textiles and solid material fires |
| DO NOT USE on: |  |  |  |
| Flammable Liquid, Electrical or Metal fires | Metal fires | Metal fires | Electrical or Metal fires |

- Use the extinguisher:

- PULL the safety pin out, to free the lever on top of the extinguisher

- AIM the fire extinguisher nozzle or hose at the base of the fire, standing around 2.5 metres back from the fire

- SQUEEZE the handle to release the fire-fighting agent

- SWEEP the nozzle or hose across the base of the fire (not the flames) until it is fully extinguished; with foam and powder, lay a blanket of foam or powder over the burning area

Warning:- Do not touch the horn of the CO2 extinguisher as your hand could freeze to it.

- Evacuate and report to the Security Response Team

Fire Blankets may be used to extinguish cooking pan fires:

- Pull the blanket out of its case

- Check that the fire is smaller than the blanket

- Hold it well up in front of you by the top corners  and keep your hands tucked in behind the blanket

- Place it over the fire to smother it and leave it there

- Don’t take it off for half an hour, in order to let the material cool down.